THE EXISTING “HYATTS ROAD BRIDGE REPLACEMENT” PLAQUE AND CONCRETE PEDESTAL SHALL BE REMOVED, STORED, AND RECONSTRUCTED IN A SIMILAR RAISED STYLE AND LOCATION, AS APPROVED BY THE ENGINEER.  REMOVAL OF THE CONCRETE PEDESTAL SHALL BE AS-PER ITEM 202.  IF THE PEDESTAL IS NOT FULLY REMOVED, IT SHALL BE REMOVED TO A MINIMUM 1’ BELOW FINISHED GRADE.  CARE SHALL BE TAKEN TO AVOID DAMAGING THE MARKER’S PLAQUE.  PAYMENT SHALL BE MADE PER THE ITEM LISTED BELOW, AND SHALL INCLUDE REMOVAL, DISPOSAL, STORAGE, EXCAVATION, CONCRETE, FORMWORK, AND ANY OTHER MATERIALS, EQUIPMENT, OR LABOR REQUIRED TO PERFORM THE WORK NOTED ABOVE.  IF THE PLAQUE IS DAMAGED DURING CONSTRUCTION, THIS ITEM WILL INCLUDE REPAIR OR REPLACEMENT OF THE PLAQUE AS DIRECTED BY THE ENGINEER, AND SHALL BE INCLUDED IN THE UNIT COST FOR THIS ITEM.

ITEM 202 – REMOVAL MISC.: HISTORIC BRIDGE MARKER		1 EACH

ITEM 202 – GATE REMOVED, AS PER PLAN

THIS ITEM SHALL CONSIST OF REMOVING AND DISPOSING OF VARIOUS PRIVATE FENCE GATES AND THEIR APPURTENANCES.  REMOVAL ITEMS INCLUDE FENCE OR DRIVEWAY GATE (SINGLE OR DOUBLE DOOR), GATE MOUNTING POSTS AND HARDWARE, AND COMPONENTS ASSOCIATED WITH AUTOMATIC GATE OPERATION INCLUDING KEYPADS AND GATE MOTOR.  PAYMENT FOR THIS ITEM SHALL BE EACH PER LOCATION, AND SHALL INCLUDE ALL LABOR AND EQUIPMENT TO PERFORM THE WORK, AND COST FOR DISPOSAL OF THE ITEMS REMOVED.

ITEM 202 – REMOVAL MISC.: LANDSCAPING

THIS WORK SHALL CONSIST OF REMOVING AND DISPOSING OF VARIOUS LANDSCAPING ITEMS REMAINING WITHIN THE PROJECT RIGHT OF WAY, AND WHICH ARE NOT OTHERWISE QUANTIFIED INDIVIDUALLY.  THIS INCLUDES BUT IS NOT LIMITED TO LANDSCAPING TIMBERS AND TIES, STONE BED EDGING, POND LINERS AND MAN-MADE WATER FEATURES, DRIVEWAY PILLARS, AND LANDSCAPE BOULDERS/STONES/GRAVEL.  LUMP SUM PAYMENT FOR THIS ITEM SHALL INCLUDE LABOR AND EQUIPMENT TO PERFORM THE WORK, AND COST FOR DISPOSAL OF THE ITEMS REMOVED.

THIS ITEM DOES NOT INCLUDE REMOVAL OF LANDSCAPING LIGHTING OR IRRIGATION SYSTEMS, WHICH ARE INCLUDED UNDER SEPARATE ITEMS.

ITEM 202 – REMOVAL MISC.: LANDSCAPE LIGHTING

THIS ITEM SHALL CONSIST OF REMOVING AND DISPOSING OF VARIOUS PRIVATE LANDSCAPE LIGHTS AND THEIR APPURTENANCES REMAINING IN THE PROJECT RIGHT OF WAY.  THIS INCLUDES BUT IS NOT LIMITED TO BOTH LOW, HIGH VOLTAGE, AND SOLAR LIGHTS, WIRING, CONDUITS, AND THEIR CONTROL CENTERS.  PRIOR TO REMOVAL OF LIGHTING, CONTRACTOR SHALL COORDINATE WITH THE PROPERTY OWNER TO ENSURE POWER SERVICE IS DISCONNECTED.  PAYMENT FOR THIS ITEM SHALL BE EACH PER PROPERTY OWNER, AND SHALL INCLUDE ALL LABOR AND EQUIPMENT TO PERFORM THE WORK, AND COST FOR DISPOSAL OF THE ITEMS REMOVED.

ITEM 202 – REMOVAL MISC.: IRRIGATION

THIS ITEM SHALL CONSIST OF REMOVING AND DISPOSING OF VARIOUS PRIVATE IRRIGATION COMPONENTS REMAINING IN THE PROJECT RIGHT OF WAY.  THIS INCLUDES BUT IS NOT LIMITED TO SPRINKLER HEADS, VALVES, CONDUITS, CONTROL CENTERS, AND SPIGOTS.  PRIOR TO REMOVAL OF IRRIGATION COMPONENTS, CONTRACTOR SHALL COORDINATE WITH THE PROPERTY OWNER TO ENSURE WATER SERVICE IS SHUT OFF.  PAYMENT FOR THIS ITEM SHALL BE EACH PER PROPERTY OWNER, AND SHALL INCLUDE ALL LABOR AND EQUIPMENT TO PERFORM THE WORK, AND COST FOR DISPOSAL OF THE ITEMS REMOVED.