| Ver. Date | 03/21/2024 |  | PID |  |  | 111621 |
| --- | --- | --- | --- | --- | --- | --- |
| PARCEL |  |  |  | 20-T | -2 |  |
| FAI-158-4.20 |  |  |  |  |  |  |

TEMPORARY EASEMENT FOR THE PURPOSE OF PERFORMING THE WORK NECESSARY TO CONSTRUCT DRIVEWAY FOR MONTHS FROM DATE OF ENTRY BY THE STATE OF OHIO, DEPARTMENT OF TRANSPORTATION

[Surveyor’s description of the premises follows]

Situated in the State of Ohio, County of Fairfield, Township of Greenfield, in the southeast quarter of Section 14 in Township 15, Range XIX of the Congress Lands (East of Scioto River), lying on the left side of the centerline of right of way of Coonpath Road (County Road 31, 60.00 feet wide) and being part of the 2.000 acres of land deeded, now or previously, to Jeffrey J. Gressick and Denise M. Gressick in official record volume 654, page 128 of the Fairfield County Recorder’s Office and known as Fairfield County Auditor’s parcel number 013-00376.00 and further described as follows;

Beginning at the intersection of the existing northerly right of way line of Coonpath Road with the easterly line of land deeded Bonnie L. Weaver in in official record volume 1796, page 2747 (1/2 interest) and deed volume 445, page 637 (1/2 interest) in the Fairfield County Recorder’s Office, 30.00 feet left of station 47+04.93;

Thence North 73 degrees, 35 minutes, 12 seconds East, 137.64 feet to a point 75.00 feet left of station 48+35.00;

Thence North 02 degrees, 40 minutes, 13 seconds East, 20.00 feet to a point 95.00 feet left of station 48+35.00;

Thence South 87 degrees, 19 minutes, 47 seconds East, 30.00 feet to a point 95.00 feet left of station 48+65.00;

Thence South 02 degrees, 40 minutes 13 seconds West, 43.38 Feet to the northerly line of Parcel 20-SH, 51.62 feet left of station 48+65.00;

Thence along the northerly line of Parcel 20-SH, South 84 degrees, 58 minutes, 38 seconds West, 161.53 feet to the Place of Beginning and containing 0.072 acres of land, subject to easements and restrictions of record.

Bearings are based on the positions of monuments located relative to grid north of the State Plane Coordinate System, Ohio South Zone (3402), NAD83(2011), (EPOCH 2010.0000) by GNSS measurements referenced to the ODOT CORS.

This description was prepared in February 2024 and is based upon surveys performed by Thomas Fok & Associates, Inc. beginning in April 2021 under the direction of Franklin D. Snyder, Jr., P.S. S-7468.