| Ver. Date | 03/12/2024 |  | PID |  |  | 111621 |
| --- | --- | --- | --- | --- | --- | --- |
| PARCEL |  |  |  | 30-T |  |  |
| FAI-158-4.20 |  |  |  |  |  |  |

TEMPORARY EASEMENT FOR THE PURPOSE OF PERFORMING THE WORK NECESSARY TO CONSTRUCT DRIVEWAY FOR MONTHS FROM DATE OF ENTRY BY THE STATE OF OHIO, DEPARTMENT OF TRANSPORTATION

[Surveyor’s description of the premises follows]

Situated in the State of Ohio, County of Fairfield, Township of Greenfield, in the southeast quarter of Section 14 in Township 15, Range XIX of the Congress Lands (East of Scioto River), lying on the left side of the centerline of right of way of Coonpath Road (County Road 31, 60.00 feet wide) and being part of the0.750 acres of land deeded, now or previously, to Bonnie L. Weaver in official record volume 1796, page 2747 (1/2 interest) and deed volume 445, page 637 (1/2 interest) of the Fairfield County Recorder’s Office, and known as Fairfield County Auditor’s parcel number 013-00375.00 and further described as follows;

Beginning at an iron pin set at the intersection of the existing northerly right of way line of Coonpath Road with the westerly line of the land deeded to Jeffery J. Gressick and Denise M. Gressick in official record volume 654, page 128 of the Fairfield County Recorder’s Office, 30.00 feet left of station 47+04.93;

Thence along the existing northerly right of way line of Coonpath Road, North 87 degrees, 19 minutes, 47 seconds West, 54.93 feet to a point 30.00 feet left of station 46+50.00;

Thence North 02 degrees, 40 minutes, 13 seconds East, 25.00 feet to a point 55.00 feet left of station 46+50.00;

Thence South 87 degrees, 19 minutes, 47 seconds East, 54.99 feet to the westerly line of Gressicks’ land, 55.00 feet left of station 47+04.99;

Thence along the westerly line of Gressicks’ land, South 02 degrees, 48 minutes, 45 minutes West, 25.00 feet to the Place of Beginning and containing 0.031 acres of land, none of which is within the present road occupied by Coonpath Road, subject to easements and restrictions of record.

The iron pin described as set is to be a ¾” x 30” reinforcing bar with a 2” aluminum cap stamped ODOT R/W S-7468 THOMAS FOK & ASSOC and will be placed upon notification by ODOT District 5.

Bearings are based on the positions of monuments located relative to grid north of the State Plane Coordinate System, Ohio South Zone (3402), NAD83(2011), (EPOCH 2010.0000) by GNSS measurements referenced to the ODOT CORS.

This description was prepared in February 2024 and is based upon surveys performed by Thomas Fok & Associates, Inc. beginning in April 2021 under the direction of Franklin D. Snyder, Jr., P.S. S-7468.