# UTILITIES

THERE ARE NO UNDERGROUND UTILITIES SHOWN ON THIS PLAN. THE NATURE

OF WORK REQUIRED BY THIS PROJECT WILL NOT AFFECT ANY KNOWN UNDERGROUND UTILITIES THAT EXIST UNDER OR ADJACENT TO THE WORK

AREA.

# CONTINGENCY QUANTITIES

THE CONTRACTOR SHALL NOT ORDER MATERIALS OR PERFORM WORK FOR

ITEMS DESIGNATED BY PLAN NOTE TO BE USED "AS DIRECTED BY THE ENGINEER" UNLESS AUTHORIZED BY THE ENGINEER.  THE ACTUAL WORK LOCATIONS AND QUANTITIES USED FOR SUCH ITEMS SHALL BE INCORPORATED INTO THE FINAL CHANGE ORDER GOVERNING COMPLETION OF THIS PROJECT.

# PAVEMENT MARKINGS

ALL LONG LINE AND AUXILIARY PAVEMENT MARKINGS (STOP LINES, CROSSWALK LINES, CHANNELIZING LINES, ETC.) SHOWN IN THE PLANS ARE TAKEN FROM EXISTING LOCATIONS. THE CONTRACTOR SHALL DOCUMENT ALL PAVEMENT MARKING LOCATIONS THAT WILL BE REMOVED/ OBLITERATED DURING THIS PROJECT AND PLACE MARKINGS AT THE LOCATION OF THE EXISTING MARKINGS, UNLESS SHOWN OR STATED DIFFERENTLY IN THE PLANS AND/OR DIRECTED OTHERWISE BY THE ENGINEER. THE CONTRACTOR SHALL NOTIFY THE PROJECT ENGINEER A MINIMUM OF 24 HOURS PRIOR TO APPLYING PAVEMENT MARKING MATERIALS ON ANY ROUTES SO THAT ODOT PERSONNEL MAY BE PRESENT DURING PAVEMENT MARKING OPERATIONS.

PER CMS 641.04, THE CONTRACTOR SHALL PROVIDE ODOT PERSONNEL A

COPY OF THE DLS SHORT REPORT AT THE END OF EVERY WORKDAY THROUGH CLOUD BASED REPORTING. THE CONTRACTOR SHALL NOT RECEIVE PAYMENT

FOR ANY WORK DONE WITHOUT NOTIFICATION AS STATED ABOVE OR IF DSL SHORT REPORTS ARE NOT PROVIDED DAILY. SEPARATE DLS REPORTS SHALL BE PROVIDED FOR EVERY RUN OF LONG LINE MARKING PLACED. COMBINED DLS REPORTS (I.E. CENTER LINE AND EDGE LINE) WILL NOT BE ACCEPTED OR PAID

BY THE DEPARTMENT.

ENVIRONMENTAL NOTES

THE PROPOSED PROJECT INVOLVES WORK OVER STREAMS. NO MATERIAL INCLUDING CONSTRUCTION DEBRIS, EARTHEN DEBRIS AND EXCESS ASPHALT

OR CONCRETE MAY ENTER THE STREAMS AT ANY TIME, AND NO WORK IS PERMITTED BELOW THE OHWM IN ANY AQUATIC RESOURCE.

SEEDING AND MULCHING

DUE TO LINEAR GRADING, THE FOLLOWING QUANTITIY IS BEING PROVIDED

TO PROMOTE GROWTHAND CARE OF PERMANENT SEEDED AREAS OUTSIDE

THE COMPACTED AGGREGATE SHOULDER. SEEDING AND MULCHING

SHALL BE PERFORMED NO LATER THAN 7 DAYS AFTER LINEAR GRADING OPERATIONS. SEEDING IS NOT REQUIRED BEHIND GUARDRAIL SECTIONS.

GUE-70 EB/WB: 20.75 MILES X 6’ WIDTH = 73,040 SY (LT & RT SIDES)

RAMPS: 1.42 MILES X 6’ WIDTH = 4,999 SY (LT & RT SIDES)

GUARDRAIL DEDUCTIONS: 18,000’ X 6’ WIDTH = 12,000 SY (APPROX.)

ITEM 659, SEEDING AND MULCHING, CLASS 2

LOCATION 1:  66,039 SY

ITEM 209, LINEAR GRADING, AS PER PLAN

IN ORDER TO PROVIDE POSITIVE DRAINAGE FROM THE ROADWAY SURFACE,

THE EXISTING ROADWAY SHOULDERS SHALL BE GRADED A MINIMUM OF 10’ FROM THE EXISTING PAVED SHOULDERS AND SHAPED USING A GRADER OF ADEQUATE SIZE TO PERFORM THE WORK TO THE SATISFACTION OF THE

ENGINEER. grading may BE REDUCED TO 4’ IN GUARDRAIL SECTIONS,

as directed by the engineer,

ALL EXCESS MATERIAL REMAINING AFTER THE GRADER WORK IS COMPLETED SHALL BE REMOVED AND DISPOSED OF OFF-SITE BY THE CONTRACTOR. ALL

LINEAR GRADING WORK SHALL BE COMPLETED PRIOR TO PLACEMENT OF

THE ASPHALT SURFACE COURSE. LINEAR GRADING HAS BEEN DEDUCTED

AND WILL NOT BE PAID IN BRIDGE SECTIONS.

NON-PERFORM LINEAR GRADING IN MONITORING WELL SECTION ON

GUE-70 WESTBOUND FROM SLM 14.06 TO 14.41.

ALL EQUIPMENT, LABOR, DISPOSAL, AND INCIDENTALS REQUIRED TO COMPLETE THIS ITEM AS DESCRIBED ABOVE SHALL BE INCLUDED FOR PAYMENT IN THE UNIT PRICE BID FOR THE ABOVE-DESCRIBED PURPOSE.

GUE-70 EB: 10.02 (LT & RT SIDES)	WB: 9.31 MILES (LT & RT SIDES)

RAMPS: 1.42 MILES (LT & RT SIDES)

ITEM 209, LINEAR GRADING, AS PER PLAN

LOCATION 1:  20.75 MILES

ITEM 253, PAVEMENT REPAIR

AN ESTIMATED QUANTITY FOR PAVEMENT REPAIR HAS BEEN INCLUDED IN

THE PLAN TO BE USED AS DIRECTED BY THE ENGINEER. REPAIRS SHALL

TAKE PLACE PRIOR TO ANY PLANING OPERATIONS.

THE INTENT OF THIS OPERATION IS TO REPAIR THOSE AREAS OF PAVEMENT

WHICH HAVE COMPLETELY FAILED (PUMPING OF SUB-BASE MATERIAL) AND

NOT TO CORRECT SURFACE IRREGULARITIES. DEPTH OF EXCAVATION SHALL

BE 7". THE MINIMUM WIDTH SHALL BE 4 FT. AFTER EXCAVATION HAS BEEN COMPLETED, THE FACE OF THE REPAIR SHALL BE COATED WITH 407 TACK COAT. REPLACEMENT MATERIAL WILL BE 7" OF ITEM 301 ASPHALT CONCRETE BASE, PG64-22 (PLACED, COMPACTED, AND TACKED IN TWO LIFTS). REPAIR QUANTITIES MAY BE USED ON THE MAINLINE PAVEMENT OR ON PAVED SHOULDERS. ALL EXCAVATION, MATERIALS, LABOR, EQUIPMENT, TOOLS, AND INCIDENTALS

NEEDED TO COMPLETE THE WORK DESCRIBED ABOVE SHALL BE PAID FOR

UNDER ITEM 253, PAVEMENT REPAIR (A).

ITEM 253, PAVEMENT REPAIR

LOCATION 1:   250 CU.YD.

ITEM 254, PATCHING PLANED SURFACE

THIS ITEM IS BEING PROVIDED TO FILL SPALLED/ UNSOUND PAVEMENT OF UNDERLYING COURSES AFTER COMPLETION OF ITEM 254, PAVEMENT

PLANING AND AHEAD OF THE PLACEMENT OF ITEM 442, SURFACE COURSE

PER 254.04, AS DIRECTED BY THE ENGINEER. THIS OPERATION SHALL OCCUR CONCURRENTLY WITH MILLING AND PAVING OPERATIONS. THE BELOW QUANTITY IS BEING CARRIED TO THE GENERAL SUMMARY FOR THE ABOVE-DESCRIBED PURPOSE (APPROX. 5% OF PAVEMENT PLANING AREA).

ITEM 209, PATCHING PLANED SURFACE

LOCATION 1:  7,081 SQ.YD.

ITEM 254, PAVEMENT PLANING, ASPHALT CONCRETE, BY DEPTH

DEPTH OF PLANING SHALL BE AS SHOWN ON THE PAVEMENT AND SHOULDER DATA TABLES. PLANING SHALL BE THE FULL WIDTH OF THE EXISTING PAVEMENT, INCLUDING PAVED SHOULDERS. THE ROADWAY SHALL BE

PLANED SUCH THAT POSITIVE DRAINAGE IS CREATED FROM THE CENTER LINE

TO THE EDGE OF PAVEMENT IN TANGENT SECTIONS AND SHALL FOLLOW EXISTING SUPERELEVATIONS WHERE APPLICABLE. ALL REQUIREMENTS OF

ITEM 254 SHALL APPLY.

IF DURING PLANING OPERATIONS EXCESSIVE RIDGES OR OTHER IRREGULARITIES ARE FOUND, PLANING DEPTH ADJUSTMENTS SHALL

BE MADE UP TO 3/8 INCH, AS DIRECTED BY THE ENGINEER. PAYMENT

SHALL BE INCLUDED IN THE UNIT PRICE BID PER CMS 254.07.

# ITEM 407, NON-TRACKING TACK COAT

THE RATE OF APPLICATION OF THE ITEM 407, NON-TRACKING TACK

COAT SHALL BE PER CMS TABLE 407.06-1 AND SUBJECT TO ADJUSTMENT

AS DIRECTED BY THE ENGINEER.  PLAN QUANTITIES INDICATE AN

AVERAGE APPLICATION RATE OF 0.08 GAL/SY FOR TACK COAT UNDER

SURFACE COURSE, (FOR ESTIMATING PURPOSES ONLY).

# ITEM 408, PRIME COAT, AS PER PLAN

THE CONTRACTOR SHALL APPLY ONE COAT OF MC-70 (AS PER CMS 702)

AT A RATE OF 0.40 GAL/SY TO THE COMPLETED AGGREGATE SHOULDER.

TO REDUCE THE AGGREGATE LOSS, THE PRIME COAT SHALL BE APPLIED

WITHIN SEVEN (7) DAYS AFTER PLACEMENT OF THE AGGREGATE SHOULDER OR LIQUATED DAMAGES PER CMS 108.07 WILL BE ASSESSED. THE CONTRACTOR SHALL PROVIDE A SHIELD TO PREVENT THE SPRAYING OR DRIFTING OF LIQUID BITUMINOUS MATERIAL ONTO THE EDGE OF PAVEMENT OR EDGE LINE. THE ATTENTION OF THE CONTRACTOR IS DIRECTED TO 107.10

OF THE SPECIFICATIONS.

# ITEM 516, 2" DEEP JOINT SEALER, AS PER PLAN (A)

THE CONTRACTOR SHALL PLACE A 1" X 2.0" DEEP BEAD OF JOINT SEALER

(AS PER 705.04) AT THE LOCATIONS SHOWN IN PLANS. THE CONTRACTOR

SHALL SAW CUT A CHANNEL FOR THE JOINT SEALER. THE COST FOR SAW

CUTTING THE CHANNEL FOR THE JOINT SEALER SHALL BE INCLUDED FOR

PAYMENT WITH ITEM 516, 2" DEEP JOINT SEALER, AS PER PLAN (A).

# ITEM 617, COMPACTED AGGREGATE, AS PER PLAN

ALL AGGREGATE SHALL BE EITHER 100% CRUSHED LIMESTONE OR RECYCLED ASPHALT CONCRETE PAVEMENT (RAP) MEETING REQUIREMENTS OF 703.18.

ALL AREAS SHALL BE LOOSENED AND FREE OF VEGETATION PER 617.04 PRIOR

TO PLACEMENT OF COMPACTED AGGREGATE. AGGREGATE SHOULDERS SHALL

BE SLOPED TO PROVIDE POSITIVE DRAINAGE AWAY FROM THE ROADWAY.

SHOULDER PREPARATION SHALL BE INCLUDED FOR PAYMENT IN THE

UNIT PRICE BID FOR ITEM 617, COMPACTED AGGREGATE, AS PER PLAN.

# ITEM 621, RAISED PAVEMENT MARKER REMOVED

# RPM REMOVAL SHALL NOT OCCUR SOONER THAN 10 DAYS PRIOR TO RESURFACING OF THE ROADWAY.  ALL RPM'S REMOVED SHALL BECOME

THE PROPERTY OF THE CONTRACTOR.

# ITEM 647, PAVEMENT MARKING MISC.: WRONG WAY ARROW, TYPE B90, CONTRAST

# THE ABOVE ITEM SHALL ADHERE TO CMS 647 AND INCLUDE A BLACK CONTRAST-ENHANCING SYSTEM. THE BLACK CONTRAST MARKING SHALL EXTEND A

# MINIMUM OF 3” PAST THE ORIGINAL DIMENSIONS OF THE WHITE PAVEMENT MARKING.

PROTECTION OF TRAFFIC MONITORING EQUIPMENT

PRIOR TO BEGINNING ANY PAVEMENT PLANING ACTIVITIES ON GUE-70

AT SLM 14.00 westbound: THE CONTRACTOR, THE PROJECT ENGINEER,

AND A REPRESENTATIVE FROM THE OWNER SHALL COORDINATE A TIME

FOR THE OWNER/MAINTAINING AGENCY TO DISCONNECT THE EQUIPMENT.

FOLLOWING THE DISCONNECTION BY THE OWNER, THE CONTRACTOR

WILL BE ALLOWED TO PERFORM THEIR PAVEMENT PLANING ACTIVITIES.

THE REMOVED LOOPS AND SENSORS WILL BECOME THE PROPERTY OF

THE CONTRACTOR TO DISPOSE OFF-SITE.

OWNER CONTACT INFORMATION:

ODOT, OFFICE OF TECHNICAL SERVICES, TRAFFIC MONITORING SECTION

1980 WEST BROAD STREET, COLUMBUS, OHIO 43223

ELIZABETH MURPHY

Elizabeth.Murphy@dot.ohio.gov

614-204-0291

DURING THE MEETING, THE OWNER/MAINTAINING AGENCY WILL IDENTIFY EQUIPMENT LOCATIONS. DO NOT DISTURB PULL BOXES, CONTROLLERS, CABINENTS, POLES AND CONDUITS. ANY DAMAGE WILL BE THE

RESPONSIBILITY OF THE CONTRACTOR AND REPAIRS MUST BE ACCEPTED

BY THE OWNER.

# ITEM SPECIAL, PAVER MOUNTED THERMAL PROFILING (PMTP)

# This item consists of providing a paver mounted thermal profiling (PMTP) system to identify the presence of any thermal segregation

# of an uncompacted mat of hot mix asphalt. Conform to the specifications found in the special provision for methods and procedures for determining the thermal profile using a PMTP system.

# Notify the ODOT Office of Pavement Engineering (OPE) at least two weeks prior to the start of PMTP data collection.

# OPE will conduct density gauge testing on some or all of thermal profile data sublots categorized as having severe thermal segregation, as defined in the special provision. The results of this testing are for research purposes and will not result in any additional adjustment to the item unit bid price. Provide traffic control to allow OPE to conduct testing. The Department will not require more than four days of additional lane closures to perform density gauge testing.

# Include the cost of all labor, equipment, software, and incidentals necessary to install the equipment and analyze the data in the Lump Sum bid for Item Special, Paver Mounted Thermal Profiling (PMTP).

# ITEM SPECIAL, INTELLIGENT COMPACTION (IC)

This item consists of providing an intelligent compaction (IC) system to monitor compaction during placement of asphalt concrete. Instrument all rollers involved in the compaction of the asphalt concrete mat. Conform to the specifications found in the special provision.

Notify the ODOT Office of Pavement Engineering (OPE) at least two weeks prior to the start of IC data collection.

Include the cost of all labor, equipment, software, and incidentals necessary to install the equipment and analyze the data in the Lump Sum bid for Item Special, Intelligent Compaction (IC).

ITEM 204, SUBGRADE COMPACTION AND PROOF ROLLING

CONSTRUCT THE SUBGRADE AS FOLLOWS AND, IN THE FOLLOWING SEQUENCE:

- SHAPE THE SUBGRADE TO WITHIN 0.2 FEET OF THE PLAN

SUBGRADE ELEVATION.

- EXCAVATE AND REPLACE UNSUITABLE SUBGRADE BEFORE PROOF ROLLING.  THE EXCAVATION LIMITS ARE SHOWN AND LABELED ON THE CROSS SECTIONS AS UNSUITABLE SUBGRADE.  UNSUITABLE SUBGRADE INCLUDES UNSUITABLE SOIL (A-4B, A-2-5, A-5, A-7-5, AND SOIL WITH A LIQUID LIMIT GREATER THAN 65) AND ANY COAL, SHALE, OR ROCK WHICH NEEDS TO BE REMOVED ACCORDING TO CMS 204.05. IF THERE IS UNSUITABLE SUBGRADE IN A SHALLOW FILL LOCATION, EXCAVATE AND REPLACE THE UNSUITABLE SUBGRADE BEFORE CONSTRUCTING THE SHALLOW FILL AND SHAPING THE SUBGRADE. (NOT APPLICABLE)

- COMPACT THE SUBGRADE ACCORDING TO CMS 204.03.

- APPROXIMATE LIMITS FOR EXCAVATION OF UNSTABLE SUBGRADE ARE SHOWN BELOW. THE ENGINEER WILL IDENTIFY THE ACTUAL LIMITS OF EXCAVATION FOR UNSTABLE SUBGRADE BASED ON THE PROOF ROLLING RESULTS AND VISUAL OBSERVATIONS.PROOF ROLL THE COMPACTED SUBGRADE ACCORDING TO CMS 204.06.

- EXCAVATE UNSTABLE SUBGRADE AS DIRECTED BY THE ENGINEER AND STABILIZE BY REPLACING WITH THE SPECIFIED MATERIALS ACCORDING TO CMS 204.07.  EXCAVATIONS WILL EXTEND 18 INCHES BEYOND THE EDGE OF THE SURFACE OF THE PAVEMENT, PAVED SHOULDERS, OR PAVED MEDIANS.

- PROOF ROLL THE STABILIZED AREAS ACCORDING TO

CMS 204.06 TO VERIFY STABILITY.

- FINE GRADE THE SUBGRADE TO THE SPECIFIED GRADE.

THE QUANTITIES FOR EXCAVATING THE UNSUITABLE SUBGRADE AND UNSTABLE SUBGRADE ARE BOTH PAID UNDER ITEM 204, EXCAVATION OF SUBGRADE.

UNSUITABLE SUBGRADE: N/A

UNSTABLE SUBGRADE: SEE BELOW CONTINGENCY QUANTITIES FOR

S.R. 285 RAMPS, AS DIRECTED BY THE ENGINEER.

# ITEM 204, EXCAVATION OF SUBGRADE

# LOCATION 1:   150 CY

# ITEM 204, GRANULAR MATERIAL TYPE C

# LOCATION 1:  150 CY

# ITEM 204, PROOF ROLLING

# LOCATION 1:  2 HOUR

# ITEM 204, GEOTEXTLE FABRIC

# LOCATION 1:  450 SY

ITEM 442, ASPHALT CONCRETE SURFACE COURSE, 12.5 MM, TYPE A (447),

PWL, 2025, AS PER PLAN

All requirements of CMS Item 442 apply except as shown.

Mat Density Acceptance - Follow the requirements of 447 Mat Density Acceptance, except as modified below.

Obtain 6-inch diameter cores for each Lot.

The PWL calculator, located on the ODOT website at the Office of Construction Administration, will be used to determine the Lot PWL and the Lot AASHTO pay factors.

https://www.dot.state.oh.us/Divisions/ConstructionMgt/Admin/Pages/asphalt-concrete.aspx

The Department will determine the pay factor for each Lot cored by the following tables.

| Lower Specification Limit | SURFACE WITH NO JOINT CORES PAY FACTOR CRITERIA | Pay Factor (PF) |
| --- | --- | --- |
| 92.6% | If AVE density is ≥ 93% AND PWL ≥ 80 | PF =1 or AASHTO PF whichever is greater |
|  | If 80 > PWL > 50 | AASHTO PF |
|  | If PWL ≤ 50 | REMOVE AND REPLACE |

| LOWER SPECIFICATION LIMIT | SURFACE WITH 3 JOINT CORES PAY FACTOR CRITERIA | PAY FACTOR (PF) |
| --- | --- | --- |
| 92% | IF AVE DENSITY IS ≥ 92.4% AND PWL ≥ 80 | PF =1 OR AASHTO PF WHICHEVER IS GREATER |
|  | IF 80 > PWL > 50 | AASHTO PF |
|  | IF PWL ≤ 50 | REMOVE AND REPLACE |

Provide location reference coordinates for each core taken for mat and joint density acceptance. Utilize a Global Navigation Satellite System (GNSS) receiver with an accuracy of ± 2 inches (50 mm) in the X and Y directions. Use a GNSS with the ability to obtain positional corrections using the ODOT Real Time Network (RTN) or a Base-Rover real-time kinematic (RTK) system. Tie data to the most current version of the National Spatial Reference System (NSRS) through Ohio County Coordinate System (OCCS) Coordinates. Use the RTN only when coverage is available throughout the project limits.

PAYMENT FOR ABOVE-DESCRIBED WORK SHALL BE INCLUDED ITEM 442, ASPHALT CONCRETE SURFACE COURSE, 12.5 MM, TYPE A (447), PWL, 2025, AS PER PLAN