# Initial Parameters

## Traffic Data:

|  | LAW-650 |
| --- | --- |
| Current Year ADT (2025) | 464 |
| Opening Year ADT (2027) | 464 |
| Design Year ADT (2047) | 464 |
| Design Hourly Volume (2047) | 54 |
| Opening Year | 2027 |
| Directional distribution | 60% |
| Trucks (24-hour B/C) | 0.03 |
| Design Speed | 55mph |
| Legal Speed | 55 mph (posted) |
| Functional Class | Minor Collector (6) |

## ODOT Pavement Design Manual Values

|  | LAW-650 |
| --- | --- |
| B:C Ratio | 2:1 |
| Lane Factor | 100% |
| Directional Distribution | 50% |
| ESAL Conversion Factor (B) | 0.79 |
| ESAL Conversion Factor (C) | 0.48 |
| Reliability | 85% |
| Standard Deviation | 0.49 |
| Design Serviceability loss | 2.0 |

# Calculations

Using LAW-650 values, Calculations were complete for Mid-year 18 kip ESAL and Subgrade resilient modulus (Mr)

## Mid-year ESAL (B&C)

From ADT (2027) to Mid-year ADT (2037) = 464 + (464-464)*(10/20)

2041 ADT = 464

ESAL Equation = 464 * %T24 * %D * %LF * % (B or C) * CF

ESAL (B trucks) = 464 * (0.03) * 0.6 * 1.0 * (2/3) * (0.79) = 4.40

ESAL (C trucks) = 464 * (0.03) * 0.6 * 1.0 * (1/3) * (0.48) = 1.34

Total Daily ESAL = 4.40 + 1.40 = 5.74 ESAL/Day

Design ESAL = Total Daily ESAL * 365.25 day/yr * Design Period (20 years)

Design ESAL = 5.74* 365.25 * 20 = 41930.7 = 0.041 * 10^6 ESAL

ESAL =0.05*106  (Will use minimum value)

## Subgrade Resilient Modulus (Mr)

No Cement Stabilization

Mr = 1200 * CBR

CBR = California Bearing Ratio = 7

Mr = 1200 * 7 = 8400 psi

## Structure Number (SN)

Using the values of ESAL, Mr, Reliability and Overall Std. Dev, the Structure Number is determined using PDM figures 402-2 and 402-3. See Below:

# Preliminary Pavement Buildup

Using PDM figure 401-1 and guidance in section 400, a preliminary pavement buildup was determined to get a SN above the required SN of 1.9.

Opening Day Truck volume (2027) = 464*0.03 = 14 trucks/day

Table 1: Preliminary Pavement Buildup

| Item | Thickness | Coefficient | SN |
| --- | --- | --- | --- |
| Item 441 Asphalt Concrete Surface Course, Type 1, (448), PG64-22 | 1.25” | 0.43 | 0.54 |
| Item 441 Asphalt Concrete Intermediate Course, Type 1, (448) | 1.75” | 0.43 | 0.75 |
| Item 301 Asphalt Concrete Base (448) | 4” | 0.36 | 1.44 |
| Item 304 Aggregate Base | 6’’ | 0.14 | 0.84 |
|  |  | Total | 3.57 |