GENERAL

ROUNDING

THE ROUNDING AT SLOPE BREAKPOINTS SHOWN ON THE TYPICAL SECTIONS APPLIES TO ALL CROSS-SECTIONS, EVEN THOUGH OTHERWISE SHOWN.

UTILITIES

THE CONTRACTOR SHALL USE THE FOLLOWING PROCEDURE AT EACH LOCATION WHERE WORK IS PERFORMED, IN ACCORDANCE WITH SECTIONS 105.07 AND 107.16 IN THE CONSTRUCTION AND MATERIALS SPECIFICATIONS:

THE CONTRACTOR SHALL NOTIFY THE PROJECT ENGINEER, OHIO811, THE OHIO DEPARTMENT OF TRANSPORTATION DISTRICT 4 HEADQUARTERS (MICHELLE CHANEY AT 330-786-2267) AND ALL NON REGISTERED UTILITY OWNERS AT LEAST TWO (2) WORKING DAYS PRIOR TO COMMENCING CONSTRUCTION OPERATIONS IN ALL AREAS.

THE LOCATION OF THE UNDERGROUND UTILITIES SHOWN ON THE PLANS ARE AS OBTAINED FROM THE OWNERS AS REQUIRED BY SECTION 153.64 O.R.C.

LISTED BELOW ARE ALL UTILITIES LOCATED WITHIN THE PROJECT CONSTRUCTION LIMITS TOGETHER WITH THEIR RESPECTIVE OWNERS:

ELECTRIC AND TRANSMISSIONS:

HUDSON PUBLIC POWER

1769 GEORGETOWN ROAD

HUDSON, OH 44236

ATTN: KEVIN POWELL

PHONE: (330) 342-1755

GAS:

DOMINION ENERGY OHIO

320 SPRINGSIDE DRIVE

AKRON, OH 44333

ATTN: JOSHUA J. MIS

PHONE: (330) 571-9135

TELECOMMUNICATIONS:

CHARTER

1200 BROWNSTONE AVE.

AKRON, OH 44310

ATTN: JIM LONG

PHONE: (330) 622-4106

WINDSTREAM

245 NORTH MAIN STREET

HUDSON, OH 44236

ATTN: BRIAN VINSON

PHONE: (330) 422-9128

WATER:

HUDSON PUBLIC WORKS – WATER

1769 GEORGETOWN ROAD

HUDSON, OH 44236

ATTN: SCOTT ANGEL

PHONE: (330) 342-1751

STORM SEWER:

CITY OF HUDSON

1769 GEORGETOWN ROAD

HUDSON, OH 44236

ATTN: ERIC HUTCHINSON

PHONE: (330) 342-1750

SANITARY SEWER:

SUMMIT COUNTY DEPARTMENT OF SANITARY SEWER SERVICES

1180 SOUTH MAIN STREET

AKRON, OH 44301

ATTN: MIKE VINAY

PHONE: (330) 926-2567

AGENCIES:

CITY OF HUDSON – ENGINEERING

1140 TEREX ROAD

HUDSON, OH 44236

ATTN: BRAD KOSCO, CITY ENGINEER

PHONE: (330) 342-1770

NORFOLK SOUTHERN CORPORATION

ELDRIDGE W. CHAMBERS

ENGINEER PUBLIC IMPROVEMENTS

NORFOLK SOUTHERN CORPORATION

650 PEACHTREE STREET, NW, BOX 45

ATLANTA, GA 30308

(404) 463-6307

ELDRIDGE.CHAMBERS@NSCORP.COM

PROJECT LOCATION: MIDWEST DIVISION MP RD-96.64

USE EMAIL SUBJECT LINE: HUDSON, OH – NS FILE

BR0027698 – SR 91 REHAB – MIDWEST DIV. MP RD-96.64

METRO REGIONAL TRANSIT AUTHORITY

416 KENMORE BOULEVARD

AKRON, OH 44301

ATTN: VALERIE SHEA

PHONE: (330) 762-0341

THE UNDERGROUND UTILITIES ON THIS PLAN HAVE BEEN LOCATED BY USING A SUBSURFACE UTILITY ENGINEERING COMPANY [SUE]. IF THERE ARE ANY DISCREPANCIES BETWEEN FIELD MARKINGS AND WHAT THE PLAN INDICATES, PLEASE CONTACT MATTHEW STEELE, DISTRICT UTILITY COORDINATOR 330-786-4832, PRIOR TO ANY SUBSURFACE WORK BEING INITIATED.

NORFOLK SOUTHERN RAILROAD

“ONE CALL” SERVICES DO NOT LOCATE BURIED RAILROAD SIGNAL AND COMMUNICATIONS LINES. THE CONTRACTOR SHALL CONTACT THE RAILROAD’S REPRESENTATIVE 2-DAYS IN ADVANCE OF WORK AT THOSE PLACES WHERE EXCAVATION, PILE DRIVING, OR HEAVY LOADS MAY DAMAGE THE RAILROAD'S UNDERGROUND FACILITIES. UPON REQUEST FROM THE CONTRACTOR OR SPONSOR, RAILROAD FORCES WILL LOCATE AND PAINT MARK OR FLAG THE RAILROAD'S UNDERGROUND FACILITIES.

DOMINION ENERGY

The Company has facilities within the project limits as shown on the plans. The Company will be relocating their facilities for this project as follows:

Note

Station/Offset		Station/Offset	Depth

Cut & Cap

765+18.0, 21.0’ Lt

Tie-in to Existing

765+23.0, 16.3’ Lt	to	765+23.0, 23.8’ Lt	36”

NEW

766+74.5, 17.2’ Lt	to	766+74.5, 23.4’ Lt	36”

NEW

766+81.0, 14.7’ Lt	to	766+81.0, 23.3’ Lt	36”

Tie-in to Existing

766+93.5, 14.8’ Lt	to	766+93.5, 23.3’ Lt	36”

Cut & Cap

766+93.5, 22.0’ Lt

Dominion will relocate 192’ of 6” MP gas main with approx. 170’ of 6” MP steel gas main due to conflict with the proposed storm catch basin and curb.

The Company does not anticipate any other conflicts with proposed work at this time. If a conflict does arise, the State’s Highway Contractor will immediately contact the Company and the ODOT Project Engineer so the proper actions can be taken to prevent delay of construction.

The State’s Highway Contractor shall use caution when digging near the Company’s facilities including but not limited to test boxes, curb stops, gate valve boxes, mainline and services. Costs incurred by damages to the Company’s facilities could be billed to damaging party.

CONSTRUCTION NOISE

ACTIVITIES AND LAND USE ADJACENT TO THIS PROJECT MAY BE AFFECTED BY CONSTRUCTION NOISE. IN ORDER TO MINIMIZE ANY ADVERSE CONSTRUCTION NOISE IMPACTS, DO NOT OPERATE POWER-OPERATED CONSTRUCTION-TYPE DEVICES BETWEEN THE HOURS OF 7:00 PM AND 7:00 AM. IN ADDITION, DO NOT OPERATE AT ANY TIME ANY DEVICE IN SUCH A MANNER THAT THE NOISE CREATED SUBSTANTIALLY EXCEEDS THE NOISE CUSTOMARILY AND NECESSARILY ATTENDANT TO THE REASONABLE AND EFFICIENT

PERFORMANCE OF SUCH EQUIPMENT.

WORK LIMITS

THE WORK LIMITS SHOWN ON THESE PLANS ARE FOR PHYSICAL CONSTRUCTION ONLY. PROVIDE THE

INSTALLATION AND OPERATION OF ALL WORK ZONE TRAFFIC CONTROL AND WORK ZONE TRAFFIC CONTROL DEVICES REQUIRED BY THESE PLANS WHETHER INSIDE OR OUTSIDE THESE WORK LIMITS.

CLEARING AND GRUBBING

ALTHOUGH THERE ARE NO TREES OR STUMPS SPECIFICALLY MARKED FOR REMOVAL WITHIN THE LIMITS OF THE PROJECT, A LUMP SUM QUANTITY IS INCLUDED IN THE GENERAL SUMMARY FOR ITEM 201, CLEARING AND GRUBBING. ALL PROVISIONS AS SET FORTH IN THE SPECIFICATIONS UNDER THIS ITEM ARE INCLUDED IN THE LUMP SUM PRICE BID FOR ITEM 201, CLEARING AND GRUBBING.

ITEM 202 – REMOVAL MISC.: RAILING REMOVED AND SALVAGED

DESCRIPTION: THIS WORK SHALL CONSIST OF REMOVAL OF THE EXISTING RAILING LOCATED ON THE WEST SIDE OF THE SIDEWALK UNDERNEATH THE EXISTING SUM-91-14.51 STRUCTURE. THE RAILINGS WILL BE CAREFULLY REMOVED, LOADED, TRANSPORTED AND OFFLOADED.

THE REMOVAL OF THE WEST SIDE RAILINGS WILL REQUIRE THE WEST SIDE CURB AND WALK TO BE REMOVED AND REPLACED. THE CURB AND WALK REMOVAL AND REPLACEMENT WILL BE PAID SEPARATELY.

THE SALVAGED RAILINGS ARE TO BE DELIVERED TO:

HUDSON PUBLIC WORKS DEPARTMENT

1769 GEORGETOWN ROAd, Hudson, OH 44236

Contact: DAN ROTH

Phone: (330) 342-9246

MEASUREMENT AND PAYMENT: THE DEPARTMENT WILL MEASURE THE QUANTITY OF REMOVALS ON A LUMP SUM BASIS. THE DEPARTMENT WILL PAY FOR THE ACCEPTED QUANTITIES OF REMOVALS AT THE CONTRACT PRICE FOR ITEM 202 – REMOVAL MISC.: RAILING REMOVED AND SALVAGED.

ITEM SPECIAL - VERTICAL CLEARANCE

AFTER ALL CONSTRUCTION HAS BEEN COMPLETED, A REGISTERED SURVEYOR WILL TAKE VERTICAL CLEARANCE MEASUREMENTS AT LOCATIONS INDICATED ON THE

APPROVED ODOT FORM (AVAILABLE IN THE DISTRICT 4 STRUCTURES AND PAVEMENT OFFICE). THE FINAL MEASUREMENTS SHALL BE RECORDED ON THE FORM AND

SUBMITTED TO THE PROJECT ENGINEER AND THE DISTRICT 4 STRUCTURES AND PAVEMENT ENGINEER. THE RECORD SHALL BEAR THE SEAL OF THE LICENSED SURVEYOR WHO HAS TAKEN THE MEASUREMENTS. THIS WORK SHALL BE PERFORMED AT THE FOLLOWING STRUCTURES:

SUM-91-1451: NORFOLK SOUTHERN RAILROAD OVER SR 91

THE FOLLOWING QUANTITY HAS BEEN CARRIED TO THE GENERAL SUMMARY:

SPECIAL - VERTICAL CLEARANCE, 1 EACH

| S. MAIN ST. (SR-091) |  | PROJECT COORDINATES (GROUND) |  | PROJECT COORDINATES (GRID) |  | ELEV. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| STATION | OFFSET | NORTH (Y) | EAST (X) | NORTH (Y) | EAST (X) | ELEV (Z) | DESCRIPTION |
| 762+43.26 | 41.95 | 573840.52 | 2260177.46 | 573782.10 | 2259947.38 | 1065.05 | PT. #114 - IRON PIN FOUND |
| 763+45.93 | 238.97 | 573945.79 | 2260373.11 | 573887.36 | 2260143.00 | 1065.13 | PT. #113 - IRON PIN FOUND |
| 764+14.08 | -40.74 | 574010.22 | 2260092.52 | 573951.78 | 2259862.44 | 1059.27 | PT. #2 - CONTROL POINT |
| 764+87.46 | 41.98 | 574084.69 | 2260174.26 | 574026.25 | 2259944.18 | 1056.26 | PT. #112 - IRON PIN FOUND |
| 766+31.7 | 29.58 | 574228.75 | 2260159.95 | 574170.29 | 2259929.86 | 1056.09 | PT #111 - IRON PIN FOUND |
| 766+99.7 | -42.52 | 574295.79 | 2260086.96 | 574237.33 | 2259856.88 | 1057.37 | PT. #6 - BENCH MARK, BM#1 |
| 767+77.57 | 30.99 | 574374.63 | 2260159.43 | 574316.16 | 2259929.35 | 1048.89 | PT. #7 - CONTROL POINT |
| 767+92.01 | 55.76 | 574389.40 | 2260184.01 | 574330.93 | 2259953.92 | 1056.72 | PT. #4 - CONTROL POINT |
| 767+56.6 | 332.46 | 574357.66 | 2260461.15 | 574299.19 | 2260231.04 | 1054.65 | PT. #8 - CONTROL POINT |
| 767+93.77 | -40.72 | 574389.88 | 2260087.51 | 574331.41 | 2259857.43 | 1056.41 | PT. #3 - CONTROL POINT |
| 768+62.97 | 31.68 | 574460.04 | 2260158.99 | 574401.56 | 2259928.90 | 1059.74 | PT. #5 - BENCH MARK, BM#2 |
| 769+28.2 | -235.32 | 574521.72 | 2259891.15 | 574463.23 | 2259661.09 | 1056.07 | PT. #109 - IRON PIN FOUND |
| 769+36.2 | 41.95 | 574533.40 | 2260168.29 | 574474.91 | 2259938.21 | 1058.04 | PT. #110 - IRON PIN FOUND |
| 770+84.26 | 204.91 | 574683.60 | 2260329.27 | 574625.09 | 2260099.17 | 1057.16 | PT. #100 - IRON PIN FOUND |
| 771+03.29 | 229.11 | 574702.86 | 2260353.23 | 574644.36 | 2260123.13 | 1057.13 | PT. #102 - MONUMENT BOX |
| 771+09.74 | 221.74 | 574709.30 | 2260345.77 | 574650.79 | 2260115.67 | 1056.64 | PT. #103 - MONUMENT BOX |
| 771+09.76 | 175.88 | 574708.71 | 2260299.91 | 574650.21 | 2260069.81 | 1058.21 | PT. #101 - PIN FOUND |
| 771+10.4 | -23.95 | 574706.70 | 2260100.09 | 574648.20 | 2259870.01 | 1058.80 | PT. #106 - IRON PIN FOUND |
| 771+10.55 | -148.79 | 574705.20 | 2259975.25 | 574646.70 | 2259745.19 | 1058.42 | PT. #108 - IRON PIN FOUND |
| 771+10.69 | -29.71 | 574706.92 | 2260094.32 | 574648.41 | 2259864.25 | 1059.21 | PT. #107 - IRON PIN FOUND |
| 771+39.96 | 131.50 | 574738.32 | 2260255.14 | 574679.81 | 2260025.04 | 1058.22 | PT. #104 - MONUMENT BOX |
| 771+71.78 | 45.89 | 574769.00 | 2260169.11 | 574710.49 | 2259939.02 | 1060.57 | PT. #1 - CONTROL POINT |
| 774+25.35 | 256.32 | 575025.34 | 2260376.17 | 574966.80 | 2260146.06 | 1073.32 | PT. # 117 - MONUMENT BOX |
| 774+26.46 | -718.01 | 575014.17 | 2259401.90 | 574955.63 | 2259171.89 | 1054.07 | PT. #118 - MONUMENT BOX |

SURVEYING PARAMETERS

PRIMARY PROJECT CONTROL MONUMENTS GOVERN ALL POSITIONING ON ODOT PROJECTS. SEE PREVIOUS SHEET FOR A TABLE CONTAINING PROJECT CONTROL INFORMATION.

USE THE FOLLOWING PROJECT CONTROL, VERTICAL POSITIONING, AND HORIZONTAL POSITIONING

PARAMETERS FOR ALL SURVEYING:

PROJECT CONTROL

POSITIONING METHOD: GPS

MONUMENT TYPE: 5/8” Rebar in Concrete

VERTICAL POSITIONING

ORTHOMETRIC HEIGHT DATUM: NAVD 88

GEOID: 2012B

HORIZONTAL POSITIONING

REFERENCE FRAME: NAD 83 (CORS 2011)

ELLIPSOID: GRS80

MAP PROJECTION: Lambert Conformal Conic

COORDINATE SYSTEM: Ohio North Zone (3401)

COMBINED SCALE FACTOR: 0.99989820

ORIGIN OF COORDINATE SYSTEM: 0,0

USE THE POSITIONING METHODS AND MONUMENT TYPE USED IN THE ORIGINAL SURVEY TO RESTORE ALL MONUMENTS RELATED TO PRIMARY PROJECT CONTROL THAT ARE DAMAGED OR DESTROYED BY CONSTRUCTION ACTIVITIES. RESTORE THE DAMAGED OR DESTROYED MONUMENTS IN ACCORDANCE WITH CMS 623.

UNITS ARE IN U.S. SURVEY FEET.

ITEM SPECIAL – SURVEY CONTROL VERIFICATION

THE CONTRACTOR SHALL PERFORM THIS WORK TO VERIFY THE PROVIDED SURVEY CONTROL. THE CONTRACTOR WILL PERFORM THE VERIFICATION USING ONE OF THE TWO METHODS BELOW DEPENDENT UPON THE CONTRACTOR’S CHOSEN MEANS OF SURVEY CONTROL TO BE USED ON THE PROJECT. THE WORK SHALL BE PERFORMED UNDER THE DIRECT SUPERVISION OF AN OHIO LICENSED SURVEYOR.

- IF USING GPS DEVICES TO ESTABLISH AND OR PROVIDE SUPPLEMENTAL HORIZONTAL AND VERTICAL SURVEY CONTROL

  - LOCATE VERTICAL CONTROL POINTS PROVIDED IN THE PLANS AND PERFORM A DIFFERENTIAL LEVEL CIRCUIT.

  - PERFORM A SITE CALIBRATION UTILIZING THE AVAILABLE HORIZONTAL AND VERTICAL CONTROL POINTS PROVIDED IN THE PLAN.

  - PROVIDE A REPORT, SIGNED BY AN OHIO LICENSED SURVEYOR, TO THE PROJECT ENGINEER COMPARING THE OBSERVED DATA TO THE PLAN DATA ALONG WITH A NARRATIVE DETAILING ANY DISCREPANCIES FOUND.

- IF USING CONVENTIONAL SURVEY INSTRUMENTATION TO ESTABLISH AND OR PROVIDE SUPPLEMENTAL HORIZONTAL AND VERTICAL SURVEY CONTROL

  - LOCATE VERTICAL CONTROL POINTS PROVIDED IN THE PLANS AND PERFORM A DIFFERENTIAL LEVEL CIRCUIT.

  - LOCATE AND OBSERVE ANGLE AND DISTANCE TO ALL AVAILABLE HORIZONTAL CONTROL POINTS PROVIDE IN THE PLAN

  - PROVIDE A REPORT, SIGNED BY AN OHIO LICENSED SURVEYOR, TO THE PROJECT ENGINEER COMPARING THE OBSERVED DATA TO THE PLAN DATA ALONG WITH A NARRATIVE DETAILING ANY DISCREPANCIES FOUND.

ALL MATERIALS, LABOR, EQUIPMENT, TOOLS, AND INCIDENTALS NECESSARY TO COMPLETE THIS WORK SHALL BE INCLUDED IN THE LUMP SUM BID ITEM.

PAVEMENT

PART-WIDTH CONSTRUCTION

BECAUSE OF THE NECESSITY TO BUILD THIS PROJECT UNDER TRAFFIC AND TO CONSTRUCT THE FULL PAVEMENT WIDTH IN STAGES, EXERCISE CARE TO PREVENT THE CONSTRUCTION OF A BUTT JOINT IN THE BASE COURSES. LAP LONGITUDINAL JOINTS AS SHOWN ON STANDARD CONSTRUCTION DRAWING BP-3.1.

PAVEMENT RESTORATION FOR PIPE INSTALLATIONS AND/OR REMOVALS

THE FOLLOWING QUANTITY HAS BEEN PROVIDED FOR PAVEMENT RESTORATION FOLLOWING INSTALLATION

AND/OR REMOVAL OF PIPES.

ITEM 301 - ASPHALT CONCRETE BASE, PG64-22, (449) 20 CU. YDS.

THE ABOVE QUANTITY IS BASED ON A 301 THICKNESS OF 9 INCHES AND A PAVEMENT RESTORATION WIDTH THAT INCLUDES THE TRENCH WIDTH PLUS TWO FEET ON EACH SIDE OF THE TRENCH.

PROVIDE ANY MATERIALS USED OUTSIDE THE LIMITS STATED ABOVE AT NO ADDITIONAL COST.

PAVEMENT RESTORATION FOR DRAINAGE STRUCTURE INSTALLATIONS

THE FOLLOWING QUANTITY IS PROVIDED FOR PAVEMENT RESTORATION FOLLOWING INSTALLATION OF ITEM 611, DRAINAGE STRUCTURES.

ITEM 301, ASPHALT CONCRETE BASE, PG64-22, (449) 5 CU. YDS.

THE ABOVE QUANTITY IS BASED ON A 301 THICKNESS OF 9 INCHES AND A WIDTH OF TWO FEET AROUND THE PERIMETER OF THE DRAINAGE STRUCTURE. PLACE 3 LIFTS OF 3 INCHES AND COMPACT EACH LIFT PRIOR TO PLACING THE NEXT LIFT.

PROVIDE ANY MATERIALS USED OUTSIDE THE LIMITS STATED ABOVE AT NO ADDITIONAL COST.

ITEM 441 - ASPHALT CONCRETE SURFACE COURSE, TYPE 1, (449), AS PER PLAN (PG64-22)

703.05 DO NOT USE COARSE AGGREGATE FROM A SOURCE DESIGNATED ‘SR’ OR ‘SRH’ ACCORDING TO THE OFFICE OF MATERIALS MANAGEMENT (OMM) IN ANY JOB MIX FORMULA (JMF) FOR THIS ITEM.

ITEM 609 – CURB, TYPE 6, AS PER PLAN

THE CURB HEIGHT SHALL BE 8” EXCEPT FOR THE TRANSITIONS BACK TO 6” AT THE BEGINNING AND END OF THE PROJECT AS SHOWN ON SHEETS 27 AND 28.

DRAINAGE

CROSSINGS AND CONNECTIONS TO EXISTING PIPES AND UTILITIES

WHERE PLANS PROVIDE FOR A PROPOSED CONDUIT TO BE CONNECTED TO, OR CROSS OVER OR UNDER AN EXISTING SEWER OR UNDERGROUND UTILITY, LOCATE THE EXISTING PIPES OR UTILITIES BOTH AS TO LINE AND GRADE BEFORE STARTING TO LAY THE PROPOSED CONDUIT.

IF IT IS DETERMINED THAT THE ELEVATION OF THE EXISTING CONDUIT, OR EXISTING APPURTENANCE TO BE CONNECTED, DIFFERS FROM THE PLAN ELEVATION OR RESULTS IN A CHANGE IN THE PLAN CONDUIT SLOPE, NOTIFY THE ENGINEER BEFORE STARTING CONSTRUCTION OF ANY PORTION OF THE PROPOSED CONDUIT WHICH WILL BE AFFECTED BY THE VARIANCE IN THE EXISTING ELEVATIONS.

IF IT IS DETERMINED THAT THE PROPOSED CONDUIT WILL INTERSECT AN EXISTING SEWER OR UNDERGROUND UTILITY IF CONSTRUCTED AS SHOWN ON THE PLAN, NOTIFY THE ENGINEER BEFORE STARTING CONSTRUCTION OF ANY PORTION OF THE PROPOSED CONDUIT WHICH WOULD BE AFFECTED BY THE INTERFERENCE WITH AN EXISTING FACILITY.

PAYMENT FOR ALL THE OPERATIONS DESCRIBED ABOVE IS INCLUDED IN THE CONTRACT PRICE FOR THE PERTINENT 611 CONDUIT ITEM.

REVIEW OF DRAINAGE FACILITIES

PRIOR TO THE START OF WORK AND AGAIN BEFORE FINAL ACCEPTANCE, PERFORM AN INSPECTION WITH REPRESENTATIVES OF THE DEPARTMENT, CONTRACTOR AND

LOCALS OF ALL EXISTING DRAINAGE FACILITIES THAT ARE TO REMAIN IN SERVICE WHICH MAY BE AFFECTED BY THE WORK. THE CONDITION OF THE EXISTING CONDUITS AND THEIR APPURTENANCES IS DETERMINED FROM FIELD

OBSERVATIONS. RECORDS OF THE INSPECTION ARE MAINTAINED BY THE DEPARTMENT.

CONFIRM ALL EXISTING SEWERS INSPECTED INITIALLY BY THE ABOVE-MENTIONED PARTIES ARE MAINTAINED AND LEFT IN A CONDITION COMPARABLE TO THAT DETERMINED BY THE ORIGINAL INSPECTION. THE CONTRACTOR IS RESPONSIBLE TO CORRECT ANY CHANGE IN THE CONDITION RESULTING FROM THEIR OPERATIONS AS DIRECTED AND APPROVED BY THE ENGINEER.

PAYMENT FOR ALL OPERATIONS DESCRIBED ABOVE IS INCLUDED IN THE CONTRACT PRICE FOR THE PERTINENT 611 CONDUIT ITEMS.

EXISTING SUBSURFACE DRAINAGE

PROVIDE UNOBSTRUCTED OUTLETS FOR ALL EXISTING UNDERDRAINS OR AGGREGATE DRAINS ENCOUNTERED DURING CONSTRUCTION.

PROVIDE AN OUTLET PER STANDARD CONSTRUCTION DRAWING DM-1.1 FOR ALL UNDERDRAINS THAT OUTLET TO A SLOPE. UNDERDRAINS THAT CAN BE CONNECTED TO THE NEW OR EXISTING UNDERDRAINS AT THE END OF THE PROJECT LIMITS AS WELL AS ALL NECESSARY BENDS OR BRANCHES REQUIRED FOR CONNECTION ARE INCLUDED IN THE BASIS OF PAYMENT FOR UNCLASSIFIED PIPE UNDERDRAINS.

THE FOLLOWING ESTIMATED QUANTITIES HAVE BEEN INCLUDED IN THE GENERAL SUMMARY FOR THE WORK NOTED ABOVE:

611, 6” CONDUIT, TYPE F, 707.45 50 FT.

611, PRECAST REINFORCED CONCRETE OUTLET 1 EACH

605, 6” UNCLASSIFIED PIPE UNDERDRAINS WITH GEOTEXTILE FABRIC 						100 FT.

ITEM 611 – MANHOLE ADJUSTED TO GRADE, AS PER PLAN

ITEM 623 – MONUMENT ASSEMBLY ADJUSTED TO GRADE, AS PER PLAN

ITEM 638 – VALVE BOX ADJUSTED TO GRADE, AS PER PLAN

IN ADDITION TO THE REQUIREMENTS OF CMS 611.10.D FOR MANHOLES, 623.05 FOR MONUMENT ASSEMBLY, OR 638.18 FOR VALVE BOXES, THE CONTRACTOR WILL MAKE A CLEAN CIRCULAR CUT AROUND THE CASTING (48” DIAMETER FOR STORM AND SANITARY MANHOLE CASTINGS, 24”-28” FOR VALVE BOXES AND MONUMENT ASSEMBLIES, AND 2’ IN DIAMETER LARGER THAN THE CASTING DIAMETER FOR ANY CASTINGS THAT ARE LARGER THAN STANDARD MANHOLES SUCH AS TELECOMMUNICATION MANHOLE CASTINGS) AND REMOVE AND DISCARD THE EXISTING CASTING. INSTALL A NEW CASTING TO GRADE (ACCORDING TO TOLERANCES AS SHOWN ON STANDARD CONSTRUCTION DRAWING BP-3.1) AFTER THE PAVEMENT SURFACE COURSE HAS BEEN REPLACED. CMS 499 CLASS QCMS CONCRETE (DYE THE CONCRETE SUCH THAT ITS COLOR CLOSELY MATCHES THE COLOR OF THE SURROUNDING PAVEMENT) WILL BE USED FOR BACKFILLING THE FULL PAVEMENT SECTION AND THE JOINT BETWEEN THE ASPHALT AND CONCRETE WILL BE SEALED WITH CMS 702.01 PG BINDER. EPOXY COATED REBAR SHALL BE PLACED IN THE CONCRETE AT 6” MAXIMUM ON CENTER AND A MINIMUM OF 3.5” CLEARANCE FROM THE TOP, BOTTOM AND SIDES. THE CONCRETE WILL BE VIBRATED SUFFICIENTLY TO ELIMINATE AIR POCKETS UNDER THE FRAME. PAYMENT WILL INCLUDE REMOVAL OF THE EXISTING MATERIAL, INSTALLATION AND FURNISHING OF A NEW CASTING, AND ALL LABOR AND MATERIALS REQUIRED TO COMPLETE THIS ITEM OF WORK AS DESCRIBED.

ITEM 611 – CATCH BASIN, NO. 3, AS PER PLAN

THE CATCH BASIN AT STA. 767+94.47, 17’ LT IS MODIFIED FOR A 8” CURB HEIGHT.

EROSION CONTROL

SODDING

THE FOLLOWING QUANTITIES ARE PROVIDED TO PROMOTE GROWTH AND CARE OF PERMANENT SODDED AREAS:

659, SOIL ANALYSIS TEST                                 2 EACH

659, topsoil                                                   26 cu. yd.

659, COMMERCIAL FERTILIZER                          0.03 TON

659, LIME                                                        0.05 ACRES

659, WATER                                                     2 M. GAL.

660, SODDING STAKED                                    232 SQ. YD.
TRAFFIC CONTROL

STRUCTURE IDENTIFICATION SIGNS

STRUCTURE IDENTIFICATION SIGNS (I-H25b) WILL BE PLACED ON EACH APPROACH OFF THE RIGHT SHOULDER, FACING TRAFFIC, AND BEHIND THE GUARDRAIL IF APPLICABLE. A QUANTITY OF ONE SIGN PER APPROACH WILL BE INSTALLED.

THE SIGNS WILL HAVE A NON-REFLECTIVE WHITE SHEETING BACKGROUND. THE SIGNS WILL BE MOUNTED ON NEW NO. 2 POSTS AND WILL BE INSTALLED AS PER STANDARD CONSTRUCTION DRAWING TC-41.20, MOST CURRENT REVISION. EACH POST WILL BE 7.5’ IN LENGTH.

INSTALL SIGNS FOR THE FOLLOWING STRUCTURES: [SUM-91-14.54]

THE FOLLOWING QUANTITIES HAVE BEEN INCLUDED FOR EACH APPROACH:

ITEM 630 – SIGN, FLAT SHEET, 730.20, 2 SQ FT

ITEM 630 – GROUND MOUNTED SUPPORT, NO. 2 POST, 15 FT

ITEM 630 – REMOVAL OF GROUND MOUNTED SIGN AND DISPOSAL, 2 EACH

MISCELLANEOUS

CITY OF HUDSON GENERAL CONSTRUCTION NOTES

1. ALL EXISTING APPURTENANCES (UTILITY POLES, VALVES, HYDRANTS, MANHOLES, ETC.) ARE TO BE MAINTAINED BY THE CONTRACTOR UNLESS OTHERWISE SHOWN ON THE PLANS.

2. NOTIFY THE CITY OF HUDSON ENGINEERING DEPARTMENT A MINIMUM OF FORTY-EIGHT HOURS (2 WORKING DAYS) PRIOR TO THE START OF CONSTRUCTION.

3. ALL DISTURBED STORM SEWERS AND/OR APPURTENANCES, SIGNS, GUARD RAILING, MAIL AND/OR PAPER BOXES, DRIVE CULVERTS, FENCES, TREES, LANDSCAPING, OR OTHER ITEMS DISTURBED BY THE CONSTRUCTION SHALL BE RESTORED OR REPAIRED TO AT LEAST THE BEFORE-CONSTRUCTION CONDITION.

ITEM SPECIAL - AS-BUILT CONSTRUCTION RECORD DRAWINGS

PRIOR TO FINAL ACCEPTANCE OF THE WORK, THE CONTRACTOR SHALL FURNISH THE DEPARTMENT FORMAL AS-BUILT CONSTRUCTION RECORD-DRAWING PLANS. THE FORMAL AS-BUILT CONSTRUCTION RECORD-DRAWING SHALL INCLUDE ALL RED-LINED CHANGES. RED-LINE CHANGE SHALL BE DENOTED UTILIZING CLOUDING IN MICROSTATION (OR OTHER CAD SOFTWARE) OR CLOUDING IN PDF EDITING SOFTWARE. THE AS-BUILT CONSTRUCTION RECORD-DRAWING SHALL HAVE A SIGNED VERIFICATION ON THE TITLE SHEET FROM THE CONTRACTOR INDICATING THAT ALL RED-LINED AND FIELD CHANGES HAVE BEEN INCORPORATED INTO AS-BUILT CONSTRUCTION RECORD-DRAWINGS.

THE CONTRACTOR’S VERIFICATION STATEMENT INDICATES ALL KNOWN FIELD MODIFICATIONS MADE HAVE BEEN INCLUDED IN THE FORMAL RECORD-DRAWING. THE CONTRACTOR’S VERIFICATION STATEMENT SHALL BE SIGNED BY THE CONTRACTOR’S PROJECT MANAGER (OR ACCEPTABLE REPRESENTATIVE).

IN ADDITION TO THE INFORMATION SHOWN ON THE CONSTRUCTION PLANS, THE ASBUILT CONSTRUCTION RECORD-DRAWINGS SHALL SHOW THE FOLLOWING:

- ALL DEVIATIONS FROM THE ORIGINAL APPROVED CONSTRUCTION PLANS WHICH RESULT IN A CHANGE OF LOCATION, MATERIAL, TYPE OR SIZE OF WORK.

- ANY UTILITIES, PIPES, WELLHEADS, ABANDONED PAVEMENTS, FOUNDATIONS OR OTHER MAJOR OBSTRUCTIONS DISCOVERED AND REMAINING IN PLACE WHICH ARE NOT SHOWN, OR DO NOT CONFORM TO LOCATIONS OR DEPTHS SHOWN IN THE PLANS. UNDERGROUND FEATURES SHALL BE SHOWN AND LABELED ON THE RECORD-DRAWING PLAN IN TERMS OF STATION, OFFSET AND ELEVATION.

- THE FINAL OPTION AND SPECIFICATION NUMBER SELECTED FOR THOSE ITEMS WHICH ALLOW SEVERAL MATERIAL OPTIONS UNDER THE SPECIFICATION (E.G., CONDUIT).

- CHANGES TO THE PAY ITEMS AND FINAL QUANTITIES AS PAID SHALL BE SHOWN ON THE GENERAL SUMMARY AND SUBSUMMARIES.

- ADDITIONAL PLAN SHEETS MAY BE NEEDED IF NECESSARY TO SHOW WORK NOT INCLUDED IN THE CONSTRUCTION PLANS. IF ADDITIONAL PLAN SHEETS ARE NEEDED, THEY ARE REQUIRED TO BE PREPARED IN CONFORMANCE WITH THE LOCATION AND DESIGN MANUAL, VOLUME 3, SECTION 1200 - PLAN PREPARATION.

NOTATION SHALL ALSO BE MADE OF LOCATIONS AND THE EXTENT OF USE OF MATERIALS, OTHER THAN SOIL, FOR EMBANKMENT CONSTRUCTION (ROCK, BROKEN CONCRETE WITHOUT REINFORCING STEEL, ETC.).

THE PLAN INDEX SHALL SHOW THE PLAN SHEETS WHICH HAVE CHANGES APPEARING ON THEM.

TWO COPIES OF THE AS-BUILT CONSTRUCTION RECORD-DRAWINGS SHALL BE DELIVERED TO THE PROJECT ENGINEER FOR APPROVAL UPON COMPLETION OF THE PHYSICAL WORK BUT PRIOR TO THE REQUEST FOR FINAL PAYMENT. AFTER THE DEPARTMENT HAS APPROVED THE AS-BUILT CONSTRUCTION RECORD-DRAWINGS, THE ASSOCIATED ELECTRONIC FILES SHALL BE DELIVERED TO THE DISTRICT CAPITAL PROGRAMS ADMINISTRATOR. ACCEPTANCE OF THESE PLANS AND DELIVERY OF THE ASSOCIATED ELECTRONIC FILES IS REQUIRED PRIOR TO THE WORK BEING ACCEPTED AND THE FINAL ESTIMATE APPROVED.

PAYMENT FOR ALL THE ABOVE SHALL BE LUMP SUM UPON PROPER EXECUTION OF ALL WORK OF THIS ITEM AS DETERMINED BY THE PROJECT ENGINEER.

RAILROAD

ITEM 900 – FLAGGING SERVICES

FLAGGING FOR WORK ON RAILROAD RIGHT OF WAY SHALL BE COORDINATED, OBTAINED AND PAID FOR BY THE CONTRACTOR. FLAGGING SHALL BE PROVIDED BY THE CONTRACTOR WHENEVER REQUIRED BY THE NORFOLK SOUTHERN SPECIAL PROVISIONS FOR THE PROTECTION OF RAILWAY INTEREST. NORFOLK SOUTHERN SHALL APPROVE THE FLAGGING SERVICE PROVIDER AND THEIR STAFF.

NORFOLK SOUTHERN HAS THE SOLE AUTHORITY TO DETERMINE THE NEED FOR PROTECTION SERVICES TO PROTECT ITS OPERATIONS IN GENERAL. THE REQUIREMENTS OF SUCH SERVICES WILL BE WHENEVER THE CONTRACTOR’S PERSONNEL OR EQUIPMENT ARE OR ARE LIKELY TO BE, WORKING ON THE RAILROAD’S RIGHT OF WAY, OR ACROSS, OVER, ADJACENT TO, OR UNDER A TRACK, OR WHEN SUCH WORK HAS DISTURBED OR IS LIKELY TO DISTURB A RAILROAD STRUCTURE OR THE RAILROAD ROADBED OR SURFACE AND ALIGNMENT OF ANY TRACK TO SUCH EXTENT THAT THE MOVEMENT OF TRAINS MUST BE CONTROLLED BY FLAGGING.

THE TOTAL DOLLARS IN THE ESTIMATED QUANTITIES IS BASED UPON AN ESTIMATE OF TOTAL FLAGGING DOLLARS NEEDED TO COMPLETE THE PLANNED WORK.

ONLY THE FOLLOWING CERTIFIED FLAGGING PROVIDERS ARE ACCEPTABLE BY NORFOLK SOUTHERN:

RAILROAD CONSULTANTS

STEVE LLOYD (VP BUSINESS DEVELOPMENT)

(615) 542-8901

RAILPROS

1320 GREENWAY DR., SUITE 490

IRVING, TX  75038

(877) 315-0513

http://www.railpros.com/services-category/field-services/

PAYMENT FOR CERTIFIED FLAGGING PROVIDERS WILL BE MADE PER ITEM 900E001000, EACH, RAILROAD FLAGGING SERVICES BASED UPON THE INVOICES RECEIVED FROM THE FLAGGING SERVICE FOR THE DOLLARS USED, INCLUDING A FIVE PERCENT MARKUP FOR CONTRACTOR OVERHEAD FOR ADMINISTERING THE CONTRACT WITH THE FLAGGING SERVICE.

IN THE EVENT THE PROJECT IS DELAYED DUE TO RAILROAD FLAGGER AVAILABILITY, THE CONTRACTOR WILL PROVIDE DOCUMENTATION SUPPORTING THEIR EFFORTS TO SCHEDULE A FLAGGER FROM THE FLAGGING SERVICE.