STANDARD DRAWINGS AND SUPPLEMENTAL

SPECIFICATIONS:

REFER TO THE FOLLOWING STANDARD BRIDGE

DRAWING(S):

NA

AND TO THE FOLLOWING SUPPLEMENTAL

SPECIFICATION(S):

940	DATED	17-APR-2015

DESIGN SPECIFICATIONS:

THIS STRUCTURE CONFORMS TO THE 8TH EDITION OF THE “LRFD BRIDGE DESIGN SPECIFICATIONS” ADOPTED BY THE AMERICAN ASSOCIATION OF STATE HIGHWAY AND TRANSPORTATION OFFICIALS, 2017, AND THE ODOT BRIDGE DESIGN MANUAL, 2019.

DESIGN LOADING:

CONDUIT: AS PER CMS 706.05 - PRECAST REINFORCED CONCRETE BOX

ALL REMAINING COMPONENTS: HL-93 WITH 60 PSF FUTURE WEARING SURFACE

DESIGN DATA:

PRECAST BOX:

DESIGN FILL: 0’ < 2’

AS PER CMS 706.05 - PRECAST REINFORCED CONCRETE BOX

REINFORCED CONCRETE ELEMENTS:

CONCRETE CLASS QC2 – COMPRESSIVE STRENGTH 4.5 KSI (SUPERSTRUCTURE)

CONCRETE CLASS QC1 – COMPRESSIVE STRENGTH 4.0 KSI (SUBSTRUCTURE)

REINFORCING STEEL - MINIMUM YIELD STRENGTH 60 KSI

DECK PROTECTION METHOD:

WATERPROOFING AND ASPHALT CONCRETE OVERLAY.

EXISTING STRUCTURE VERIFICATION:

DETAILS AND DIMENSIONS SHOWN ON THESE PLANS PERTAINING TO THE EXISTING STRUCTURE HAVE BEEN OBTAINED FROM PLANS OF THE EXISTING STRUCTURE AND FROM FIELD OBSERVATIONS AND MEASUREMENTS. CONSEQUENTLY, THEY ARE INDICATIVE OF THE EXISTING STRUCTURE AND THE PROPOSED WORK BUT THEY SHALL BE CONSIDERED TENTATIVE AND APPROXIMATE. THE CONTRACTOR IS REFERRED TO CMS SECTIONS 102.05 AND 105.02.

BASE CONTRACT BID PRICES UPON A RECOGNITION OF THE UNCERTAINTIES DESCRIBED ABOVE AND UPON A PREBID EXAMINATION OF THE EXISTING STRUCTURE. HOWEVER, THE DEPARTMENT WILL PAY FOR ALL PROJECT WORK BASED UPON ACTUAL DETAILS AND DIMENSIONS THAT HAVE BEEN VERIFIED IN THE FIELD.

PROPOSED WORK:

- DETOUR TRAFFIC FROM SR-91 (SOUTH MAIN STREET).

- REMOVE AND STORE BRIDGE RAIL FROM PARAPET.

- REMOVE ROADWAY PAVEMENT.

- INSTALL TEMPORARY SHORING AS SHOWN.

- REMOVE PARAPET, SIDEWALK, AND BRIDGE SLAB.

- EXCAVATE AS REQUIRED.

- REMOVE AND SALVAGE STONE ABUTMENT UNITS.

- PREPARE BED FOR BOX CULVERT UNITS.

- PREPARE ADJOINING CONCRETE ABUTMENTS AND WINGWALLS FOR CLOSURE POURS.

- INSTALL BOX CULVERT UNITS.

- INSTALL CLOSURE POURS, SIDEWALK, AND PARAPET.

- INSTALL WATERPROOFING MEMBRANES.

- BACKFILL AND INSTALL PAVEMENT.

- RE-OPEN TO TRAFFIC.

- RE-INSTALL BRIDGE RAIL ON PARAPET.

OTHER ITEMS OF WORK NOT LISTED MAY BE PERFORMED AT THE CONTRACTOR’S CONVENIENCE.

MAINTENANCE OF TRAFFIC:

TRAFFIC ON SR-91 (SOUTH MAIN STREET) IS TO BE DETOURED DURING CONSTRUCTION.

REFER TO THE MAINTENANCE OF TRAFFIC PLANS FOR DETAILS, OTHER REQUIREMENTS AND PAYMENT PROVISIONS.

UTILITY LINES:

THE UTILITY(IES) SHALL BEAR ALL EXPENSE INVOLVED IN RELOCATING (INSTALLING) THE AFFECTED UTILITY LINES. THE CONTRACTOR AND UTILITY(IES) ARE TO COOPERATE BY ARRANGING THEIR WORK IN SUCH A MANNER THAT INCONVENIENCE TO EITHER WILL BE HELD TO A MINIMUM.

THE UNDERGROUND GAS MAIN TO THE WEST OF THE WEST WINGWALLS WILL NOT BE AFFECTED BY THE BRIDGE CONSTRUCTION; BUT WILL HAVE TO BE PROTECTED FROM HARM BY HEAVY EQUIPMENT.

UNSUITABLE SOILS:

THE FOLLOWING ITEMS AND QUANTITIES ARE TO BE USED AS DIRECTED BY THE ENGINEER TO ADDRESS UNSUITABLE SOILS ENCOUNTERED IN THE AREA UNDER THE PROPOSED BOX CULVERT.

ITEM 203 – EXCAVATION	32 CU YD

ITEM 203 - GRANULAR MATERIAL, TYPE C (703.16)	32 CU YD

ITEM 204 - GEOTEXTILE FABRIC, TYPE D	64 SQ YD

CHAMFER ON CONCRETE STRUCTURES:

PROVIDE A ¾” x ¾” CHAMFER ON ALL EXPOSED EDGES OF CONCRETE STRUCTURE UNLESS OTHERWISE SHOWN ON THE PLANS. THIS WORK SHALL BE INCLUDED WITH THE APPROPRIATE ITEM 511.

REINFORCEMENT COVER AND SPACING:

MINIMUM CLEAR COVER TO REINFORCING BARS SHALL BE PER THE FOLLOWING TABLE, UNLESS NOTED:

- CONCRETE CAST AGAINST AND PERMANENTLY EXPOSED TO GROUND: 3"

- ALL OTHER CASES: 2"

CLEAR HORIZONTAL DISTANCE BETWEEN BARS IN A LAYER, INCLUDING THE PRESENCE OF LAP-SPLICES, SHALL NOT BE LESS THAN THE LARGER OF: 1.5 BAR DIAMETERS OR 1½", EXCEPT FOR THE BARS IN A CONTACT LAP SPLICE.

CLEAR VERTICAL DISTANCE BETWEEN LAYERS, INCLUDING THE PRESENCE OF LAP-SPLICES, SHALL NOT BE LESS THAN 1".

CONSTRUCTION JOINTS IN CONCRETE:

UNLESS NOTED OTHERWISE IN THE PLANS, ALL CONSTRUCTION JOINTS IN CONCRETE ARE AT THE CONTRACTOR’S OPTION – PROVIDED THAT ALL THE REQUIREMENTS IN THE CONTRACT DOCUMENTS ARE MET FOR REINFORCEMENT PLACEMENT AND SUPPORT, AS WELL AS CONCRETE PLACEMENT, CONSOLIDATION, AND FINISHING.

FINISH COLORS:

FINISH COLOR FOR SEALED CONCRETE SURFACES SHALL BE “WHITE”, NUMBER 17925 PER FEDERAL COLOR STANDARD 595B.

ASBESTOS NOTIFICATION:

A CERTIFIED ASBESTOS HAZARD EVALUATION SPECIALIST INSPECTED THE BRIDGE STRUCTURE SCHEDULED FOR DEMOLITION AND/OR REHABILITATION.

THE SURVEY DETERMINED THAT NO ASBESTOS IS PRESENT ON THE STRUCTURE.

THE DEPARTMENT HAS PROVIDED A COPY OF THE OHIO ENVIRONMENTAL PROTECTION AGENCY (OEPA) NOTIFICATION OF DEMOLITION AND RENOVATION FORM (PARTIALLY COMPLETED) AND THE ASBESTOS INSPECTION REPORT IN THE REFERENCE FILES FOR THIS PROJECT.

THE CONTRACTOR SHALL COMPLETE THE FORM AND SUBMIT IT TO THE OEPA AT LEAST TEN (10) WORKING DAYS PRIOR TO THE START OF ANY DEMOLITION AND/OR RENOVATION. ONLINE SUBMISSION IS AVAILABLE AT http://www.epa.ohio.gov/asbestos AND IS ENCOURAGED; OR THE CONTRACTOR SHALL SUBMIT IT TO ONE OF THE ADDRESSES BELOW:

ASBESTOS PROGRAM

OHIO EPA, DAPC

P.O. BOX 1049

COLUMBUS, OH 43216-1049

OR

ASBESTOS PROGRAM

OHIO EPA, DAPC

50 W. TOWN ST., SUITE 700

COLUMBUS, OH 43215

THE FORM SHALL INCLUDE:

1.	THE CONTRACTORS NAME AND ADDRESS,

2.	THE SCHEDULED DATES FOR THE START AND COMPLETION OF THE STRUCTURE DEMOLITION AND/OR RENOVATION,

3.	DESCRIPTION OF THE PLANNED DEMOLITION WORK AND THE METHODS BE USED, AND

4.	ALL NECESSARY FEES.

THE CONTRACTOR SHALL PROVIDE A COPY OF THE COMPLETED NOTIFICATION OF DEMOLITION AND RENOVATION FORM TO THE PROJECT ENGINEER AT LEAST TEN (10) WORKING DAYS PRIOR TO THE START OF ANY DEMOLITION AND/OR RENOVATION.

THE CONTRACTOR SHALL FURNISH ALL FEES, LABOR, AND MATERIALS NECESSARY TO COMPLETE AND SUBMIT THE OEPA NOTIFICATION FORM. PAYMENT FOR THIS WORK SHALL BE INCLUDED IN ITEM 202 – PORTIONS OF STRUCTURE REMOVED, AS PER PLAN.

ITEM 202 – PORTIONS OF STRUCTURE REMOVED, AS PER PLAN:

DESCRIPTION:

IN ADDITION TO REMOVING THE EXISTING STRUCTURE, THIS WORK SHALL CONSIST OF THE REMOVAL OF THE EXISTING BARN STONES COMPRISING THE ABUTMENTS OF THE EXISTING STRUCTURE. THE BARN STONES ARE APPROXIMATELY 30 INCHES BY 30 INCHES AND VARY IN LENGTH FROM LESS THAN 3 FEET TO MORE THAN 8 FEET LONG.

THE FOLLOWING QUANTITIES HAVE BEEN CARRIED TO THE GENERAL SUMMARY:

ITEM 202 – PORTIONS OF STRUCTURE REMOVED, AS PER PLAN:	LUMP

ITEM 202 - BRIDGE RAILING REMOVED FOR REUSE, AS PER PLAN:

DESCRIPTION:

THIS WORK SHALL CONSIST OF REMOVING AND STORING THE EXISTING ALUMINUM BRIDGE RAIL FOR REUSE IN THE PROPOSED PARAPET.

RAILING DAMAGED DURING REMOVAL AND/OR WHILE IN THE CONTRACTOR’S CUSTODY SHALL BE REPLACED IN-KIND BY THE CONTRACTOR AT NO COST TO THE DEPARTMENT.

MEASUREMENT AND PAYMENT:

THE DEPARTMENT WILL MEASURE THE LINEAR FOOT QUANTITY OF BRIDGE RAIL REMOVED AND STORED. THIS SHALL INCLUDE ALL TOOLS, LABOR, MATERIALS, AND EQUIPMENT NECESSARY TO COMPLETE THIS ITEM OF WORK AS DESCRIBED, AND TO THE SATISFACTION OF THE ENGINEER.

ITEM 503 - COFFERDAMS AND EXCAVATION BRACING, AS PER PLAN:

FURNISH STEEL SHEET PILING PER AASHTO M 202 (ASTM A328) (MINIMUM YIELD POINT 39 KSI) WITH MINIMUM SECTION MODULUS OF 10 CUBIC INCHES PER FOOT.

THE DESIGN SHOWN ON THE PLANS FOR TEMPORARY SUPPORT OF EXCAVATION IS ONE REPRESENTATIVE DESIGN THAT MAY BE USED TO CONSTRUCT THE PROJECT. THE CONTRACTOR MAY CONSTRUCT THE DESIGN SHOWN ON THE PLANS OR PREPARE AN ALTERNATE DESIGN TO SUPPORT THE SIDES OF EXCAVATIONS. IF CONSTRUCTING AN ALTERNATE DESIGN FOR TEMPORARY SUPPORT OF EXCAVATION, PREPARE AND PROVIDE PLANS IN ACCORDANCE WITH C&MS 501.05. THE DEPARTMENT WILL PAY FOR THE TEMPORARY SUPPORT OF EXCAVATION AT THE CONTRACT LUMP SUM PRICE FOR COFFERDAMS AND EXCAVATION BRACING, AS PER PLAN. NO ADDITIONAL PAYMENT WILL BE MADE FOR PROVIDING AN ALTERNATE DESIGN.

ITEM 509 - EPOXY COATED REINFORCING STEEL, AS PER PLAN:

DESCRIPTION:

IN ADDITION TO THE REQUIREMENTS OF C&MS 509, THIS SHALL INCLUDE PROVIDING HEADED DOWELS ON THOSE REINFORCING BARS SO DESIGNATED IN THE REINFORCING STEEL SCHEDULE.

HEADED DOWELS SHALL DEVELOP THE MINIMUM SPECIFIED TENSILE STRENGTH OF THE REINFORCING BAR PER ASTM A970, “STANDARD SPECIFICATION FOR HEADED STEEL BARS FOR CONCRETE REINFORCEMENT”, CLASS A OR HA. PRIOR TO ORDERING, SUBMIT FOR APPROVAL INDEPENDENT LABORATORY TESTING CERTIFICATION CONFORMING TO THE INTERNATIONAL CODE COUNCIL (ICC) THAT THE PRODUCT(S) MEET THIS REQUIREMENT.

HEADED DOWELS MAY BE COLD-SWAGED, FORGED, OR WELDED.

ACCEPTABLE PRODUCTS INCLUDE:

BUTTONHEAD BNH, 5AB

BARSPLICE PRODUCTS, INC.

DAYTON, OH

937-275-8700

WWW.BARSPLICE.COM

D158F PLAIN END ANCHOR (5AB)

DAYTON SUPERIOR

MIAMISBURG, OH

888-977-9600

WWW.DAYTONSUPERIOR.COM

HRC 555 SERIES T-HEAD

HEADED REINFORCEMENT CORP.

FOUNTAIN VALLEY, CA

714-557-1455

WWW.HRC-USA.COM

EPOXY COAT ALL HEADED DOWELS AFTER HEAD INSTALLATION/FABRICATION.

MEASUREMENT AND PAYMENT:

THE DEPARTMENT WILL MEASURE THE POUNDS OF EPOXY COATED REINFORCING STEEL INCORPORATED INTO THE WORK. THIS SHALL INCLUDE ALL TOOLS, LABOR, MATERIALS, AND EQUIPMENT NECESSARY TO COMPLETE THIS ITEM OF WORK AS DESCRIBED, AND TO THE SATISFACTION OF THE ENGINEER.

ITEM 517 – RAILING, MISC.: REINSTALL BRIDGE RAILING:

DESCRIPTION:

THIS WORK SHALL CONSIST OF REINSTALLING THE BRIDGE RAILING REMOVED AND STORED FOR RE-USE.

CORE OR FORM HOLES IN THE TOP OF THE PARAPET AS NECESSARY TO RECEIVE THE RAILING. HOLES TO BE ¼” LARGER THAN THE POST DIAGONAL DIMENSION.

INSTALL THE RAILING PLUMB AND CENTERED ON THE CONCRETE PARAPET; AS WELL AS TO THE SAME ELEVATION, AND IN LINE WITH, THE ADJACENT RAILING.

GROUT IN PLACE WITH NON-SHRINK NON-METALLIC GROUT PER C&MS 510.

WITH THE APPROVAL OF THE ENGINEER, THE CONTRACTOR MAY REPLACE THE ALUMINUM RAIL WITH ANOTHER THAT MATCHES THE EXISTING RAIL’S CONFIGURATION, DETAILS, AND FINISH; AND IS PROVIDED BY THE SAME SUPPLIER AS THE ORIGINAL.

MEASUREMENT AND PAYMENT:

THE DEPARTMENT WILL MEASURE THE LINEAR FOOT QUANTITY OF REINSTALLED BRIDGE RAILING INCORPORATED INTO THE WORK. THIS SHALL INCLUDE ALL TOOLS, LABOR, MATERIALS, AND EQUIPMENT NECESSARY TO COMPLETE THIS ITEM OF WORK AS DESCRIBED, AND TO THE SATISFACTION OF THE ENGINEER.

ITEM 601 - ROCK CHANNEL PROTECTION, TYPE B WITH AGGREGATE FILTER:

DESCRIPTION:

REPLACE ALL CHANNEL EROSION PROTECTION DISTURBED BY THE WORK WITH TYPE B ROCK CHANNEL PROTECTION (RCP) 48” DEEP WITH AN AGGREGATE FILTER INCLUDING ANY SUBGRADE PREPARATION REQUIRED TO PROVIDE SOLID BEARING OF THE RCP AND FILTER PER C&MS 601.

AN ADDITIONAL 25% CONTINGENCY IS INCLUDED IN THE QUANTITIES ESTIMATED.

MEASUREMENT AND PAYMENT:

THE DEPARTMENT WILL MEASURE THE CUBIC YARDS OF TYPE A ROCK CHANNEL PROTECTION WITH AGGREGATE FILTER INCORPORATED INTO THE WORK. THIS SHALL INCLUDE ALL TOOLS, LABOR, MATERIALS, AND EQUIPMENT NECESSARY TO COMPLETE THIS ITEM OF WORK AS DESCRIBED, AND TO THE SATISFACTION OF THE ENGINEER.

ITEM 611 - 14' x 8' CONDUIT, TYPE A, 706.05, AS PER PLAN, DESIGN COVER 0-2 FT:

DESCRIPTION:

IN ADDITION TO THE REQUIREMENTS OF C&MS 611 AND SUPPLEMENTAL SPECIFICATION 940, THIS SHALL INCLUDE PROVIDING SKEWED ENDS TO THE FIRST AND LAST UNIT AS SHOWN IN THE PLANS TO PROVIDE A UNIFORM WIDTH TO THE CAST-IN-PLACE CONCRETE CLOSURES AT EITHER END OF THE BOX CULVERT. BEVELED FACES OF THESE UNITS MAY BE LEFT SMOOTH.

THIS SHALL ALSO INCLUDE PROVIDING COMPACTED FILL IN ANY VOIDS LEFT FROM REMOVALS BELOW AND TO THE SUBGRADE, BEDDING, AND BACKFILL PER 611.06.

WATERPROOFING:

TYPE 2 WATERPROOFING, PER CMS 512.09 AND 711.25, SHALL EXTEND VERTICALLY DOWN THE ENTIRE SIDES OF THE PRECAST CULVERT SECTIONS FOR ALL PORTIONS OF THE CULVERT WHICH SHALL BE IN CONTACT WITH THE BACKFILL. PAYMENT FOR THE MEMBRANE WATERPROOFING SHALL BE AT THE CONTRACT PRICE BID PER SQUARE YARD FOR ITEM 512 - TYPE 2 WATERPROOFING.

TYPE 3 WATERPROOFING, PER CMS 512.10 AND 711.29 SHALL BE APPLIED TO THE ENTIRE TOP SURFACE OF THE PRECAST CULVERT SECTIONS AND SHALL EXTEND ONE FOOT VERTICALLY DOWN THE SIDES FOR ALL PORTIONS OF THE CULVERT WHICH SHALL BE IN CONTACT WITH THE BACKFILL. PAYMENT FOR THE MEMBRANE WATERPROOFING SHALL BE AT THE CONTRACT PRICE BID PER SQUARE YARD FOR ITEM 512 - TYPE 3 WATERPROOFING.