SURVEYING PARAMETERS for Union-33-17.95 PID#115685

PRIMARY PROJECT CONTROL MONUMENTS GOVERN ALL POSITION-

ING ON ODOT PROJECTS.  SEE SHEET ___ OF THE PLANS FOR

A TABLE CONTAINING PROJECT CONTROL INFORMATION.

USE THE FOLLOWING PROJECT CONTROL, VERTICAL POSITIONING,

AND HORIZONTAL POSITIONING PARAMETERS FOR ALL SURVEYING:

PROJECT CONTROL

POSITIONING METHOD:  ODOT VRS

MONUMENT TYPE: TYPE B

VERTICAL POSITIONING

ORTHOMETRIC HEIGHT DATUM:  NAVD 88

GEOID:  18

HORIZONTAL POSITIONING

REFERENCE FRAME:  NAD83 (2011)

ELLIPSOID:  GRS-80

MAP PROJECTION:  LAMBERT CONFORMAL CONIC

COORDINATE SYSTEM:  OHIO STATE PLANE NORTH ZONE

COMBINED SCALE FACTOR:  0.9999857102

ORIGIN OF COORDINATE

SYSTEM:  0,0,0

To go from Grid to Ground divide by 0.9999857102

TO GO FROM GROUND TO GRID DIVIDE BY 1.00001429

USE THE POSITIONING METHODS AND MONUMENT TYPE USED IN

THE ORIGINAL SURVEY TO RESTORE ALL MONUMENTS RELATED

TO PRIMARY PROJECT CONTROL THAT ARE DAMAGED OR

DESTROYED BY CONSTRUCTION ACTIVITIES.  RESTORE THE

DAMAGED OR DESTROYED MONUMENTS IN ACCORDANCE WITH

SUPPLEMENTAL SPECIFICATION 623.

UNITS ARE IN U.S. SURVEY FEET.  USE THE FOLLOWING

CONVERSION FACTOR:  1 METER = 3.280833333 U.S. SURVEY

FEET.