ASD-060-14.97

PID  115029

For this project I used the Ashland S.H. (I.C.H.) 143 SEC. D (1931) R/W Plans.  We found two monuments at the P.T. station 108+35.51. the concrete monument on the west side was in good condition so I held this monument (MN 01). We also found a good concrete monument a station P.I. 117+84.30. So I rotated the Right-of-Way plans to this good monument (MN 05). All of the other concrete monuments were disturbed, but hit within reason of the recorded Right-of-Way Plans. I also had to determine the south section line for my lead in for the new R/W takes. We found and used a pin (MN 18) located at the Southeast Corner of Section 28, and a railroad spike found (MN 19) at the Southwest Corner of the Southeast ¼, of Section 28. For the boundary lines I held the found chiseled X (MN 20) on the headwall on the East side of State Route 060, and the bent pipe (MN 16). On the west side of State Route 060 I used the bent pin (MN 14), and rotated the survey to the found pipe (MN 21). This rotation hit the bent pin (MN 11) really well.

For stationing I used DESTAPE at the intersection of State Route 060 and County Road 1600.

Brett R. Tieben, P.S.

03/23/2023