BEL-7-22.16 Structure Type Study Pedestrian Overpass over State Route 7

October 12, 2022

Prepared for:

Ohio Department of Transportation - District 11

Prepared by:

Stantec Consulting Services, Inc.

| Revision | Description | Author | Date | Quality Check | Date | Independent Review | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |

The conclusions in the Report titled BEL-7-22.16 are Stantec’s professional opinion, as of the time of the Report, and concerning the scope described in the Report. The opinions in the document are based on conditions and information existing at the time the scope of work was conducted and do not take into account any subsequent changes. The Report relates solely to the specific project for which Stantec was retained and the stated purpose for which the Report was prepared. The Report is not to be used or relied on for any variation or extension of the project, or for any other project or purpose, and any unauthorized use or reliance is at the recipient’s own risk.

Stantec has assumed all information received from Ohio Department of Transportation - District 11 (the “Client”) and third parties in the preparation of the Report to be correct. While Stantec has exercised a customary level of judgment or due diligence in the use of such information, Stantec assumes no responsibility for the consequences of any error or omission contained therein.

This Report is intended solely for use by the Client in accordance with Stantec’s contract with the Client. While the Report may be provided to applicable authorities having jurisdiction and others for whom the Client is responsible, Stantec does not warrant the services to any third party. The report may not be relied upon by any other party without the express written consent of Stantec, which may be withheld at Stantec’s discretion.

| Prepared by: |  |
| --- | --- |
|  | Signature |
|  | Brandon S. McNeal, P.E. |
|  | Printed Name |
| Reviewed by: |  |
|  | Signature |
|  | Michael R. Sturdevant, P.E. |
|  | Printed Name |

Table of Contents

Executive Summary	ii

1	Introduction	1

1.1	Purpose	1

1.2	Existing Structure	1

1.3	Proposed Structure	1

1.3.1	Abutment stairs	1

1.3.2	Pedestrian Bridge Superstructure	2

2	Design Considerations	2

2.1	Design Specification	2

2.2	Design Criteria	2

2.3	Alignment	2

2.4	Maintenance of Traffic	2

2.5	Railroad protection	3

2.6	Geotechnical	3

3	Alternatives	3

3.1	Abutment Stairs	3

3.1.1	Alternative 1	3

3.1.2	Alternative 2	4

3.1.3	Alternative 3	4

3.2	Superstructure	5

3.2.1	Rehabilitation Alternative	5

3.2.2	Replacement Alternative	6

4	Alternative Comparison	7

4.1	Abutment Stairs	7

4.1.1	Economic Comparison	7

4.1.2	Discussion of Alternatives	8

4.2	Superstructure	9

4.2.1	Economic Comparison	9

4.2.2	Discussion of Alternative	9

5	Recommendations	10

List of Appendices

Appendix A drawings	1

A.1	Site Plan and Profile	2

A.2	Stair Replacement Alternatives	2

Appendix B	Cost Breakdowns	3

B.1	Stair Replacement Alternatives Cost Breakdown	4

B.2	Superstructure Rehabilitation and Replacement Cost Breakdown	5

Appendix C	Geotechnical Report	6

Executive Summary

The purpose of this study is to evaluate the differences between and economy of alternative structures to replace or rehabilitate the existing BEL-7-22.16 superstructure and the stairs on the east end used to access the pedestrian bridge crossing State Route 7 (S.R. 7) in Martins Ferry, Ohio. The existing bridge is a 3-span continuous steel beam bridge with spans of 62’, 78’ and 81.5’ measured from center to center of bearings. The west abutment is located adjacent to the parking lot for East Ohio Regional Hospital and runs east across S.R. 7 with piers located before the south bound lanes of S.R. 7 and in the median of S.R. 7 with the east abutment being located adjacent to Crummitt and Son Vault Corporation, Matheson Valley Gas and Purple Rider Stadium. The eastern abutment is approximately 33.5’ tall with 5 flights of stairs down to ground level. The existing stairs are in poor condition and will be replaced. Along with the replacement of the stairs the existing superstructure is being considered for rehabilitation or replacement.  and the superstructure will be raised as needed to achieve the required 16.5 feet of clearance over the S.R. 7 northbound lanes.

Three alternatives have been identified for evaluation for the replacement of the existing stairs.

- Alternative 1 is a cast-in-place concrete stair replacement. The configuration will be very similar to the existing stairs.

- Alternative 2 is a steel framed option with serrated steel grating for the landings and stairs.

- Alternative 3 is a steel framed option with precast concrete landings and stairs.

Two alternatives have been identified for the superstructure of the pedestrian bridge.

- Superstructure Alternative 1 is a superstructure rehabilitation option.

- Superstructure Alternative 2 is a replacement of the superstructure with composite precast prestressed concrete I-Beams.

# Introduction

## Purpose

The purpose of this study is to evaluate alternative structures to replace or rehabilitate the existing superstructure and to evaluate alternative structures to replace the stairs at the east end of the bridge to access the pedestrian bridge crossing State Route 7 (S.R. 7) in Martins Ferry, Ohio.

## Existing Structure

The existing structure is a 3-span structure with spans of 62’, 78’ and 81’-6” measured center to center of bearings. The bridge was constructed in 1968 and carries pedestrian traffic over S.R. 7 providing access to Crummitt and Sons Vault company, Matheson Valley Gas and the Purple Rider Football Stadium. The superstructure consists of (2)-30WF99 beams supporting a 6” reinforced concrete deck. The total width of the superstructure is 9’-6” outside of deck to outside of deck. The pedestrian path width is 8’ measured from toe to toe of parapet. The piers are T-type with expansion abutments on either end. In 1998 plans were developed to top modify the superstructure by applying a ¼-inch epoxy topping to the deck, removing the 6” curb and 4’-6” tall chain link fence and replacing them with a 1’-6” tall concrete parapet with 10’-0” curved vandal protection fencing.

The east bridge abutment is a reinforced concrete abutment approximately 33’-6” tall with access to the pedestrian bridge provided by reinforced concrete stairs. Currently the stairs are in poor condition and have been repaired and in some locations have been reinforced with steel beams.

## Proposed Structure

For the purposes of this study there will be two independent evaluations completed. The first evaluation discussed will be the replacement of the cast-in-place concrete stairs, the second will be the rehabilitation versus replacement of the superstructure of the pedestrian bridge.

### Abutment stairs

The proposed abutment stairs will provide landings and stairs with a configuration similar to what is present currently. Variability in the width of the stairs themselves between the alternatives will be due to differing material types. In no case will the width of the provided stairways and landings be smaller than what is currently at the site. In general, the landings will be 5’-0” wide against the abutment and 5’-6” wide for the landings further from the abutment. Stair widths will remain relatively the same. For the levels where two separate flights of steps are provided, each staircase will have a width of 4’-6” clear between railings. For the interior single flight of stairs a width of 8’-6” between railings has been provided.

### Pedestrian Bridge Superstructure

The proposed superstructure will provide a bridge deck width of 10’-4” out-to-out with a walking path width of 8’ toe-to-toe of parapet and 1’ wide BR-2-15 railings. This study will evaluate rehabilitating the existing superstructure on the 30WF99 beams versus replacing the entirety of the superstructure with prestressed concrete I-Beams. In both cases the superstructure will be raised in order to provide a minimum of 16’-6” of clearance over the S.R. 7 northbound lanes. Raising the profile will require extending the east abutment seat vertically to accommodate the increased clearance. The railing style selected will be per standard bridge drawing BR-2-15 which is a 42” tall combination pedestrian railing.

# Design Considerations

## Design Specification

The proposed structures will be designed in accordance with the ODOT 2020 Bridge Design Manual, the AASHTO LRFD Bridge Design Specifications (9th edition) and the LRFD Guide Specifications for the Design of Pedestrian Bridges with 2015 Interim Revisions.

## Design Criteria

- Proposed pedestrian path width: 8’-0” toe-to-toe of railing

- Proposed structure width: 10’-4” out-to-out of deck

- Clearance over S.R. 7 a minimum of 16’-6”.

- Live load on proposed structures: 90 psf

## Alignment

The existing horizontal alignment will be maintained, the existing vertical alignment will be modified to raise the east end of the bridge 1.6’ in order to achieve the required 16’-6” clearance over S.R. 7 northbound lanes below. This will reduce the profile grade of the bridge from 6.0% to 5.3%.

## Maintenance of Traffic

The project will be phased so that construction of the stairs and superstructure rehabilitation/replacement will not take place during football season, minimizing interruptions to pedestrians traveling to and from the football stadium during game nights.

Traffic on S.R. 7 will need to be maintained during construction of the rehabilitation or replacement of the superstructure. Potential means of maintaining traffic are through lane closures and contraflow of traffic or detouring of traffic onto local streets. Contraflow of traffic would require the removal of the concrete median north of the bridge to allow traffic to cross over. South of the bridge it is possible that the cross over could occur prior to the beginning of the concrete median but it would need to be evaluated. The local road detour option can be achieved by detouring traffic onto Hanover Street and Zane Highway, a detour of approximately 1 mile.

## Railroad protection

Temporary shoring and support of the existing superstructure will be provided in order to protect the existing railroad immediately adjacent to the east bridge abutment. The demolition and construction of the new stairs will need to exercise caution to avoid interfering with railroad operations.

## Geotechnical

A geotechnical study has been performed based on soil borings advanced at the site. The Study of Foundation Exploration has been included as Appendix C.

# Alternatives

The alternatives section will be broken into two sections. The first section will be for the replacement of the existing cast-in-place concrete steps and the second section will be for the rehabilitation or replacement of the BEL-7-2216 Bridge superstructure.

## Abutment Stairs

Three alternative materials were evaluated for the replacement of the existing stairs, cast-in-place concrete, steel beams with serrated grating, and steel beams with precast concrete landings and stairs. Each alternative will require the addition of cast-in-place concrete columns and beams to the east face of the east abutment that will be doweled into the existing abutment to provide support for the proposed stairs and landings. The proposed stairs will need to provide lateral support for the abutment of the pedestrian bridge in the longitudinal direction of the bridge. This is due to the pile layout supporting the existing abutment foundation having very little lateral resistance capacity as the piles are spaced at only 1’-3” in the longitudinal direction of the bridge. All proposed loads will be supported by piles supporting the east abutment and the existing piles supporting the stair columns. Concrete caps will be added to the top of the existing footings to transmit the loads from the columns into the footings. The caps will be doweled into the existing footing.

### Alternative 1

Alternative 1 replaces the existing stairs with all new cast-in-place concrete stairs and landings. The columns, beams, landings, and stairs will be poured integrally to provide lateral support for the abutment. In this alternative the horizontal load path includes the stairs and landings acting as diaphragms to distribute the loads to the piles beneath the existing abutment footing and to the existing piles supporting the stair columns.

Thicknesses for concrete members have initially been set as 24” and will be refined during final analysis of the structure. The cast-in-place option provides the benefits of not requiring repeated painting throughout the life of the structure and the general look of the structure will largely remain unchanged in appearance.

### Alternative 2

Alternative 2 replaces the existing stairs with a new steel beam framing system using serrated galvanized steel grating landings and stairs. In this alternative each level of the stairs has a horizontal steel frame that sits on the new cast-in-place concrete beams attached to the east abutment and either stops at the proposed concrete columns or extends beyond the columns to form the landings furthest from the abutment. This framework is then braced in each direction to distribute the applied loading to both the footings and pile systems. Preliminary evaluations shows that framing members should be W12x26 steel beams with stair stringer members being C12x20. During final analysis of the structure, member sizes may change due to connection detailing and refined loading.

The steel beam and grating option provides the advantage of being the easiest option to construct with all the steel coming to the site the proper length and only requiring connection to other pieces with bolts. The drawback to using an all steel option is that approximately every 30 years the steel members will require painting and the galvanized serrated steel grating and stairs will require replacement to maintain the serration and anti-slip requirements.

### Alternative 3

Alternative 3 replaces the existing stairs with a new steel beam framing system using precast concrete landings and stairs. In this alternative each level of the stairs has a horizontal steel frame that sits on the new cast-in-place concrete beams attached to the east abutment and either stops at the proposed concrete columns or extends beyond the columns to form the landings furthest from the abutment. This framework is then braced in each direction to distribute the applied loading to both the footings and pile systems. Preliminary evaluations shows that framing members will be a combination of W12x50 and W12x26 steel beams. The landings and stairs will be precast concrete with an anti-slip broom finish formed into the concrete from the precast manufacturer. The precast stairs can be cast with an anti-slip nosing which will increase the durability of the steps as well as provide slip resistance. During final analysis of the structure, member sizes may change due to connection detailing and refined loading.

The steel beam and precast concrete landing and stairs option provides the advantage of combining the simpler construction of steel framing along with the durability of concrete while not requiring the more complicated formwork to construct the cast-in-place concrete option. The drawback to using the steel framing option with precast landings and stairs is that the steel members will require painting approximately every 30 years.

## Superstructure

This study is to evaluate two alternatives for rehabilitating the superstructure. The existing three span bridge has a walkway width of 8’-0” with concrete railings 1’-6” tall and 9” wide. Vandal protection fencing is provided on top of the railings. The concrete deck has a ¼” thick epoxy overlay wearing surface. One alternative is to remove the existing ¼” thick epoxy waterproofing overlay and replace it with a thin polymer epoxy overlay, ¼” thick. The second is to replace the existing superstructure with a new cast in place concrete deck on prestressed concrete box beams or I-beams.

### Rehabilitation Alternative

The major rehabilitation work proposed includes:

- Cleaning the deck drainage system

- Removing the existing ¼” epoxy waterproofing overlay

- Applying a ¼” thin polymer epoxy overlay

- Replacing the expansion joints

  - For the purposes of this evaluation, it is assumed the expansion joints will be replaced with elastomeric strip seals with steel cover plates.

- Jacking the superstructure, rotating it about the centerline of bearing at the rear abutment, to achieve 16’-6” vertical clearance under the bridge. Temporary support of the beams will be required

- Trimming the ends of the beams to compensate for the change in grade

- Replacing all the bearings with elastomeric bearings

- Providing shims or new concrete caps at the piers and forward abutment as required by the change in grade

  - Based on the amount the bridge will need to be raised at piers 1 and 2 and the forward abutment, it is assumed concrete caps will be utilized.

- Modifying the railing to meet current AASHTO requirements without removing existing rail

  - The current pedestrian railing shown in the ODOT Standard Drawings utilizes a 2’-0” tall concrete railing with two steel tubes. We propose adding a 1’-0” tall concrete cap to the existing 1’-6” concrete railing utilizing dowels to tie the two together. This will allow the installation of the anchor bolts for the steel tube railing posts, and possibly the vandal protection fence posts, in the new concrete cap. The total railing height will be 4’-0” instead of the 3’-6” shown on BR-2-15.

- Replacing the abutment backwalls

- Providing new porous backfill with geotextile fabric and drainage pipe behind the rear abutment

- Patching the substructure where required

  - For this evaluation, an area of 200 sq. ft. has been assumed for patching

- Providing fiber wrap on the piers as required to meet current AASHTO seismic requirements

  - For the purposes of this evaluation, it is assumed the fiber wrap will be required from the bottom of the cap to the top of footing.

- Removing and rebuilding the vandal protection fencing

An alternative to replacing the waterproofing wearing surface would be to replace the entire concrete deck. This would provide a longer life for the structure versus replacing the waterproofing wearing surface. Our field observations noted some cracking with leaching on the underside of the deck and minor spalling. However, after considering our field observations, the recent inspection reports, and the additional cost which would be incurred, we concur with replacing the waterproofing wearing surface.

### Replacement Alternative

During the initial meeting with District 11, the decision was made to evaluate only superstructure replacement types which eliminated the fracture critical nature of the existing bridge. This restricts the superstructure types to those with at least three lines of beams. For the span lengths of the bridge, 62’-0”, 78’-0”, and 81’-6”, rolled steel beams, prestressed precast concrete box beams or I-beams are feasible superstructure types.

Rolled steel beams were considered and rejected as a viable alternative due to the life-cycle cost of the beams. In this location, weathering steel does not appear to be a good option as the bridge is very close to meeting the tunnel-like conditions noted in the commentary for BDM 308.2.2.1.d.1. The only criteria the bridge does not meet is the posted speed limit for traffic under the bridge. The speed limit is 50 mph versus the 55 mph in the criteria. Therefore, a steel bridge with a coating system, whether galvanized or three-coat system, will require re-coating 2-3 times over the life of the bridge, making the life-cycle cost more expensive than for a precast concrete superstructure.

Prestressed, precast concrete box beams or I-beams require very little maintenance over the life of the bridge compared to a steel beam bridge. For this bridge, the prestressed concrete I-beams provide the most cost-effective superstructure. Preliminary design indicates Type 2 I-beams or CB33-36 box beams are required for the superstructure. Three lines of the I-beams or three lines of the box beams would be provided. The unit cost for the box beams ($20,000) is slightly higher than for the Type 2 I-beams ($18,300). However, the use of box beams would prevent the use of scuppers on the bridge, which will be required as the deck drainage must be collected before the end of the bridge due to the presence of the stairs. Therefore, the superstructure replacement alternative has been based on the use of the Type 2 I-beams.

# Alternative Comparison

Each alternative for both the Abutment Stairs and Superstructure were compared on the basis of initial construction, life cycle costs and the advantage/disadvantage of each alternative evaluated.

## Abutment Stairs

### Economic Comparison

Estimated unit costs for the various pay items have been based on the average unit bid costs shown in ODOT’s Historical Bid Data Item Search spreadsheet from 2019 through the first quarter of 2022, ODOT’s Estimator program and, where prices were not available in either location, unit costs were estimated by contacting manufacturers or through RSMeans construction cost data. A 30% contingency has been added to the cost for each alternative to cover items not specifically listed in the estimate. Costs are in current 2022 dollars and are not adjusted for inflation over the life of the bridge for the life-cycle costs

To determine the construction cost for each alternative, preliminary quantities were developed for the major cost drivers impacted by the different alternatives. These drivers included concrete quantities, reinforcing steel, structural steel members, pedestrian railing, galvanized steel stair treads, 1-1/2” serrated galvanized steel grating, shop painting of steel members, precast concrete slabs and precast concrete stairs. A breakdown of the preliminary quantities can be found in Appendix B.

Each alternative will have an anticipated life span of 75-100 years. The alternatives that include steel framing (Alternative 2 and Alternative 3) will incur additional life cycle costs of painting the steel members every 30 years or twice over the life of the structure. Alternate 2 will have an additional cost of replacing the galvanized serrated steel grating every 30 years, matching the number of times the structure requires painting.

The estimated initial construction costs are presented in the table below. The detailed breakdown for each alternative can be found in Appendix B. Please note these are total structure costs, not total project costs.

Alternative 1	Alternative 2	Alternative 3

(Cast-In-Place	(Steel Beam	(Steel Beam

Concrete)	and Grating)	Precast Concrete)

Estimated Initial Construction Cost	$386,129	$472,505	$587,847

Estimated Lifecycle Cost	$0

Painting (Year 30)		$164,290	$152,680

Grating Replacement (Year 30)		$79, 010	$0

Painting (Year 60)		$164,290	$152,680

Grating Replacement (Year 60)		$79,010	$0

Total over Life of Stairs	$382,839	$959,105	$893,207

### Discussion of Alternatives

Beyond the economic considerations, each alternative has advantages and drawbacks. These will be considered in this section for each alternative.

Alternate 1 - Cast-In-Place Concrete Replacement Advantages

- Lowest initial cost of construction and maintenance.

- Minimal change in look and design from existing stairs.

- No complicated steel connections to design, detail and manufacture.

- Increased durability of walking surfaces utilizing concrete with broom finish for slip resistance.

- Little vibration from live load.

- Eliminates concerns of dirt and other objects falling through grating onto pedestrians below.

Alternate 2 - Steel beam and Grating Replacement Advantages

- Faster and easier on-site construction.

  - Utilizes prefabricated steel beams, grating and connections.

  - No complicated forming and falsework for cast-in-place concrete stairs.

- Easy replacement of walking surfaces.

Alternate 3 - Steel beam and Precast Concrete Landings and Stairs Replacement Advantages

- Faster on-site construction.

  - Utilizes prefabricated steel beams, and precast concrete landings and stairs.

  - No complicated forming and falsework for cast-in-place stairs.

- Increased durability of walking surfaces utilizing concrete with broom finish for slip resistance.

- Little vibration from live load.

- Eliminates concerns of dirt and other objects falling through grating onto pedestrians below.

- Walking surfaces will be easier to replace than the cast-in-place option but will require heavier equipment to replace than the Steel beam and Grating option.

## Superstructure

### Economic Comparison

Estimated unit costs for the various pay items have been based on the average unit bid costs shown in ODOT’s Historical Bid Data Item Search spreadsheet from 2015 through the first quarter of 2022. For a breakdown of the costs, see Appendix B. A 30% contingency has been added to the cost for each alternative to cover items not specifically listed in the estimate. Costs are in current 2022 dollars and are not adjusted for inflation over the life of the bridge for the life-cycle costs.

The superstructure replacement is anticipated to have a 75-100 year life. The piers and abutments are approximately 60 years old, are in good condition, and should continue to adequately support the superstructure throughout this life-span with minor maintenance. The only anticipated maintenance cost during the life of the bridge is removal and replacement of the concrete sealing and minor patching.

For the superstructure rehabilitation to provide a similar life span, major maintenance will be required during the life span. The major items will be painting and replacement of the deck. The bridge was painted in 1999, so the current paint coating is nearing the end of the expected 30-year service. Therefore, the bridge will require painting in approximately 8 years and twice after that in the 75-100 life cycle comparison. Additionally, based on the cracking currently visible in the underside of the deck, it can be anticipated the deck will need to be replaced once during this life cycle. Other anticipated maintenance costs during the life of the bridge will be removal and replacement of the concrete sealing and minor patching. Since this is common to both alternatives, no cost is assigned to this work.

Rehabilitation Option	Replacement Option

Estimated Initial Construction Cost	$550,380	$986,315

Estimated Life Cycle Cost		$0

Painting (Year 8)	$226,470

Painting (Year 38)	$226,470

Painting (Year 68)	$226,470

Deck Replacement	$319,495

Total Estimated Cost over Life of Bridge	$1,549,285	$986,315

### Discussion of Alternative

Beyond the economic considerations, each alternative has significant advantages and drawbacks. These will be considered in this section for each alternative. Typically, what is an advantage for one alternative is a drawback for the other, so the drawbacks are not listed for each alternative.

Bridge Rehabilitation Advantages

- Lower initial construction cost.

- Shorter duration of construction. The superstructure replacement will require removal of the existing superstructure, fabrication and setting of beams, setting formwork for and placing the concrete slab all of which will require significantly more time.

- Less disruption to traffic on SR 7 during initial construction. The superstructure replacement will require short-term closures of SR 7 for removal of existing beams and for setting proposed beams. Closures may also be required while placing the formwork for the new concrete slab.

- Minimal coordination required with the railroad. The superstructure replacement will require coordination with the railroad for removing the existing steel beams and setting the new beams.

Superstructure Replacement Advantages

- Lower total cost over the life of the structure.

- Less disruption to traffic on SR 7 over the life of the bridge. The bridge rehabilitation will require several lane closures and/or detours on SR 7 over the life of the bridge for painting the structural steel and for removing and replacing the concrete slab.

- Eliminates the cost and risk associated with jacking the superstructure different heights at each substructure.

- Minimal maintenance over the life of the structure. The bridge rehabilitation will require repeated painting of the structural steel and removing and replacing the concrete slab once over the life of the structure.

The new superstructure would carry a significantly larger live load. The original bridge was designed to carry 85 psf live load. With the additional height added to the curb in the 1998 rehab and the further addition proposed by this rehab, the live load capacity of the bridge has been reduced. The new superstructure would be designed for 90 psf which, when load factors are considered, is equivalent to an actual live load of 158 psf. (AASHTO LRFD Guide Specifications for the Design of Pedestrian Bridges, 2009) Thus, the new superstructure would support nearly twice the load the existing superstructure is capable of supporting.

# Recommendations

Based on the information presented in this Structure Type Study, Stantec recommends the cast-in-place concrete stairs (Alternative 1) for the replacement of the existing stair access on the east abutment along with a complete replacement of the superstructure (Replacement Alternative). These alternatives provide the most economical solutions despite having some drawbacks to the construction.

- Appendix A drawings

Site Plan and Profile

Stair Replacement Alternatives

Cost Breakdowns

Stair Replacement Alternatives Cost Breakdown

Superstructure Rehabilitation and Replacement Cost Breakdown

Geotechnical Report