Survey Parameters: BEL-CR4-27.05

Project Control:

Positioning Method: ODOT Real Time Network (RTK)

Monument Type: Type B

Vertical Positioning:

Orthometric Height Datum: NAVD 88

Geoid: 18

Horizontal Positioning:

Reference Frame: NAD83 (2011 ADJ, EPOCH 2010.0)

Ellipsoid: GRS 80

Ohio County Coordinate System: Belmont County LDP*

Projection: Transverse Mercator

Central Latitude: N 38°30’00”

Central Longitude: W 81°00’00”

False Northing: 0 Meters

False Easting 50,000 Meters

Project Scale Factor: 1.000041

Units are in U.S. Survey Feet

* LDP – Low Distortion Projection: A local county project developed by ODOT. The Distortion between Ground and Grid is so minimal that there is no need for a Scale Factor to adjust between Grid and Ground Coordinates. Contact the District Survey Department for further information or questions.

Coordinates:

| Name | Easting | Northing | Elevation |
| --- | --- | --- | --- |
| CP01 | 241538.374 | 588851.551 | 715.451 |
| CP02 | 241760.740 | 588977.247 | 704.649 |
| CP03 | 242093.009 | 589319.767 | 701.657 |

Control Points:

Proposed Property Points:

| Name | Easting | Northing |
| --- | --- | --- |
| PP100 | 241838.754 | 589028.081 |
| PP101 | 241852.870 | 589040.258 |
| PP102 | 241875.609 | 589061.870 |
| PP103 | 241901.939 | 589084.948 |