ABSTRACT

This document reports the hydraulic analysis performed for project BEL-149-5.32, PID 119917 for the rehabilitation of the State Route 149 Bridge (SFN 0706736) over Little McMahon Creek in Pultney Township, Belmont County, Ohio.

The document covers the hydraulic analysis for Little McMahon Creek at the structure using information from USGS Ohio StreamStats, survey, and other historical information in HEC-RAS modeling

software.

ABSTRACT

This document reports the hydraulic analysis performed for project BEL-149-5.32, PID 119917 for the rehabilitation of the State Route 149 Bridge (SFN 0706736) over Little McMahon Creek in Pultney Township, Belmont County, Ohio.

The document covers the hydraulic analysis for Little McMahon Creek at the structure using information from USGS Ohio StreamStats, survey, and other historical information in HEC-RAS modeling

software.

Introduction

Due to deteriorating structural elements, Bridge No. BEL-149-05.320 (SFN: 0706736) is being repaired under the PID 119917. Bridge No. BEL-149-05.320 carries S.R. 149 over Little McMahon Creek. S.R. 149 is classified as a Major Collector (Rural) with a Design Year ADT of 3,000 and 24-Hour B&C Truck of 4%. The stream was analyzed using topographic survey information, USGS Ohio StreamStats, and HEC-RAS modeling software. Both the existing and proposed conditions have been analyzed, and the results are reported herein.

Drainage Basin

According to USGS Ohio StreamStats, the drainage basin area for Little McMahon Creek at the project

location is 14.3 square miles. The existing plan lists a drainage area of 14.26 square miles, which validates the accuracy of the StreamStats results. The drainage basin consists of 47% forest and 0.8% covered by water and wetlands. For the Basin Characteristics Report, see Appendix A.

Existing Condition

The existing structure is a continuous concrete slab with stub abutments and cap and column piers. The existing structure is shown on the Site Plan included in Appendix C. Little McMahon Creek passes under S.R. 149 at approximately a 09˚30’00” ± R.F. skew. The existing abutments are stub type on concrete piles, and the existing piers are cap and column. The drilled shafts supporting the capped piers are beginning to experience scour and section loss. All substructure units are skewed to align with Little McMahon Creek. The confluence of Little McMahon Creek and McMahon Creek occurs approximately 1330’ (0.25 miles) downstream of the structure. Hydraulic analysis was completed for the 25- and 100-year (4% AEP and 1% AEP) frequencies in accordance with ODOT Location and Design (L&D) Manual Volume 2 § 1004.2. In addition, hydraulic analysis was completed for the 2-year (50% AEP) frequency.

Proposed Condition

The proposed structure rehabilitation will involve removal of existing concrete of the drilled shafts until they are flush with the existing concrete pier columns. The existing pier columns will then be encased in concrete. The stay-in-place encasement concrete will be placed in a 48” diameter polyethylene or pvc pipe that will be used as a stay-in-place form. To protect against erosion, rock channel protection will be extended and placed in front of the drilled shafts. Hydraulic analysis was completed for the 2-, 25-, and 100-year (50% AEP, 4% AEP, & 1% AEP) frequencies based on available data and in accordance with L&D Manual Volume 2 § 1004.2.

FEMA Flood Insurance Rate Map (FIRM)

A copied portion of the applicable FEMA Flood Insurance Rate Map (FIRM) is included in Appendix B. The FIRM indicates the structure is in Zone A.  For this project, L&D Vol. 2 1006.3 sets the parameters for the bridge headwater controls are: 1) attempt to match the existing headwater for the design (25 year, 4% AEP) storm, and 2) do not allow the design (25 year, 4% AEP) flood to contact the low chord of the proposed structure. This location is not inherently part of the FIS and is not within the floodway. The low chord elevation 727.88 of the bridge is located at the downstream deck edge beam at the face of the rear abutment. The existing inside downstream 25-year, 4% AEP flood elevation 726.37 clears the low chord of the bridge by 1.51 feet. The proposed inside downstream 25-year, 4% AEP flood elevation 726.50 clears the low chord of the bridge by 1.38 feet.

Hydraulic Model

General Setup

The project site was modeled using a combination of topographic survey information (TIN model) and the original construction plans. Cross section locations were selected to properly represent significant topography changes and provide the most accurate model. HEC-RAS requires the cross sections to be placed perpendicular to the direction of flow and divided into 3 sections – left overbank, main channel, and right overbank. The cross-section layout can be seen in the HEC-RAS Model Plan View in Appendix D. In addition, see Appendix D for HEC-RAS Cross Sections at the existing and proposed structure conditions.

Flow Rates

The peak flow rates were estimated using the USGS Ohio StreamStats modeling software/website. The results are tabulated below:

| USGS Ohio StreamStats Peak Flows |  |  |
| --- | --- | --- |
| Q2 (50% AEP) | 1,510 | cubic feet per second |
| Q25 (4% AEP) | 2,730 | cubic feet per second |
| Q100 (1% AEP) | 3,950 | cubic feet per second |

To view the complete Peak Flows Streamflow Statistics table, see Appendix A.

Study Limitations

The limitations of hydraulic analysis have been minimized to the maximum extent practicable. As discussed above, the main limitation is the need to estimate the 25-year (4% AEP) downstream water surface elevation.

Results

HEC-RAS was used to compute the headwater (HW) elevations and channel velocities. HEC-RAS Output Files are shown for existing conditions in Table 1 and proposed conditions in Table 2 in Appendix D.

Scour

This report did not analyze scour using HEC-RAS and HEC 18. Due to the presence of bedrock close to the surface and the bridge foundations being socketed 10’ into the rock, the department shall instead monitor potential scour during annual inspections.

Flood Hazard Evaluation

The existing structure clears the design (25-year, 4% AEP) storm event, but the 100-year (1% AEP) storm event overtops the existing roadway. The proposed structure condition clears the design (25-year, 4% AEP) storm event, but the 100-year (1% AEP) storm event overtops the existing roadway. A preliminary Proposed Structure Site Plan has been included in Appendix C and displays the existing and proposed 25- and 100-year (4% AEP and 1% AEP) storm event HW elevations.

The existing roadway profile was not altered by this work. A summary of existing and proposed HEC-RAS results is provided below. For more detailed HEC-RAS Output Files, see Appendix D.

| Elevations at Lowest Superstructure Point (Upstream) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Profile Grade | Low Chord | HW 25 (4% AEP) | HW 100 (1% AEP) |
| Ex. 3-Span Slab | 731.10 | 730.21 | 727.89 | 731.13 |
| Pr. 3-Span Slab | 731.10 | 730.21 | 728.25 | 730.12 |

• The existing and proposed structures clear the design (25-year, 4% AEP) storm event.

• The existing structure has 1.96’ of freeboard for the design storm (at the upstream side).

• The proposed structure has 1.87’ of freeboard for the design storm (at the upstream side).

• The existing and proposed structures are overtopped by the check (100-year, 1% AEP) storm event.

The channel velocities of the existing and rehabilitated structures for the design (25-year, 4% AEP) storm event are 9.35 ft/s and 8.42 ft/s, and the 100-year storm event are 9.05 ft/s and 9.48 ft/s (at the upstream side) respectively. Scour is not currently an issue at this location. The proposed regrading of the sloping and rock channel protection under the bridge contributes to reducing design stream velocities.

Based on the HEC-RAS analysis, no additional flooding will be incurred due to the proposed project.

APPENDIX A:

StreamStats Output

APPENDIX B:

FIRM Map

APPENDIX D:

HEC-Ras Output

Figure 1.1: HEC-Ras Model Plan View

Table 1: HEC-Ras Existing Reach Output Data

Table 1: HEC-Ras Existing Bridge Reach Output Data

Figure 1.2: HEC-Ras Existing Profile Output Data

Figure 1.3: HEC-Ras Existing Cross Sections Output Data

Figure 1.4: HEC-Ras Existing Cross Sections Output Data

Figure 1.5: HEC-Ras Existing Cross Sections Output Data

Figure 1.6: HEC-Ras Existing Cross Sections Output Data

Figure 1.7: HEC-Ras Existing Cross Sections Output Data

Figure 1.8: HEC-Ras Existing Cross Sections Output Data

Figure 1.9: HEC-Ras Existing Cross Sections Output Data

Figure 1.10: HEC-Ras Existing Cross Sections Output Data

Figure 1.11: HEC-Ras Existing Cross Sections Output Data

Figure 1.12: HEC-Ras Existing Cross Sections Output Data

Figure 1.13: HEC-Ras Existing Cross Sections Output Data

Figure 1.14: HEC-Ras Existing Cross Sections Output Data

Figure 1.15: HEC-Ras Existing Cross Sections Output Data

Figure 1.16: HEC-Ras Existing Cross Sections Output Data

Figure 1.17: HEC-Ras Existing Cross Sections Output Data

Figure 1.18: HEC-Ras Existing Cross Sections Output Data

Figure 1.19: HEC-Ras Existing Cross Sections Output Data

Figure 1.20: HEC-Ras Existing Cross Sections Output Data