# HY-8 Culvert Analysis Report

# Table 1 - Project Headwater Table

| Crossing Name | Culvert Name | Discharge Names | Total Discharge (cfs) | Culvert Discharge (cfs) | Headwater Elevation (ft) | Inlet Control Depth (ft) | Outlet Control Depth (ft) | HW / D (ft) | Normal Depth (ft) | Critical Depth (ft) | Outlet Depth (ft) | Outlet Velocity (ft/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TH-SF2-Culvert_Exist | Existing | 100 year | 800.00 | 293.59 | 126.12 | 33.19 | 27.466 | 6.53 | 5.08 | 5.08 | 5.08 | 24.37 |
| TH-SF2-Culvert_Prop | Proposed | 100 year | 800.00 | 800.00 | 101.30 | 8.25 | 8.302 | 1.38 | -1.00 | 5.17 | 5.17 | 12.90 |

### Site Data - Existing

Site Data Option: Culvert Invert Data

Inlet Station: 0.00 ft

Inlet Elevation: 92.43 ft

Outlet Station: 39.58 ft

Outlet Elevation: 92.23 ft

Number of Barrels: 1

### Culvert Data Summary - Existing

Barrel Shape: Pipe Arch

Barrel Span: 95.00 in

Barrel Rise: 67.00 in

Barrel Material: Steel or Aluminum

Embedment: 6.00 in

Barrel Manning's n: 0.0280 (top and sides)

Manning's n: 0.0300 (bottom)

Culvert Type: Straight

Inlet Configuration: Thin Edge Projecting (Ke=0.9)

Inlet Depression: None

### Table 2 - Culvert Summary Table: Existing

| Discharge Names | Total Discharge (cfs) | Culvert Discharge (cfs) | Headwater Elevation (ft) | Inlet Control Depth (ft) | Outlet Control Depth (ft) | HW / D (ft) | Flow Type | Normal Depth (ft) | Critical Depth (ft) | Outlet Depth (ft) | Tailwater Depth (ft) | Outlet Velocity (ft/s) | Tailwater Velocity (ft/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 year | 361.00 | 265.42 | 99.49 | 6.55 | 6.564 | 1.29 | 7-M2c | 5.08 | 3.27 | 3.27 | 1.12 | 10.88 | 11.79 |
| 25 year | 519.00 | 277.41 | 99.74 | 6.81 | 6.812 | 1.34 | 7-M2c | 5.08 | 3.36 | 3.36 | 1.39 | 11.11 | 13.42 |
| 50 year | 653.00 | 285.66 | 99.92 | 6.98 | 6.986 | 1.37 | 7-M2c | 5.08 | 3.42 | 3.42 | 1.59 | 11.26 | 14.55 |
| 100 year | 800.00 | 293.59 | 126.12 | 33.19 | 27.466 | 6.53 | 6-FFc | 5.08 | 5.08 | 5.08 | 1.79 | 24.37 | 15.60 |
| Overtopping | 800.00 | 293.59 | 100.09 | 7.15 | 7.156 | 1.41 | 7-M2c | 5.08 | 3.48 | 3.48 | 1.79 | 11.41 | 15.60 |

### Culvert Barrel Data

Culvert Barrel Type: Straight Culvert

Inlet Elevation(invert): 92.93 ft

Outlet Elevation (invert): 92.73 ft

Culvert Length: 39.58 ft

Culvert Slope: 0.01 ft/ft

## Tailwater Channel Data for Crossing: TH-SF2-Culvert_Exist

Tailwater Channel Option: Trapezoidal Channel

Bottom Width: 25.00 ft

a_side Slope (H:V): 2.00 (_:1)

Channel Slope: 0.01 ft/ft

Channel Manning's n: 0.0150

Channel Invert Elevation: 92.00 ft

## Table 3 - Downstream Channel Rating Curve (crossing: TH-SF2-Culvert_Exist)

| Flow (cfs) | Water Surface Elev (ft) | Depth (ft) | Velocity (ft/s) | Shear (psf) | Froude Number |
| --- | --- | --- | --- | --- | --- |
| 361.00 | 93.12 | 1.12 | 11.79 | 0.97 | 2.04 |
| 519.00 | 93.39 | 1.39 | 13.42 | 1.20 | 2.10 |
| 653.00 | 93.59 | 1.59 | 14.55 | 1.37 | 2.14 |
| 800.00 | 93.79 | 1.79 | 15.60 | 1.54 | 2.18 |

## Roadway Data for crossing: TH-SF2-Culvert_Exist

Roadway Profile Shape: Constant Roadway Elevation

Crest Length: 200.00 ft

Crest Elevation: 99.20 ft

Roadway Surface: Paved

Roadway Top Width: 14.00 ft

## Crossing Discharge Data

Discharge Selection Method: Recurrence

### Table 4 - Summary of Culvert Flows at crossing: TH-SF2-Culvert_Exist

| Headwater Elevation (ft) | Discharge Names | Total Discharge (cfs) | Existing Discharge (cfs) | Roadway Discharge (cfs) | Iterations |
| --- | --- | --- | --- | --- | --- |
| 99.49 | 10 year | 361.00 | 265.42 | 95.37 | 13 |
| 99.74 | 25 year | 519.00 | 277.41 | 241.54 | 6 |
| 99.92 | 50 year | 653.00 | 285.66 | 367.31 | 5 |
| 126.12 | 100 year | 800.00 | 293.59 | 506.36 | 4 |
| 99.20 | Overtopping | 250.81 | 250.81 | 0.00 | Overtopping |

### Site Data - Proposed

Site Data Option: Culvert Invert Data

Inlet Station: 0.00 ft

Inlet Elevation: 92.00 ft

Outlet Station: 60.00 ft

Outlet Elevation: 92.00 ft

Number of Barrels: 1

### Culvert Data Summary - Proposed

Barrel Shape: Concrete Box

Barrel Span: 12.00 ft

Barrel Rise: 7.00 ft

Barrel Material: Concrete

Embedment: 12.00 in

Barrel Manning's n: 0.0120 (top and sides)

Manning's n: 0.0350 (bottom)

Culvert Type: Straight

Inlet Configuration: 1.5:1 Bevel (90º) Headwall (Ke=0.2)

Inlet Depression: None

### Table 5 - Culvert Summary Table: Proposed

| Discharge Names | Total Discharge (cfs) | Culvert Discharge (cfs) | Headwater Elevation (ft) | Inlet Control Depth (ft) | Outlet Control Depth (ft) | HW / D (ft) | Flow Type | Normal Depth (ft) | Critical Depth (ft) | Outlet Depth (ft) | Tailwater Depth (ft) | Outlet Velocity (ft/s) | Tailwater Velocity (ft/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 year | 361.00 | 361.00 | 98.14 | 4.74 | 5.143 | 0.86 | 7-H2c | -1.00 | 3.04 | 3.04 | 2.14 | 9.89 | 5.76 |
| 25 year | 519.00 | 519.00 | 99.46 | 6.03 | 6.464 | 1.08 | 7-H2c | -1.00 | 3.87 | 3.87 | 2.64 | 11.17 | 6.50 |
| 50 year | 653.00 | 653.00 | 100.48 | 7.07 | 7.480 | 1.25 | 7-H2c | -1.00 | 4.51 | 4.51 | 3.01 | 12.06 | 7.00 |
| 100 year | 800.00 | 800.00 | 101.30 | 8.25 | 8.302 | 1.38 | 7-H2c | -1.00 | 5.17 | 5.17 | 3.37 | 12.90 | 7.47 |
| Overtopping | 1000.00 | 896.98 | 102.09 | 9.09 | 8.894 | 1.51 | 7-H2t | -1.00 | 5.58 | 5.58 | 3.82 | 13.40 | 8.01 |

### Culvert Barrel Data

Culvert Barrel Type: Straight Culvert

Inlet Elevation(invert): 93.00 ft

Outlet Elevation (invert): 93.00 ft

Culvert Length: 60.00 ft

Culvert Slope: 0.00 ft/ft

## Tailwater Channel Data for Crossing: TH-SF2-Culvert_Prop

Tailwater Channel Option: Trapezoidal Channel

Bottom Width: 25.00 ft

a_side Slope (H:V): 2.00 (_:1)

Channel Slope: 0.01 ft/ft

Channel Manning's n: 0.0450

Channel Invert Elevation: 92.00 ft

## Table 6 - Downstream Channel Rating Curve (crossing: TH-SF2-Culvert_Prop)

| Flow (cfs) | Water Surface Elev (ft) | Depth (ft) | Velocity (ft/s) | Shear (psf) | Froude Number |
| --- | --- | --- | --- | --- | --- |
| 361.00 | 94.14 | 2.14 | 5.76 | 1.84 | 0.74 |
| 519.00 | 94.64 | 2.64 | 6.50 | 2.27 | 0.76 |
| 653.00 | 95.01 | 3.01 | 7.00 | 2.59 | 0.78 |
| 800.00 | 95.37 | 3.37 | 7.47 | 2.91 | 0.79 |

## Roadway Data for crossing: TH-SF2-Culvert_Prop

Roadway Profile Shape: Constant Roadway Elevation

Crest Length: 200.00 ft

Crest Elevation: 101.78 ft

Roadway Surface: Paved

Roadway Top Width: 15.00 ft

## Crossing Discharge Data

Discharge Selection Method: Recurrence

### Table 7 - Summary of Culvert Flows at crossing: TH-SF2-Culvert_Prop

| Headwater Elevation (ft) | Discharge Names | Total Discharge (cfs) | Proposed Discharge (cfs) | Roadway Discharge (cfs) | Iterations |
| --- | --- | --- | --- | --- | --- |
| 98.14 | 10 year | 361.00 | 361.00 | 0.00 | 1 |
| 99.46 | 25 year | 519.00 | 519.00 | 0.00 | 1 |
| 100.48 | 50 year | 653.00 | 653.00 | 0.00 | 1 |
| 101.30 | 100 year | 800.00 | 800.00 | 0.00 | 1 |
| 101.78 | Overtopping | 862.10 | 862.10 | 0.00 | Overtopping |