# HY-8 Culvert Analysis Report

# Culvert Data: Existing

## Site Data - Existing

Site Data Option: Culvert Invert Data

Inlet Station: 0.00 ft

Inlet Elevation: 92.43 ft

Outlet Station: 39.58 ft

Outlet Elevation: 92.23 ft

Number of Barrels: 1

## Culvert Data Summary - Existing

Barrel Shape: Pipe Arch

Barrel Span: 95.00 in

Barrel Rise: 67.00 in

Barrel Material: Steel or Aluminum

Embedment: 6.00 in

Barrel Manning's n: 0.0280 (top and sides)

Manning's n: 0.0300 (bottom)

Culvert Type: Straight

Inlet Configuration: Projecting

Inlet Depression: None

### Table 1 - Culvert Summary Table: Existing

| Total Discharge (cfs) | Culvert Discharge (cfs) | Headwater Elevation (ft) | Inlet Control Depth (ft) | Outlet Control Depth (ft) | Flow Type | Normal Depth (ft) | Critical Depth (ft) | Outlet Depth (ft) | Tailwater Depth (ft) | Outlet Velocity (ft/s) | Tailwater Velocity (ft/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 722.00 cfs | 289.44 cfs | 100.00 | 7.06 | 7.066 | 7-M2c | 5.08 | 3.45 | 3.45 | 1.69 | 11.34 | 15.06 |
| 777.80 cfs | 292.43 cfs | 100.06 | 7.13 | 7.131 | 7-M2c | 5.08 | 3.47 | 3.47 | 1.76 | 11.39 | 15.45 |
| 833.60 cfs | 295.27 cfs | 100.12 | 7.19 | 7.193 | 7-M2c | 5.08 | 3.49 | 3.49 | 1.84 | 11.45 | 15.83 |
| 889.40 cfs | 297.67 cfs | 100.18 | 7.25 | 7.246 | 7-M2c | 5.08 | 3.50 | 3.50 | 1.91 | 11.49 | 16.18 |
| 945.20 cfs | 299.73 cfs | 100.24 | 7.31 | 7.291 | 7-M2c | 5.08 | 3.52 | 3.52 | 1.98 | 11.53 | 16.52 |
| 1001.00 cfs | 301.72 cfs | 100.30 | 7.37 | 7.336 | 7-M2c | 5.08 | 3.53 | 3.53 | 2.04 | 11.57 | 16.84 |
| 1030.00 cfs | 302.74 cfs | 100.33 | 7.40 | 7.359 | 7-M2c | 5.08 | 3.54 | 3.54 | 2.08 | 11.59 | 17.00 |
| 1112.60 cfs | 305.53 cfs | 100.41 | 7.48 | 7.422 | 7-M2c | 5.08 | 3.56 | 3.56 | 2.17 | 11.64 | 17.45 |
| 1168.40 cfs | 307.39 cfs | 100.46 | 7.53 | 7.464 | 7-M2c | 5.08 | 3.57 | 3.57 | 2.23 | 11.67 | 17.74 |
| 1224.20 cfs | 309.20 cfs | 100.51 | 7.58 | 7.506 | 7-M2c | 5.08 | 3.59 | 3.59 | 2.30 | 11.71 | 18.02 |
| 1280.00 cfs | 310.98 cfs | 100.57 | 7.64 | 7.547 | 7-M2c | 5.08 | 3.60 | 3.60 | 2.36 | 11.74 | 18.29 |

## Culvert Barrel Data

Culvert Barrel Type Straight Culvert

Inlet Elevation (invert): 92.93 ft,

Outlet Elevation (invert): 92.73 ft

Culvert Length: 39.58 ft,

Culvert Slope: 0.0051

### Culvert Performance Curve Plot: Existing

### Water Surface Profile Plot for Culvert: Existing

## Tailwater Data for Crossing: TH-SF2-Culvert_Exist

### Table 2 - Downstream Channel Rating Curve (Crossing: TH-SF2-Culvert_Exist)

| Flow (cfs) | Water Surface Elev (ft) | Velocity (ft/s) | Depth (ft) | Shear (psf) | Froude Number |
| --- | --- | --- | --- | --- | --- |
| 722.00 | 93.69 | 1.69 | 15.06 | 1.45 | 2.16 |
| 777.80 | 93.76 | 1.76 | 15.45 | 1.52 | 2.17 |
| 833.60 | 93.84 | 1.84 | 15.83 | 1.58 | 2.19 |
| 889.40 | 93.91 | 1.91 | 16.18 | 1.64 | 2.20 |
| 945.20 | 93.98 | 1.98 | 16.52 | 1.70 | 2.21 |
| 1001.00 | 94.04 | 2.04 | 16.84 | 1.76 | 2.22 |
| 1030.00 | 94.08 | 2.08 | 17.00 | 1.79 | 2.22 |
| 1112.60 | 94.17 | 2.17 | 17.45 | 1.87 | 2.24 |
| 1168.40 | 94.23 | 2.23 | 17.74 | 1.92 | 2.24 |
| 1224.20 | 94.30 | 2.30 | 18.02 | 1.98 | 2.25 |
| 1280.00 | 94.36 | 2.36 | 18.29 | 2.03 | 2.26 |

## Tailwater Channel Data - TH-SF2-Culvert_Exist

Tailwater Channel Option: Trapezoidal Channel

Bottom Width: 25.00 ft

Side Slope (H:V): 2.00 (_:1)

Channel Slope: 0.0138

Channel Manning's n: 0.0150

Channel Invert Elevation: 92.00 ft

## Roadway Data for Crossing: TH-SF2-Culvert_Exist

Roadway Profile Shape: Constant Roadway Elevation

Crest Length: 200.00 ft

Crest Elevation: 99.20 ft

Roadway Surface: Paved

Roadway Top Width: 14.00 ft

## Crossing Discharge Data

Discharge Selection Method: Specify Minimum, Design, and Maximum Flow

Minimum Flow: 722.00 cfs

Design Flow: 1030.00 cfs

Maximum Flow: 1280.00 cfs

### Table 3 - Summary of Culvert Flows at Crossing: TH-SF2-Culvert_Exist

| Headwater Elevation (ft) | Total Discharge (cfs) | Existing Discharge (cfs) | Roadway Discharge (cfs) | Iterations |
| --- | --- | --- | --- | --- |
| 100.00 | 722.00 | 289.44 | 431.56 | 10 |
| 100.06 | 777.80 | 292.43 | 485.14 | 4 |
| 100.12 | 833.60 | 295.27 | 538.19 | 4 |
| 100.18 | 889.40 | 297.67 | 590.77 | 3 |
| 100.24 | 945.20 | 299.73 | 644.62 | 3 |
| 100.30 | 1001.00 | 301.72 | 698.62 | 3 |
| 100.33 | 1030.00 | 302.74 | 726.94 | 3 |
| 100.41 | 1112.60 | 305.53 | 806.54 | 3 |
| 100.46 | 1168.40 | 307.39 | 860.73 | 3 |
| 100.51 | 1224.20 | 309.20 | 914.81 | 3 |
| 100.57 | 1280.00 | 310.98 | 968.89 | 3 |
| 99.20 | 250.80 | 250.80 | 0.00 | Overtopping |

### Rating Curve Plot for Crossing: TH-SF2-Culvert_Exist

# Culvert Data: Proposed

## Site Data - Proposed

Site Data Option: Culvert Invert Data

Inlet Station: 0.00 ft

Inlet Elevation: 92.00 ft

Outlet Station: 60.00 ft

Outlet Elevation: 92.00 ft

Number of Barrels: 1

## Culvert Data Summary - Proposed

Barrel Shape: Concrete Box

Barrel Span: 18.00 ft

Barrel Rise: 7.00 ft

Barrel Material: Concrete

Embedment: 12.00 in

Barrel Manning's n: 0.0120 (top and sides)

Manning's n: 0.0350 (bottom)

Culvert Type: Straight

Inlet Configuration: 1.5:1 Bevel (90º) Headwall (Ke=0.2)

Inlet Depression: None

### Table 4 - Culvert Summary Table: Proposed

| Total Discharge (cfs) | Culvert Discharge (cfs) | Headwater Elevation (ft) | Inlet Control Depth (ft) | Outlet Control Depth (ft) | Flow Type | Normal Depth (ft) | Critical Depth (ft) | Outlet Depth (ft) | Tailwater Depth (ft) | Outlet Velocity (ft/s) | Tailwater Velocity (ft/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 722.00 cfs | 722.00 cfs | 99.15 | 5.73 | 6.147 | 7-H2c | -1.00 | 3.68 | 3.68 | 3.18 | 10.89 | 7.23 |
| 806.80 cfs | 806.80 cfs | 99.59 | 6.17 | 6.595 | 7-H2c | -1.00 | 3.97 | 3.97 | 3.39 | 11.30 | 7.49 |
| 891.60 cfs | 891.60 cfs | 100.03 | 6.61 | 7.027 | 7-H2c | -1.00 | 4.24 | 4.24 | 3.59 | 11.68 | 7.73 |
| 976.40 cfs | 976.40 cfs | 100.45 | 7.05 | 7.446 | 7-H2c | -1.00 | 4.50 | 4.50 | 3.77 | 12.04 | 7.95 |
| 1030.00 cfs | 1030.00 cfs | 100.70 | 7.33 | 7.696 | 7-H2c | -1.00 | 4.67 | 4.67 | 3.89 | 12.26 | 8.08 |
| 1146.00 cfs | 1146.00 cfs | 101.10 | 7.95 | 8.100 | 7-H2c | -1.00 | 5.01 | 5.01 | 4.13 | 12.70 | 8.35 |
| 1230.80 cfs | 1230.80 cfs | 101.42 | 8.42 | 8.422 | 7-H2c | -1.00 | 5.26 | 5.26 | 4.29 | 13.01 | 8.53 |
| 1315.60 cfs | 1304.85 cfs | 101.85 | 8.85 | 8.722 | 7-H2t | -1.00 | 5.46 | 5.46 | 4.46 | 13.27 | 8.71 |
| 1400.40 cfs | 1332.81 cfs | 102.01 | 9.01 | 8.840 | 7-H2t | -1.00 | 5.54 | 5.54 | 4.61 | 13.36 | 8.87 |
| 1485.20 cfs | 1354.15 cfs | 102.14 | 9.14 | 8.932 | 7-H2t | -1.00 | 5.60 | 5.60 | 4.76 | 13.43 | 9.03 |
| 1570.00 cfs | 1372.53 cfs | 102.25 | 9.25 | 9.012 | 7-H2t | -1.00 | 5.65 | 5.65 | 4.91 | 13.49 | 9.18 |

## Culvert Barrel Data

Culvert Barrel Type Straight Culvert

Inlet Elevation (invert): 93.00 ft,

Outlet Elevation (invert): 93.00 ft

Culvert Length: 60.00 ft,

Culvert Slope: 0.0000

### Culvert Performance Curve Plot: Proposed

### Water Surface Profile Plot for Culvert: Proposed

## Tailwater Data for Crossing: TH-SF2-Culvert_Prop

### Table 5 - Downstream Channel Rating Curve (Crossing: TH-SF2-Culvert_Prop)

| Flow (cfs) | Water Surface Elev (ft) | Velocity (ft/s) | Depth (ft) | Shear (psf) | Froude Number |
| --- | --- | --- | --- | --- | --- |
| 722.00 | 95.18 | 3.18 | 7.23 | 2.74 | 0.78 |
| 806.80 | 95.39 | 3.39 | 7.49 | 2.92 | 0.79 |
| 891.60 | 95.59 | 3.59 | 7.73 | 3.09 | 0.80 |
| 976.40 | 95.77 | 3.77 | 7.95 | 3.25 | 0.80 |
| 1030.00 | 95.89 | 3.89 | 8.08 | 3.35 | 0.80 |
| 1146.00 | 96.13 | 4.13 | 8.35 | 3.55 | 0.81 |
| 1230.80 | 96.29 | 4.29 | 8.53 | 3.70 | 0.81 |
| 1315.60 | 96.46 | 4.46 | 8.71 | 3.84 | 0.82 |
| 1400.40 | 96.61 | 4.61 | 8.87 | 3.97 | 0.82 |
| 1485.20 | 96.76 | 4.76 | 9.03 | 4.10 | 0.82 |
| 1570.00 | 96.91 | 4.91 | 9.18 | 4.23 | 0.83 |

## Tailwater Channel Data - TH-SF2-Culvert_Prop

Tailwater Channel Option: Trapezoidal Channel

Bottom Width: 25.00 ft

Side Slope (H:V): 2.00 (_:1)

Channel Slope: 0.0138

Channel Manning's n: 0.0450

Channel Invert Elevation: 92.00 ft

## Roadway Data for Crossing: TH-SF2-Culvert_Prop

Roadway Profile Shape: Constant Roadway Elevation

Crest Length: 200.00 ft

Crest Elevation: 101.78 ft

Roadway Surface: Paved

Roadway Top Width: 15.00 ft

## Crossing Discharge Data

Discharge Selection Method: Specify Minimum, Design, and Maximum Flow

Minimum Flow: 722.00 cfs

Design Flow: 1030.00 cfs

Maximum Flow: 1570.00 cfs

### Table 6 - Summary of Culvert Flows at Crossing: TH-SF2-Culvert_Prop

| Headwater Elevation (ft) | Total Discharge (cfs) | Proposed Discharge (cfs) | Roadway Discharge (cfs) | Iterations |
| --- | --- | --- | --- | --- |
| 99.15 | 722.00 | 722.00 | 0.00 | 1 |
| 99.59 | 806.80 | 806.80 | 0.00 | 1 |
| 100.03 | 891.60 | 891.60 | 0.00 | 1 |
| 100.45 | 976.40 | 976.40 | 0.00 | 1 |
| 100.70 | 1030.00 | 1030.00 | 0.00 | 1 |
| 101.10 | 1146.00 | 1146.00 | 0.00 | 1 |
| 101.42 | 1230.80 | 1230.80 | 0.00 | 1 |
| 101.85 | 1315.60 | 1304.85 | 10.44 | 10 |
| 102.01 | 1400.40 | 1332.81 | 67.19 | 6 |
| 102.14 | 1485.20 | 1354.15 | 130.60 | 5 |
| 102.25 | 1570.00 | 1372.53 | 196.51 | 4 |
| 101.78 | 1293.03 | 1293.03 | 0.00 | Overtopping |

### Rating Curve Plot for Crossing: TH-SF2-Culvert_Prop