| Ver. Date | 03/12/2024 |  | PID |  |  | 111621 |
| --- | --- | --- | --- | --- | --- | --- |
| PARCEL |  |  |  | 27-T |  |  |
| FAI-158-4.20 |  |  |  |  |  |  |

TEMPORARY EASEMENT FOR THE PURPOSE OF PERFORMING THE WORK NECESSARY TO REMOVE TREE FOR MONTHS FROM DATE OF ENTRY BY THE STATE OF OHIO, DEPARTMENT OF TRANSPORTATION

[Surveyor’s description of the premises follows]

Situated in the State of Ohio, County of Fairfield, Township of Greenfield, in the northeast quarter of Section 23, Township 15, Range XIX of the Congress Lands (East of Scioto River), lying on the right side of the centerline of right of way of Coonpath Road, (County Road 31, 60.00 feet wide) and being part of the 1.180 acres of land deeded, now or previously, to Holly A. Richardson in official record volume 1516, page 3001 of the Fairfield County Recorder’s Office and known as Fairfield County Auditor’s parcel number 013-00596.00 and further described as follows;

Beginning in the southerly line of Parcel 27-SH at a point 41.12 feet right of station 44+70.00;

Thence along the southerly line of Parcel 27-SH, South 82 degrees, 48 minutes, 51 seconds East, 50.16 feet to a point 45.07 feet right of station 45+20.00;

Thence South 07 degrees, 11 minutes, 09 seconds West, 20.00 feet to a point 65.01 feet right of station 45+18.43;

Thence North 82 degrees, 48 minutes, 51 seconds West, 50.16 feet to a point 61.06 feet right of station 44+68.43;

Thence North 07 degrees, 11 minutes, 09 seconds East, 20.00 feet to the Place of Beginning and containing 0.023 acres of land, subject to easements and restrictions of record.

Bearings are based on the positions of monuments located relative to grid north of the State Plane Coordinate System, Ohio South Zone (3402), NAD83(2011), (EPOCH 2010.0000) by GNSS measurements referenced to the ODOT CORS.

This description was prepared in February 2024 and is based upon surveys performed by Thomas Fok & Associates, Inc. beginning in April 2021 under the direction of Franklin D. Snyder, Jr., P.S. S-7468.