| Ver. Date | 03/12/2024 |  | PID |  |  | 111621 |
| --- | --- | --- | --- | --- | --- | --- |
| PARCEL |  |  |  | 28-T |  |  |
| FAI-158-4.20 |  |  |  |  |  |  |

TEMPORARY EASEMENT FOR THE PURPOSE OF PERFORMING THE WORK NECESSARY TO CONSTRUCT DRIVEWAY FOR MONTHS FROM DATE OF ENTRY BY THE STATE OF OHIO, DEPARTMENT OF TRANSPORTATION

[Surveyor’s description of the premises follows]

Situated in the State of Ohio, County of Fairfield, Township of Greenfield, in the southeast quarter of Section 14 in Township 15, Range XIX of the Congress Lands (East of Scioto River), lying on the left side of the centerline of right of way of Coonpath Road (County Road 31, 60.00 feet wide) and being part of the 0.750 acres of land deeded, now or previously to Alma C. Kiger in instrument number 202100032507 of the Fairfield County Recorder’s Office and known as Fairfield County Auditor’s parcel number 013-00374.00 and further described as follows;

Beginning at the intersection of the existing northerly right of way line of Coonpath Road with the westerly line of the land deeded to Bonnie L. Weaver in official record volume 1796, page 2747 (1/2 interest) and deed volume 445, page 637 (1/2 interest) of the Fairfield County Recorder’s Office, 30.00 feet left of station 45+84.93;

Thence along the existing northerly right of way line of Coonpath Road, North 87 degrees, 19 minutes, 47 seconds West, 44.93 feet to a point 30.00 feet left of station 45+40.00;

Thence North 02 degrees, 40 minutes, 13 seconds East, 15.00 feet to a point 45.00 feet left of station 45+40.00;

Thence South 87 degrees, 19 minutes, 47 seconds East, 44.96 feet to the westerly line of Weaver’s land, 45.00 feet left of station 45+84.96;

Thence along the westerly line of Weaver’s land, South 02 degrees, 48 minutes, 45 seconds West, 15.00 feet to the Place of Beginning and containing 0.015 acres of land, none of which is within the present road occupied by Coonpath Road, subject to easements and restrictions of record.

Bearings are based on the positions of monuments located relative to grid north of the State Plane Coordinate System, Ohio South Zone (3402), NAD83(2011), (EPOCH 2010.0000) by GNSS measurements referenced to the ODOT CORS.

This description was prepared in February 2024 and is based upon surveys performed by Thomas Fok & Associates, Inc. beginning in April 2021 under the direction of Franklin D. Snyder, Jr., P.S. S-7468.