January 16, 2023

Mr. Mark Seger, P.E.

Transportation Engineer

ODOT – District 6

400 East William Street

Delaware, Ohio 43015

Re:  	Airway/Highway Clearance Analysis FRA-270-32.92, PID: 113663

Dear Mr. Seger,

We have reviewed the above mentioned project and have determined that the project will encroach into the (X:1) notification surface for the following conditions:

Project Point Closest to John Glenn International Airport:

Per Section 1407.1.2, using a (100:1) notification surface.

This determination is based on an airport elevation of (A) 809 feet, a project elevation of (B) 865 feet plus a traverse way adjustment of 17 feet, per section 1407.1.3, with a distance of (C) 16,365 feet between the airport and the project locations.   The attached diagram will provide a detailed analysis of the study. The study has also determined that a height of (Z) 90 feet exists between the (X:1) notification surface and the project.  This allows for the 50 foot maximum operating height of a cherry picker for traffic signal work as described in Reference Section 1407-1.5.

Z = 	 C  +  (A-B)

X

X:1 – Notification Slope [100:1 per Section 1404.1.2]

- – Airport Elevation (809 feet)

- – Project Elevation (865 +17 = 882feet)

- – Distance between Project and Heliport (16365 feet)

Z – A positive value of Z is the amount of clearance. (90 feet)

Therefore, based on these heights and the heights from Figure 1407-3, plan note G118B will be required for John Glenn International Airport since the construction equipment does not penetrate the notification surface of a public-use airport runway.

Respectfully,

Abby Cueva, PE

Project Manager

Enclosure: 	Elevation Calculation Attachment