JAC-32-0362 OPUS CP01(A)

IRON PIN & CAP SET

PID: 116023

FILE: 87203250.DAT OP1669117837071

2005   NOTE:  The IGS precise and IGS rapid orbits were not available

2005   at processing time.  The IGS ultra-rapid orbit was/will be used to

2005   process the data.

2005

NGS OPUS-RS SOLUTION REPORT

========================

All computed coordinate accuracies are listed as 1-sigma RMS values.

For additional information: https://gcc02.safelinks.protection.outlook.com/?url=https%3A%2F%2Fwww.ngs.noaa.gov%2FOPUS%2Fabout.jsp%23accuracy&amp;data=05%7C01%7Cjason.robinson%40dot.ohio.gov%7C4069eec6b4f24b140c7708dacc8068b0%7C50f8fcc494d84f0784eb36ed57c7c8a2%7C0%7C0%7C638047149146088056%7CUnknown%7CTWFpbGZsb3d8eyJWIjoiMC4wLjAwMDAiLCJQIjoiV2luMzIiLCJBTiI6Ik1haWwiLCJXVCI6Mn0%3D%7C3000%7C%7C%7C&amp;sdata=KirBfOFaUl1y5d9Uqzx3WDxwuwk9SQExfe8gFfvcoes%3D&amp;reserved=0

USER: jason.robinson@dot.ohio.gov              DATE: November 22, 2022

RINEX FILE: 8720325p.22o                             TIME: 11:54:21 UTC

SOFTWARE: rsgps  1.38 RS291.prl 1.99.3            START: 2022/11/21 15:27:00

EPHEMERIS: igu22371.eph [ultra-rapid]               STOP: 2022/11/21 16:00:45

NAV FILE: brdc3250.22n                         OBS USED:  2736 /  3330   :  82%

ANT NAME: TRMR8           NONE              QUALITY IND.   7.10/  9.87

ARP HEIGHT: 2.000                         NORMALIZED RMS:        0.325

REF FRAME: NAD_83(2011)(EPOCH:2010.0000)              ITRF2014 (EPOCH:2022.88947)

X:       627441.984(m)   0.006(m)            627441.042(m)   0.006(m)

Y:     -4922769.713(m)   0.028(m)          -4922768.293(m)   0.028(m)

Z:      3993508.398(m)   0.013(m)           3993508.320(m)   0.013(m)

LAT:   39  0 44.97635      0.010(m)        39  0 45.00554      0.010(m)

E LON:  277 15 48.92447      0.005(m)       277 15 48.89311      0.005(m)

W LON:   82 44 11.07553      0.005(m)        82 44 11.10689      0.005(m)

EL HGT:          180.457(m)   0.030(m)               179.221(m)   0.030(m)

ORTHO HGT: 	702.58’         214.146(m)   0.035(m) [NAVD88 (Computed using GEOID18)]

UTM COORDINATES    STATE PLANE COORDINATES

UTM (Zone 17)         SPC (3402 OH S)

Northing (Y) [meters]     4319597.256           112427.061	Grid: 368,854.4493	Ground: 368,880.8076

Easting (X)  [meters]      349662.842           579525.172            1,901,325.5020                     1,901,461.3710

Convergence  [degrees]    -1.09325556          -0.15000556

Point Scale                0.99987830           0.99995685

Combined Factor            0.99984999           0.99992854	P.A.F. USED FROM CP01(B) = 1.00007146

US NATIONAL GRID DESIGNATOR: 17SLD4966319597(NAD 83)

BASE STATIONS USED

PID       DESIGNATION                        LATITUDE    LONGITUDE DISTANCE(m)

DI1856 OHHO HOCKING COUNTY CORS ARP        N393207.276 W0822637.876   63301.9

DH9001 WVHU MARSHALL UNIV-HUN CORS ARP     N382522.617 W0822528.918   70839.4

DI1876 OHAD ADAMS COUNTY CORS ARP          N384709.347 W0833322.647   75442.0

DI1854 OHHI HIGHLAND COUNTY CORS ARP       N391200.738 W0833550.494   77332.3

AJ7194 MCON MCCONNELSVILLE CORS ARP        N393939.031 W0814945.121  106297.0

DH9003 WVRA MARSHALL UNIV-RAV CORS ARP     N385628.863 W0814504.842   85726.1

DF4050 GRTN GEORGETOWN CORS ARP            N385134.330 W0835244.711  100521.3

AJ7184 COLB COLUMBUS CORS ARP              N395735.112 W0830244.747  108486.1

DI1858 OHLI LICKING COUNTY CORS ARP        N395709.138 W0822451.031  107990.1

NEAREST NGS PUBLISHED CONTROL POINT

JY0622      B 220                          N390146.000 W0824529.000    2656.2

This position and the above vector components were computed without any knowledge by the National Geodetic Survey regarding the equipment or field operating procedures used.