10/13/23 Meeting w Jeff

- James response on alignments

- Plan Sheets

- Areas

  - Curb ramp at Village Blvd - remove pavement?

  - Perry South - 2ft lawn, 5ft walk, with ditch realignment

    - 6:1 vs 4:1 backslope

  - Perry North - 3ft lawn, 5ft walk, with ditch realignment

    - Drive pipe. Add CB (250+33) and MH (250+57)?

    - Storm sewer doesn’t seem to flow

  - US 42 - 7 ft walk, 2 ft from R/W with 6” deep, 1’ wide ditch

    - Ditch. Prop MH and prop CB

  - Cross slope transition

  - West Ave North - 7ft on EOP, 2ft from parking

  - West Ave 400 - 7 ft from R/W, 6 ft walk, 4 to 5ft from road

    - 4.8 ft grading from R/W

  - South West Ave - 2ft from R/W, then 4ft. 6ft walk width.

    - Profile

      - Cover of 9”

        - Curb ramp

      - May need “underdrains” where damming park runoff. 70-ft segment. But. Where would they outfall

    - 60ft culvert extension

      - Manhole at connection?

    - Grading vs headwall

Notes on NE run of walk

- This run of sidewalk is modeled as three different corridors.

  - 42 baseline is from back of walk 2% to roadway

    - Transition from typical to backpitched from 352+25 to 352+00

  - Radius baseline is at EOP with point control

    - Back of walk point control - linework is in KM file. The profile is “designed” to match existing at 352+00 and meet 1.56% toward parking lot at 351+78

    - Curb ramp joint point control - linework is in KM file. The profile is “designed” to vary from front pitch to back pitch over 20-ft. Profile is a slope offset from back of walk.

- Need to remove parking lot curb at driveway, STA 350+00

- Curve radius of the NE corner is in BK003. Used for sheet production only.

South Profile

- Yellow - Projected profile 4:1 from ditch bottom

- Purple - Projected profile 4% from EOP

- Blue - Projected profile 33” = 2.75’ from bottom of ditch. Cover for pipe.

- Red - Projected profile 1.56% from back of walk.