HUR-61-11.97

PID 107727

CENTERLINE DETERMINATION

For this project I started with the HUR-61-(6.67-8.14)(9.20-12.08) (1957) plans. We found a lot of the reference monuments called for on the centerline plat. The monuments were really hard to find because they only had a small nail in the 10” concrete monument. I did not want to change the centerline alignment, so I did a best fit with the found monuments that were not disturbed. I held the monuments at station 623+58.24 and rotated the centerline plat to the found monument at station 589+35.45. This alignment fit the best with all the other disturbed and good monuments.  We also located three centerline monument boxes on the HUR-61-12.08 (1966) plans. These plans used the same stationing and called the alignment off the older plans by 0.27’ at station 637+00. After I rotated my centerline alignment for the older plans, it actually matched the found centerline monument boxes very well. I believe this is the best centerline alignment for matching the two plans.  If I had to determine the centerline alignment south of station 591+85.45. I would change the delta at this PI to the south.

I used the plan stationing, because it was very close to DESTAPE.

Brett R. Tieben, P.S.

February 27, 2023