Calculations performed by Adam Mellen, P.E.

ODOT District 3 Engineering Department

RIC-545-2.36 (OHWM = 1293.79’)

StreamStats was used at this location to calculate peak flow and monthly flow rates.

The pipe chosen to replace the existing structure has a larger hydraulic opening than the existing structure.

Per StreamStats the largest monthly flow rate is 0.0781 ft3/s.

Standard temporary discharge per L&D Volume 2, Section 1010:

0.0781 ft3/s X 2 = 0.1562 ft3/s

The water surface elevation was determined using the FHWA HY-8 hydraulic modeling software:

The standard temporary discharge does not create a rise in back water greater than the OHWM; therefore, the TAF is acceptable.