ROS-207-04.76 OPUS CP02(B)

IRON PIN & CAP SET

PID: 116074

FILE: 87201731.DAT OP1655981434979

2005   NOTE:  The IGS precise and IGS rapid orbits were not available

2005   at processing time.  The IGS ultra-rapid orbit was/will be used to

2005   process the data.

2005

NGS OPUS-RS SOLUTION REPORT

========================

All computed coordinate accuracies are listed as 1-sigma RMS values.

For additional information: https://gcc02.safelinks.protection.outlook.com/?url=https%3A%2F%2Fwww.ngs.noaa.gov%2FOPUS%2Fabout.jsp%23accuracy&amp;data=05%7C01%7Cjason.robinson%40dot.ohio.gov%7C2981cf3eb9e24dac6cee08da55070a87%7C50f8fcc494d84f0784eb36ed57c7c8a2%7C0%7C0%7C637915786024886304%7CUnknown%7CTWFpbGZsb3d8eyJWIjoiMC4wLjAwMDAiLCJQIjoiV2luMzIiLCJBTiI6Ik1haWwiLCJXVCI6Mn0%3D%7C3000%7C%7C%7C&amp;sdata=n0CBOJObU0skq5z4VloQqYy%2B6%2FCjQH0AEHce9nv0DSA%3D&amp;reserved=0

USER: jason.robinson@dot.ohio.gov              DATE: June 23, 2022

RINEX FILE: 8720173s.22o                             TIME: 10:55:47 UTC

SOFTWARE: rsgps  1.38 RS252.prl 1.99.3            START: 2022/06/22 18:39:45

EPHEMERIS: igu22153.eph [ultra-rapid]               STOP: 2022/06/22 19:10:30

NAV FILE: brdc1730.22n                         OBS USED:  3546 /  4086   :  87%

ANT NAME: TRMR8           NONE              QUALITY IND.   4.56/ 18.69

ARP HEIGHT: 2.000                         NORMALIZED RMS:        0.587

REF FRAME: NAD_83(2011)(EPOCH:2010.0000)              ITRF2014 (EPOCH:2022.47339)

X:       599201.070(m)   0.006(m)            599200.132(m)   0.006(m)

Y:     -4898305.379(m)   0.030(m)          -4898303.965(m)   0.030(m)

Z:      4027549.355(m)   0.026(m)           4027549.278(m)   0.026(m)

LAT:   39 24 29.60464      0.008(m)        39 24 29.63394      0.008(m)

E LON:  276 58 27.26962      0.006(m)       276 58 27.23790      0.006(m)

W LON:   83  1 32.73038      0.006(m)        83  1 32.76210      0.006(m)

EL HGT:          178.659(m)   0.039(m)               177.438(m)   0.039(m)

ORTHO HGT:	696.73’          212.364(m)   0.043(m) [NAVD88 (Computed using GEOID18)]

UTM COORDINATES    STATE PLANE COORDINATES

UTM (Zone 17)         SPC (3402 OH S)

Northing (Y) [meters]     4364036.976           156463.303	Grid: 513,330.0199	Ground: 513,377.2719

Easting (X)  [meters]      325591.320           554721.457            1,819,948.6470                      1,820,116.1730

Convergence  [degrees]    -1.28635833          -0.33360278

Point Scale                0.99997453           0.99993599

Combined Factor            0.99994650           0.99990796	P.A.F. USED FROM CP02(A) = 1.00009205

US NATIONAL GRID DESIGNATOR: 17SLD2559164037(NAD 83)

BASE STATIONS USED

PID       DESIGNATION                        LATITUDE    LONGITUDE DISTANCE(m)

DI1856 OHHO HOCKING COUNTY CORS ARP        N393207.276 W0822637.876   52024.4

AI4945 STKR STOCKER OHIO UNIV CORS ARP     N391933.824 W0820625.629   79690.6

DI1854 OHHI HIGHLAND COUNTY CORS ARP       N391200.738 W0833550.494   54445.9

DI1686 OHUN UNION COUNTY CORS ARP          N401358.849 W0832139.074   95968.7

DI1858 OHLI LICKING COUNTY CORS ARP        N395709.138 W0822451.031   80034.8

DF4048 GALP GALLIPOLIS CORS ARP            N385039.148 W0821640.091   90022.7

DI1876 OHAD ADAMS COUNTY CORS ARP          N384709.347 W0833322.647   82942.5

AJ7192 LEBA LEBANON CORS ARP               N392549.788 W0841659.283  108305.6

DL6453 ACSO ALUMCREEK_OH2006 CORS ARP      N401356.469 W0825853.489   91584.7

NEAREST NGS PUBLISHED CONTROL POINT

JY1589      ROSSPORT                       N392619.591 W0830134.529    3392.2

This position and the above vector components were computed without any knowledge by the National Geodetic Survey regarding the equipment or field operating procedures used.