ROS-207-04.76 OPUS CP02(C)

IRON PIN & CAP SET

PID: 116074

FILE: 87201732.DAT OP1655981463051

2005   NOTE:  The IGS precise and IGS rapid orbits were not available

2005   at processing time.  The IGS ultra-rapid orbit was/will be used to

2005   process the data.

2005

6023   **************   WARNING   ***********************

6023   The Quality Indicator for the network solution

6023   is less than 3.0. This is often a warning sign that

6023   the network solution was weak and tropospheric and

6023   ionospheric refraction in the project area were not strongly

6023   determined.

6023   **************   WARNING   ***********************

6024

6024   **************   WARNING   ***********************

6024   The Network Quality Indicator for the rover  solution

6024   is less than 1.0. This is often a warning sign that

6024   one or more of the baselines involving your station were

6024   weakly determined. You should check other quality indicators,

6024   such as standard errors. If possible, corroborate

6024   this position with a solution involving a different time period.

6024   **************   WARNING   ***********************

6024

6030   **************   WARNING   ***********************

6030   One or both of the standard deviations associated with

6030   horizontal coordinates is greater than 5 cm, and/or the

6030   standard deviation associated with the vertical coordinate

6030   is greater than 10 cm. This means that the vectors used to

6030   determine your position did not agree as well as expected.

6030   Often this is the result of problems with the adopted coordinates

6030   at one or more of the reference stations selected by OPUS-RS.

6030   If a problem reference station can be identified, it can

6030   be excluded with the Exclude feature on the OPUS Options

6030   page.

6030

NGS OPUS-RS SOLUTION REPORT

========================

All computed coordinate accuracies are listed as 1-sigma RMS values.

For additional information: https://gcc02.safelinks.protection.outlook.com/?url=https%3A%2F%2Fwww.ngs.noaa.gov%2FOPUS%2Fabout.jsp%23accuracy&amp;data=05%7C01%7Cjason.robinson%40dot.ohio.gov%7C833f424e3a464af8a0b908da55074585%7C50f8fcc494d84f0784eb36ed57c7c8a2%7C0%7C0%7C637915787009556837%7CUnknown%7CTWFpbGZsb3d8eyJWIjoiMC4wLjAwMDAiLCJQIjoiV2luMzIiLCJBTiI6Ik1haWwiLCJXVCI6Mn0%3D%7C3000%7C%7C%7C&amp;sdata=9vQkZZvbFb2VcKt0o7EXbTHOJtO5%2BBKqeSrGJRWiVeQ%3D&amp;reserved=0

USER: jason.robinson@dot.ohio.gov              DATE: June 23, 2022

RINEX FILE: 8720173t.22o                             TIME: 10:57:35 UTC

SOFTWARE: rsgps  1.38 RS272.prl 1.99.3            START: 2022/06/22 19:11:45

EPHEMERIS: igu22153.eph [ultra-rapid]               STOP: 2022/06/22 19:44:30

NAV FILE: brdc1730.22n                         OBS USED:  2870 /  3297   :  87%

ANT NAME: TRMR8           NONE              QUALITY IND.   0.73/  0.04

ARP HEIGHT: 2.000                         NORMALIZED RMS:        0.702

REF FRAME: NAD_83(2011)(EPOCH:2010.0000)              ITRF2014 (EPOCH:2022.47346)

X:       599201.085(m)   0.090(m)            599200.147(m)   0.090(m)

Y:     -4898305.361(m)   0.099(m)          -4898303.947(m)   0.099(m)

Z:      4027549.345(m)   0.039(m)           4027549.268(m)   0.039(m)

LAT:   39 24 29.60474      0.076(m)        39 24 29.63405      0.076(m)

E LON:  276 58 27.27034      0.079(m)       276 58 27.23858      0.079(m)

W LON:   83  1 32.72966      0.079(m)        83  1 32.76142      0.079(m)

EL HGT:          178.640(m)   0.087(m)               177.419(m)   0.087(m)

ORTHO HGT: 	696.67’         212.345(m)   0.088(m) [NAVD88 (Computed using GEOID18)]

UTM COORDINATES    STATE PLANE COORDINATES

UTM (Zone 17)         SPC (3402 OH S)

Northing (Y) [meters]     4364036.979           156463.306	Grid: 513,330.0297	Ground: 513,377.2817

Easting (X)  [meters]      325591.337           554721.475            1,819,948.7060                      1,820,116.2320

Convergence  [degrees]    -1.28635833          -0.33360278

Point Scale                0.99997453           0.99993599

Combined Factor            0.99994650           0.99990797	P.A.F. USED FROM CP02(A) = 1.00009205

US NATIONAL GRID DESIGNATOR: 17SLD2559164037(NAD 83)

BASE STATIONS USED

PID       DESIGNATION                        LATITUDE    LONGITUDE DISTANCE(m)

DI1856 OHHO HOCKING COUNTY CORS ARP        N393207.276 W0822637.876   52024.4

AI4945 STKR STOCKER OHIO UNIV CORS ARP     N391933.824 W0820625.629   79690.6

DI1854 OHHI HIGHLAND COUNTY CORS ARP       N391200.738 W0833550.494   54445.9

DI1686 OHUN UNION COUNTY CORS ARP          N401358.849 W0832139.074   95968.7

DF4048 GALP GALLIPOLIS CORS ARP            N385039.148 W0821640.091   90022.7

DI1876 OHAD ADAMS COUNTY CORS ARP          N384709.347 W0833322.647   82942.5

AJ7192 LEBA LEBANON CORS ARP               N392549.788 W0841659.283  108305.6

NEAREST NGS PUBLISHED CONTROL POINT

JY1589      ROSSPORT                       N392619.591 W0830134.529    3392.2

This position and the above vector components were computed without any knowledge by the National Geodetic Survey regarding the equipment or field operating procedures used.