PID 109586 SEN-23-4.88

Ex. Structure: Concrete slab, widened on reinf. Concrete substructure, extended.

Span: 11’ x approx. 7’ rise

Roadway: 40’ f/f guardrail

Skew: 18 deg. L.F.

Date Built: 1925

Reconstructed/Extended: 1955

Over Wolf Creek Branch

Clear Zone Width (60 mph Design Speed, 3900 Design Year ADT, 6:1 or Flatter Foreslope: 28’

25-year design frequency

Flood Insurance Zone: No

25 Year Peak Flood: 167 cfs

100 Year Peak Flood: 210 cfs

Approximate existing hydraulic opening: 67.38 square feet