| TO: | Thomas Powell, P.E., District 4 Geotechnical Engineer |
| --- | --- |
| FROM: | Amal Mohi, P.E., Office of Geotechnical Engineering |
| DATE: | March 27, 2026 |
| SUBJECT: | TRU-5-10.75 Noise Abatement, PID 114708, Foundation Design Memo |

# Introduction

The Office of Geotechnical Engineering (OGE) has completed foundation design for the noise barrier along SR5/SR82. We understand that a single noise barrier wall is to be constructed from Station 559+51.03 to 580+87.03 along the SR5/SR82 alignment (Station 30+00.00 to 51+36.00 of the wall alignment). An 81-inch concrete barrier for noise abatement is also to be constructed along the right shoulder of SR5/SR82 from Station 547+67.62 to 558+46.87; no geotechnical design is necessary for the barrier shape.

# Exploration and Findings

The ODOT Office of Geotechnical Engineering (OGE) performed a geotechnical exploration consisting of nineteen (19) project borings between the dates of April 29 and June 03, 2025, with thirteen (13) of those borings being along the alignment of the noise barrier wall, and the remaining six (6) borings being along the alignment of the 81-inch concrete barrier. The borings along the noise barrier wall alignment (B-007-0-25 through B-019-0-25) generally revealed stiff to hard cohesive soils (A-4a, A-4b, A-6a, A-6b, and A-7-6) immediately below the surface materials (topsoil or aggregate base), overlying deeper very loose to very dense granular soils (A-1-b, A-2-4, A-3a, A-4a, and A-4b). Borings on the west end of the noise barrier wall alignment (B-007-0-25 through B-013-0-25) generally encountered more granular soils, while borings on the east end of the alignment (B-014-0-25 through B-019-0-25) generally encountered more cohesive soils. Borings B-007-0-25 through B-013-0-25 encountered bedrock at a depth ranging from 13.5 to 23.5 feet (elevation 879.8 to 886.4). Bedrock was primarily shale (dark gray to black, moderately to highly weathered, very weak to slightly strong) with siltstone being encountered in boring B-013-0-25 (gray, moderately weathered, slightly strong).

# Analysis and Recommendations

Drilled Shaft Foundation Analyses

Per the revised Stage 3 Plan Set, the proposed noise barrier wall has 88 panels and 89 posts supported by drilled shaft foundations. Standard noise barrier drilled shaft foundations, in accordance with Standard Construction Drawing NBS-1-09, of from 6.0 to 16.0 feet deep can be used to support the proposed noise barrier wall. OGE designed the drilled shaft foundations in accordance with Geotechnical Design Manual (GDM) Section 1600. The following table presents a summary of the drilled shaft designs.

| POST | Boring | WALL | TOP | BOTTOM | HEIGHT | SHAFT | Top Shaft |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NO. | ID | STATION | WALL (FT) | WALL (FT) | WALL (FT) | LENGTH (FT) | Elev. |
| 1 | B-007-0-25 | 30+00.00 | 906.50 | 898.00 | 8.5 | 16.0 | 897.55 |
| 2 | B-007-0-25 | 30+24.00 | 910.50 | 895.00 | 15.5 | 16.0 | 894.55 |
| 3 | B-007-0-25 | 30+48.00 | 914.50 | 896.00 | 18.5 | 16.0 | 895.55 |
| 4 | B-008-0-25 | 30+72.00 | 914.50 | 896.00 | 18.5 | 11.0 | 895.55 |
| 5 | B-008-0-25 | 30+96.00 | 914.50 | 896.00 | 18.5 | 11.0 | 895.55 |
| 6 | B-008-0-25 | 31+20.00 | 915.50 | 896.00 | 19.5 | 11.0 | 895.55 |
| 7 | B-008-0-25 | 31+44.00 | 915.50 | 897.50 | 18.0 | 11.0 | 897.05 |
| 8 | B-008-0-25 | 31+68.00 | 916.50 | 898.00 | 18.5 | 11.0 | 897.55 |
| 9 | B-008-0-25 | 31+92.00 | 916.50 | 898.50 | 18.0 | 11.0 | 898.05 |
| 10 | B-008-0-25 | 32+16.00 | 917.00 | 899.00 | 18.0 | 11.0 | 898.55 |
| 11 | B-008-0-25 | 32+40.00 | 917.00 | 899.50 | 17.5 | 11.0 | 899.05 |
| 12 | B-009-0-25 | 32+64.00 | 918.00 | 899.50 | 18.5 | 11.0 | 899.05 |
| 13 | B-009-0-25 | 32+88.00 | 918.00 | 900.00 | 18.0 | 11.0 | 899.55 |
| 14 | B-009-0-25 | 33+12.00 | 918.00 | 900.00 | 18.0 | 11.0 | 899.55 |
| 15 | B-009-0-25 | 33+36.00 | 918.00 | 900.00 | 18.0 | 11.0 | 899.55 |
| 16 | B-009-0-25 | 33+60.00 | 918.00 | 900.00 | 18.0 | 11.0 | 899.55 |
| 17 | B-009-0-25 | 33+84.00 | 918.00 | 899.50 | 18.5 | 11.0 | 899.05 |
| 18 | B-009-0-25 | 34+08.00 | 918.00 | 899.50 | 18.5 | 11.0 | 899.05 |
| 19 | B-009-0-25 | 34+32.00 | 918.00 | 899.50 | 18.5 | 11.0 | 899.05 |
| 20 | B-010-0-25 | 34+56.00 | 918.00 | 899.50 | 18.5 | 11.0 | 899.05 |
| 21 | B-010-0-25 | 34+80.00 | 918.00 | 900.00 | 18.0 | 11.0 | 899.55 |
| 22 | B-010-0-25 | 35+04.00 | 918.00 | 900.00 | 18.0 | 11.0 | 899.55 |
| 23 | B-010-0-25 | 35+28.00 | 918.00 | 900.00 | 18.0 | 11.0 | 899.55 |
| 24 | B-010-0-25 | 35+52.00 | 918.00 | 900.00 | 18.0 | 11.0 | 899.55 |
| 25 | B-010-0-25 | 35+76.00 | 918.50 | 900.50 | 18.0 | 11.0 | 900.05 |
| 26 | B-010-0-25 | 36+00.00 | 918.50 | 900.50 | 18.0 | 11.0 | 900.05 |
| 27 | B-010-0-25 | 36+24.00 | 918.50 | 900.50 | 18.0 | 11.0 | 900.05 |
| 28 | B-011-0-25 | 36+48.00 | 918.50 | 900.50 | 18.0 | 11.0 | 900.05 |
| 29 | B-011-0-25 | 36+72.00 | 919.00 | 900.50 | 18.5 | 11.0 | 900.05 |
| 30 | B-011-0-25 | 36+96.00 | 919.00 | 901.00 | 18.0 | 11.0 | 900.55 |
| 31 | B-011-0-25 | 37+20.00 | 919.00 | 901.00 | 18.0 | 11.0 | 900.55 |
| 32 | B-011-0-25 | 37+44.00 | 919.00 | 901.00 | 18.0 | 11.0 | 900.55 |
| 33 | B-011-0-25 | 37+68.00 | 919.00 | 901.00 | 18.0 | 11.0 | 900.55 |
| 34 | B-011-0-25 | 37+92.00 | 919.50 | 901.50 | 18.0 | 11.0 | 901.05 |
| 35 | B-011-0-25 | 38+16.00 | 919.50 | 901.50 | 18.0 | 11.0 | 901.05 |
| 36 | B-011-0-25 | 38+40.00 | 919.50 | 901.50 | 18.0 | 11.0 | 901.05 |
| 37 | B-012-0-25 | 38+64.00 | 920.00 | 901.50 | 18.5 | 11.0 | 901.05 |
| 38 | B-012-0-25 | 38+88.00 | 920.00 | 902.00 | 18.0 | 11.0 | 901.55 |
| 39 | B-012-0-25 | 39+12.00 | 920.00 | 902.00 | 18.0 | 11.0 | 901.55 |
| 40 | B-012-0-25 | 39+36.00 | 920.00 | 902.00 | 18.0 | 11.0 | 901.55 |
| 41 | B-012-0-25 | 39+60.00 | 919.50 | 901.50 | 18.0 | 11.0 | 901.05 |
| 42 | B-012-0-25 | 39+84.00 | 919.50 | 901.50 | 18.0 | 11.0 | 901.05 |
| 43 | B-012-0-25 | 40+08.00 | 919.50 | 901.50 | 18.0 | 11.0 | 901.05 |
| 44 | B-012-0-25 | 40+32.00 | 919.50 | 901.50 | 18.0 | 11.0 | 901.05 |
| 45 | B-013-0-25 | 40+56.00 | 919.50 | 901.50 | 18.0 | 11.0 | 901.05 |
| 46 | B-013-0-25 | 40+80.00 | 919.50 | 901.50 | 18.0 | 11.0 | 901.05 |
| 47 | B-013-0-25 | 41+04.00 | 919.50 | 902.00 | 17.5 | 11.0 | 901.55 |
| 48 | B-013-0-25 | 41+28.00 | 920.50 | 902.00 | 18.5 | 11.0 | 901.55 |
| 49 | B-013-0-25 | 41+52.00 | 920.50 | 902.00 | 18.5 | 11.0 | 901.55 |
| 50 | B-013-0-25 | 41+76.00 | 921.00 | 902.50 | 18.5 | 11.0 | 902.05 |
| 51 | B-013-0-25 | 42+00.00 | 921.00 | 903.00 | 18.0 | 11.0 | 902.55 |
| 52 | B-013-0-25 | 42+24.00 | 921.00 | 903.00 | 18.0 | 11.0 | 902.55 |
| 53 | B-014-0-25 | 42+48.00 | 921.00 | 903.50 | 17.5 | 6.0 | 903.05 |
| 54 | B-014-0-25 | 42+72.00 | 921.00 | 903.50 | 17.5 | 6.0 | 903.05 |
| 55 | B-014-0-25 | 42+96.00 | 921.00 | 903.50 | 17.5 | 6.0 | 903.05 |
| 56 | B-014-0-25 | 43+20.00 | 922.00 | 903.50 | 18.5 | 6.0 | 903.05 |
| 57 | B-014-0-25 | 43+44.00 | 922.00 | 903.50 | 18.5 | 6.0 | 903.05 |
| 58 | B-014-0-25 | 43+68.00 | 922.00 | 904.00 | 18.0 | 6.0 | 903.55 |
| 59 | B-014-0-25 | 43+92.00 | 922.00 | 904.00 | 18.0 | 6.0 | 903.55 |
| 60 | B-014-0-25 | 44+16.00 | 922.00 | 904.00 | 18.0 | 6.0 | 903.55 |
| 61 | B-014-0-25 | 44+40.00 | 923.00 | 904.50 | 18.5 | 6.0 | 904.05 |
| 62 | B-015-0-25 | 44+64.00 | 923.00 | 905.00 | 18.0 | 11.0 | 904.55 |
| 63 | B-015-0-25 | 44+88.00 | 923.00 | 905.00 | 18.0 | 11.0 | 904.55 |
| 64 | B-015-0-25 | 45+12.00 | 924.00 | 905.50 | 18.5 | 11.0 | 905.05 |
| 65 | B-015-0-25 | 45+36.00 | 924.00 | 905.50 | 18.5 | 11.0 | 905.05 |
| 66 | B-015-0-25 | 45+60.00 | 924.00 | 906.00 | 18.0 | 11.0 | 905.55 |
| 67 | B-015-0-25 | 45+84.00 | 924.00 | 906.50 | 17.5 | 11.0 | 906.05 |
| 68 | B-015-0-25 | 46+08.00 | 924.00 | 906.50 | 17.5 | 11.0 | 906.05 |
| 69 | B-015-0-25 | 46+32.00 | 924.00 | 906.50 | 17.5 | 11.0 | 906.05 |
| 70 | B-016-0-25 | 46+56.00 | 924.00 | 906.50 | 17.5 | 11.0 | 906.05 |
| 71 | B-016-0-25 | 46+80.00 | 924.00 | 906.50 | 17.5 | 11.0 | 906.05 |
| 72 | B-016-0-25 | 47+04.00 | 924.00 | 906.50 | 17.5 | 11.0 | 906.05 |
| 73 | B-016-0-25 | 47+28.00 | 924.00 | 906.50 | 17.5 | 11.0 | 906.05 |
| 74 | B-016-0-25 | 47+52.00 | 924.00 | 906.50 | 17.5 | 11.0 | 906.05 |
| 75 | B-016-0-25 | 47+76.00 | 924.50 | 906.50 | 18.0 | 11.0 | 906.05 |
| 76 | B-016-0-25 | 48+00.00 | 924.50 | 906.50 | 18.0 | 11.0 | 906.05 |
| 77 | B-016-0-25 | 48+24.00 | 924.50 | 906.50 | 18.0 | 11.0 | 906.05 |
| 78 | B-017-0-25 | 48+48.00 | 924.50 | 906.50 | 18.0 | 6.0 | 906.05 |
| 79 | B-017-0-25 | 48+72.00 | 924.50 | 906.50 | 18.0 | 6.0 | 906.05 |
| 80 | B-017-0-25 | 48+96.00 | 924.50 | 906.00 | 18.5 | 6.0 | 905.55 |
| 81 | B-017-0-25 | 49+20.00 | 924.50 | 906.00 | 18.5 | 6.0 | 905.55 |
| 82 | B-017-0-25 | 49+44.00 | 924.50 | 906.50 | 18.0 | 6.0 | 906.05 |
| 83 | B-017-0-25 | 49+68.00 | 925.50 | 906.50 | 19.0 | 6.0 | 906.05 |
| 84 | B-017-0-25 | 49+92.00 | 925.50 | 906.50 | 19.0 | 6.0 | 906.05 |
| 85 | B-017-0-25 | 50+16.00 | 925.50 | 907.00 | 18.5 | 6.0 | 906.55 |
| 86 | B-017-0-25 | 50+40.00 | 925.50 | 907.00 | 18.5 | 6.0 | 906.55 |
| 87 | B-018-0-25 | 50+64.00 | 921.50 | 907.50 | 14.0 | 6.0 | 907.05 |
| 88 | B-018-0-25 | 50+88.00 | 917.50 | 907.50 | 10.0 | 6.0 | 907.05 |
| 89 | B-018-0-25 | 51+12.00 | 917.50 | 907.50 | 10.0 | 6.0 | 907.05 |

The top of wall and bottom of wall elevations shown in the Profile Views (Sheets 22 through 26) differ from those listed in the table “Noise Barrier Foundation Data – Wall 1” on Sheet 19. We used the elevations shown in the Profile Views (Sheets 22 through 26) for our design. In addition, the wall stations, post spacing, and wall heights are different. We did not check the SR 5 stations, as they are not relevant to the design. Furthermore, the table shows 92 posts, whereas the Profile Views indicate 89 posts.

In addition, the top of drilled shaft elevations are a required component of the Plan Set and are currently missing. These will need to be shown. For our calculations, we assumed Type A 20‑inch precast concrete posts and therefore used 2 inches for the plate thickness (PT) and a gap of 2.5 × (1⅜). This results in top of drilled shaft elevations that are 5.44 inches lower than the bottom of wall elevation.

Attached is a spreadsheet, “2026-3-27_TRU-5-10.75 Noise Abatement, PID 114708, Noise Barrier Alignment.xlsx,” with a summary of the noise barrier foundation design. Calculations are available upon request.

If you have any questions, please feel free to contact either myself at 614-275-1308 or Alex Dettloff at 614-275-1308.

Thank you,

AM

PC: Reading File, File

Attachment:

Spreadsheet “2026-3-27_TRU-5-10.75 Noise Abatement, PID 114708, Noise Barrier Alignment.xlsx”