ITEM 519 - PATCHING CONCRETE STRUCTURE, AS PER PLAN

A QUANTITY OF 670 SQUARE FEET HAS BEEN CARRIED TO THE GENERAL SUMMARY FOR CONCRETE PATCHING ON THE FORWARD AND REAR ABUTMENTS, AS MARKED IN THE FIELD, PATCHING AREAS SHALL BE CONFIRMED BY THE CONTRACTOR BY SOUNDING THE CONCRETE.

THIS WORK SHALL CONSIST OF REMOVING ALL LOOSE AND DISINTEGRATED CONCRETE; PREPARING THE SURFACE; FURNISHING AND PLACING REINFORCING STEEL; PLACING FORMS; INSTALLING EMBEDDED GALVANIC ANODES AND PLACING CONCRETE PATCHES, INCLUDING CURING OF SAME. PERFORM WORK ACCORDING TO C&MS 519.

ALL LABOR, EQUIPMENT AND MATERIALS NECESSARY TO COMPLETE THIS WORK SHALL BE INCLUDED WITH THE UNIT PRICE BID FOR ITEM 519 - PATCHING CONCRETE STRUCTURE, AS PER PLAN

ITEM 511 - CLASS QC1 CONCRETE, ABUTMENT, AS PER PLAN

THIS WORK SHALL CONSIST OF CLEANING AND PREPARATION OF THE WEEP HOLES AND FILLING WITH CLASS QC1 CONCRETE, ENSURE THAT THE SPACE TO BE OCCUPIED BY THE CONCRETE IS FREE OF LAITANCE, SILT, DIRT, SHAVINGS, SAWDUST, LOOSE AND BUILT-UP RUST, AND OTHER DEBRIS. THE PLACEMENT OF THE CONCRETE AND SURFACE PREPRATION SHALL CONFORM TO THE REQUIREMENTS OF CM&S 511.  PLACE TYPE 2 WATERPROOFING BEHIND EACH HOLE EXTENDING A MINIMUM OF 1.5’ FROM THE CENTER OF THE WEEP HOLE.

ALL LABOR, EQUIPMENT AND MATERIALS NECESSARY TO COMPLETE THIS WORK SHALL BE INCLUDED WITH THE UNIT PRICE BID FOR ITEM 511 - CLASS QC1 CONCRETE, ABUTMENT, AS PER PLAN