WAY-250-27.5 & WAY-250-28.64

PID 109030

For these two projects I started out by drawing the centerline from the Wooster-Dover-Road S.H. 414 SEC. D (1943) plans. We did not find any of the R/W swing-tie monuments called for in these plans. We then located some boundary monuments in Mt. Eaton near the old corporation line in 1943. We also tied into some boundary monuments near the WAY-250-27.5 project. We next went to the Stark County line and found a monument box in the centerline of WAY-250 near the county line. I held 30 feet off the monuments found in Mt. Eaton and rotated the centerline alignment to the found monument box at the Stark County line. The alignment hit really well with the boundary monuments near WAY-250-27.5, and with the current centerline of pavement throughout the entire centerline plan.

For stationing I started at the old Mt. Eaton corporation line shown on the (1943) R/W plans. I established the R/W from this stationing. I then changed the stationing using the current Mt. Eaton corporation line and the Log Point from Destape. I used the boundary surveys to establish the current corporation line, which is also a section line.

Brett R. Tieben P.S.

03/12/2021

## Horizontal Alignment Review Report

Report Created:  Thursday, March 11, 2021
Time:  2:22:01 PM

| Project: | Design |  |
| --- | --- | --- |
| Description: |  |  |
| File Name: | c:\users\btieben\appdata\local\bentley\projectwise\workingdir\ohiodot-pw.bentley.com_ohiodot-pw-02\brett.tieben@dot.ohio.gov\dms23026\WAY-250-27.5-109030_BR001.dgn |  |
| Last Revised: | 3/11/2021 14:18:12 |  |
|  | Note: All units in this report are in feet unless specified otherwise. |  |

| Alignment Name: |  | CLX_250 |  |  |
| --- | --- | --- | --- | --- |
| Alignment Description: |  |  |  |  |
| Alignment Style: |  | Alignment\GE_X_Alignment |  |  |
|  |  | Station | Northing | Easting |
| Element: Linear |  |  |  |  |
| POT | ( ) | 1434+52.94 R1 | 374985.768 | 2190958.654 |
| PC | ( ) | 1473+79.22 R1 | 373312.015 | 2194510.305 |
| Tangential Direction: |  | S64°46'02.441"E |  |  |
| Tangential Length: |  | 3926.280 |  |  |
| Element: Circular |  |  |  |  |
| PC | ( ) | 1473+79.22 R1 | 373312.015 | 2194510.305 |
| HPI | ( ) | 1478+54.34 R1 | 373109.473 | 2194940.092 |
| CC | ( ) |  | 357763.352 | 2187182.833 |
| PT | ( ) | 1483+29.22 R1 | 372883.499 | 2195358.034 |
| Radius: |  | 17188.740 |  |  |
| Delta: |  | 03°10'00.000" | Right |  |
| Degree of Curvature (Arc): |  | 00°20'00.000" |  |  |
| Length: |  | 950.000 |  |  |
| Tangent: |  | 475.121 |  |  |
| Chord: |  | 949.879 |  |  |
| Middle Ordinate: |  | 6.563 |  |  |
| External: |  | 6.565 |  |  |
| Tangent Direction: |  | S64°46'02.441"E |  |  |
| Radial Direction: |  | S25°13'57.559"W |  |  |
| Chord Direction: |  | S63°11'02.441"E |  |  |
| Radial Direction: |  | S28°23'57.559"W |  |  |
| Tangent Direction: |  | S61°36'02.441"E |  |  |
| Element: Linear |  |  |  |  |
| PT | ( ) | 1483+29.22 R1 | 372883.499 | 2195358.034 |
| PC | ( ) | 1507+82.64 R1 | 371716.619 | 2197516.196 |
| Tangential Direction: |  | S61°36'02.441"E |  |  |
| Tangential Length: |  | 2453.420 |  |  |
| Element: Circular |  |  |  |  |
| PC | ( ) | 1507+82.64 R1 | 371716.619 | 2197516.196 |
| HPI | ( ) | 1515+93.68 R1 | 371330.879 | 2198229.627 |
| CC | ( ) |  | 368356.586 | 2195699.484 |
| PT | ( ) | 1523+80.97 R1 | 370688.526 | 2198724.764 |
| Radius: |  | 3819.720 |  |  |
| Delta: |  | 23°58'30.000" | Right |  |
| Degree of Curvature (Arc): |  | 01°29'59.998" |  |  |
| Length: |  | 1598.334 |  |  |
| Tangent: |  | 811.036 |  |  |
| Chord: |  | 1586.699 |  |  |
| Middle Ordinate: |  | 83.297 |  |  |
| External: |  | 85.154 |  |  |
| Tangent Direction: |  | S61°36'02.441"E |  |  |
| Radial Direction: |  | S28°23'57.559"W |  |  |
| Chord Direction: |  | S49°36'47.441"E |  |  |
| Radial Direction: |  | S52°22'27.559"W |  |  |
| Tangent Direction: |  | S37°37'32.441"E |  |  |
| Element: Linear |  |  |  |  |
| PT | ( ) | 1523+80.97 R1 | 370688.526 | 2198724.764 |
| PC | ( ) | 1534+56.76 R1 | 369836.483 | 2199381.534 |
| Tangential Direction: |  | S37°37'32.441"E |  |  |
| Tangential Length: |  | 1075.790 |  |  |
| Element: Circular |  |  |  |  |
| PC | ( ) | 1534+56.76 R1 | 369836.483 | 2199381.534 |
| HPI | ( ) | 1540+77.70 R1 | 369344.688 | 2199760.618 |
| CC | ( ) |  | 372168.423 | 2202406.814 |
| PT | ( ) | 1546+87.88 R1 | 368998.293 | 2200275.960 |
| Radius: |  | 3819.720 |  |  |
| Delta: |  | 18°28'00.000" | Left |  |
| Degree of Curvature (Arc): |  | 01°29'59.998" |  |  |
| Length: |  | 1231.112 |  |  |
| Tangent: |  | 620.940 |  |  |
| Chord: |  | 1225.790 |  |  |
| Middle Ordinate: |  | 49.492 |  |  |
| External: |  | 50.141 |  |  |
| Tangent Direction: |  | S37°37'32.441"E |  |  |
| Radial Direction: |  | S52°22'27.559"W |  |  |
| Chord Direction: |  | S46°51'32.441"E |  |  |
| Radial Direction: |  | S33°54'27.559"W |  |  |
| Tangent Direction: |  | S56°05'32.441"E |  |  |
| Element: Linear |  |  |  |  |
| PT | ( ) | 1546+87.88 R1 | 368998.293 | 2200275.960 |
| POT | ( ) | 1601+47.00 R1 | 365952.890 | 2204806.690 |
| Tangential Direction: |  | S56°05'32.441"E |  |  |
| Tangential Length: |  | 5459.120 |  |  |