# INTERSECTION # SAFETY STUDY

LIC SR 16 0.20 Broad Street (SR 16) and Taylor Road (TR 169) Study PID #119168 City of Pataskala, Licking County ODOT District 5 2021 ODOT SA – Urban Intersection Rank #131 ODOT District 5 – Urban Intersction Rank #2 Safety Annual Work Program 2024 Prepared for: Ohio Department of Transportation (ODOT) District 5 9600 Jacksontown Road Jacksontown, OH 43030 City of Pataskala - Dept. of Public Services 621 W. Broad Street, Pataskala, OH 43062 City of Reynoldsburg - Engineering Dept. 7232 E. Main Street, Reynoldsburg, OH 43068

November 1, 2023 Prepared by: Singh & Associates, Inc. 875 N. High Street, Suite 300 Columbus, OH 43215 Crawford, Murphy & Tilly, Inc. 8101 N. High Street, Suite 150 Columbus, OH 43235

Broad Street (SR 16)

Taylor Road (TR 169)

# Table of Contents

1.0	Table of Contents	2

2.0	One Page Project Summary	4

3.0	Executive Summary	5

4.0	Purpose and Need Statement	6

5.0	Existing Conditions	7

5.1	Intersection Conditions	7

5.2	Eastbound Lane Drop into Right Turn Lane	8

5.3	Eastbound Left Turn Lane Sight Distance	9

5.4	Commercial Driveway Access in the Northeast and Southeast Quadrants	9

5.5	Pedestrian Accommodations	11

6.0	Crash Data and Analysis	12

6.1	Benefit Cost Analysis	14

7.0	Recommendations and Prioritization	16

7.1	Short-Term Countermeasures	16

7.2	Mid-Term Countermeasures	16

7.3	Long-Term Countermeasures (Proposed Recommended Improvements)	17

7.4	Proposed Future Improvements	17

8.0	Summary of Supplemental Traffic Studies	19

8.1	Data Collection	19

8.2	Capacity Analysis	19

List of Figures

FIGURE 1 - LOCATION MAP	6

FIGURE 2 - STUDY AREA	7

FIGURE 3 - EASTBOUND BROAD ST (SR 16) APPROACH	9

FIGURE 4 - SOUTHEAST QUADRANT DRIVEWAYS (CIRCLE K GAS STATION)	10

FIGURE 5 - NORTHEAST QUADRANT DRIVEWAYS (DAIRY QUEEN & LICKING LINE PLAZA)	11

FIGURE 6 - SOUTH LEG CROSSWALK	11

FIGURE 7 - CRASH DIAGRAM	12

FIGURE 8 - FREQUENCY OF CRASHES BY TYPE OF CRASH	13

FIGURE 9 - 2029 FORECASTED TRAFFIC VOLUMES	19

Appendices

APPENDIX A	Existing Conditions Diagram

APPENDIX B 	Existing Traffic Data

APPENDIX C	Future Traffic Volumes

APPENDIX D 	Crash Diagram

APPENDIX E	Crash Summary (CAM Tool Output)

APPENDIX F	Capacity Analyses

APPENDIX G	Concept Plans

APPENDIX H	Cost Estimates

APPENDIX I	Benefit-Cost (ECAT) Analysis

# One Page Project Summary

LIC SR 16 0.20:  Broad Street (SR 16) and Taylor Road (TR 169), City of Pataskala, Licking County, District 5

Key Proposed Countermeasures

- Geometric Improvements. Extend two EB through lanes past Taylor Rd (TR 169) to Annette Dr. Install EB Broad St (SR 16) right turn lane. Install offset left turn lanes on both Broad St (SR 16) approaches.

- Traffic Signal Improvements. Install right turn overlap phase and 5-section signal heads for EB Broad St (SR 16) right turn lane. Install pedestrian accommodations including marked crosswalks, pedestrian signal heads, push buttons, and ADA ramps on the north, east, and south legs.

- Driveway Restrictions. Close north driveway access to Circle K on NB Taylor Rd (TR 169) and west driveway access to Licking Line Plaza on WB Broad St (SR 16).

Project Priority

- This is the #2 ranked urban intersection in the 2021 HSIP Priority Locations list within District 5 and ranked as the #131 urban intersection statewide.

Crash Diagram

EB Approach

Queuing

Location Map

# Executive Summary

The purpose of this study is to evaluate the existing safety performance and to identify potential countermeasures to reduce serious injury and overall traffic crashes at the intersection of Broad Street (SR 16) and Taylor Road (TR 169) in the City of Pataskala, Licking County, Ohio. This intersection serves as the western gateway into both the City of Pataskala and Licking County.  Within District 5, this was the #2 ranked urban intersection in the 2021 HSIP Priority Locations list and was the #131 ranked urban intersection statewide.

There were no fatalities and one serious injury crash reported within the five-year study period. Rear End (50, 40.98%), Left Turn (28, 22.95%), and Angle (26, 21.31%) were the top three crash types. These three crash types account for 85.25% (104 of 122) of all crashes at the study intersection. The following crash types and/or conditions were determined to be over-represented compared to statewide averages (shown in parenthesis).

- Minor Injury Crash: 14.75% (8.86%)

- Angle Crashes: 21.31% (12.86%)

- Left Turn Crash: 22.95% (15.07%)

Of the top three crash types, the eastbound approach had the most rear end crashes (25 of 50, 50%), the westbound approach and northern Taylor Road (TR 169) Circle K driveway tied for the most left turn crashes (8 of 28, 28.5%), and the northern Taylor Road (TR 169) Circle K driveway had the most angle crashes (16 of 26, 61.5%).

The following countermeasures are proposed to improve safety performance while balancing project costs:

- Geometric Improvements. Extend two eastbound through lanes through the Taylor Road (TR 169) intersection by providing a 5-lane configuration (2 through lanes in both directions with westbound left turn lane) on the east leg of Broad Street (SR 16) to Annette Drive. Also, install an eastbound right turn lane on the west leg of Broad Street (SR 16) approaching Taylor Road (TR 169).

- Traffic Signal Improvements. Install right turn overlap phases and 5-section signal heads for the eastbound right turn lane that will run concurrent with the corresponding northbound left turn phase. Install pedestrian accommodations on the north, east, and south legs of the intersection with installation of marked crosswalks, pedestrian signal heads, push buttons, and ADA ramps. Pedestrian accommodations on north and east legs are proposed as a framework for the future sidewalk installation in the area.

- Driveway Closures. Close the north driveway (driveway nearest to Broad Street) to the Circle K on Taylor Road (TR 169) due to northbound queue blockages. This will reduce crashes caused by vehicles turning in and out of the driveway passing through northbound queues. Also, close the west driveway to Licking Line Plaza / the Duggie Sports Bar (14952 E. Broad Street) on Broad Street (SR 16).

The proposed countermeasures outlined in this safety study will provide the foundation for a future improvement desired by the City of Pataskala to extend the 5-lane cross section (two lanes in both directions with a two-way left turn lane) on the east leg of Broad Street (SR 16) past Village Gate Boulevard. Concept plans showing both the proposed safety-based recommended countermeasures and the long-term future improvement can be found in Appendix G.

# Purpose and Need Statement

The purpose of this study is to evaluate the existing safety performance and to identify potential countermeasures to reduce injury and overall traffic crashes at the intersection of Broad Street (SR 16) and Taylor Road (TR 169). The intersection is in the City of Pataskala, Licking County, Ohio, located 5-miles west of downtown Pataskala, 12-miles east of downtown Columbus, and 0.2-miles east of the Franklin/Licking County line. Note the south leg of Taylor Road (TR 169) is located within the City of Reynoldsburg. This intersection serves as the western gateway into both the City of Pataskala and Licking County.

Broad Street (SR 16) provides a direct connection between the two downtown areas. There are several large traffic generators in the region of the intersection including The Limited campus in the southwest corner of the intersection. In addition, there are several large regional developments coming to Licking County including the Intel Licking County campus. Within District 5, this was the #2 ranked urban intersection in the 2021 HSIP Study Locations list and was the #131 ranked urban intersection statewide. The study intersection’s regional location is shown in Figure 1.

FIGURE 1 - LOCATION MAP

# Existing Conditions

Existing conditions of the study intersection are shown in Figure 2. A field review was conducted on June 28, 2023, to observe traffic operations and document field conditions. Broad Street (SR 16) is oriented in the east-west direction and Taylor Road (TR 169) is oriented in the north-south direction, and Broad Street (SR 16) is classified as an Other Principal Arterial while Taylor Road (TR 169) is classified as a Minor Arterial. The posted speed limit is 45 miles per hour on Broad Street (SR 16), recently lowered from 50 miles per hour, and 35 miles per hour on Taylor Road (TR 169). This is the western gateway intersection into the City of Pataskala and Licking County. The study area included approximately a 400-foot radius from the center of the intersection and was used to capture the crash events for the 5-year period (2018-2022), which included 122 crash events.

FIGURE 2 - STUDY AREA

## Intersection Conditions

The lane configuration at the intersection is as follows:

- Eastbound Broad Street (SR1 6) – three (3) lanes (left, through, right)

- Westbound Broad Street (SR 16) – three (3) lanes (left, through, through-right)

- Northbound Taylor Road (TR 169) – two (2) lanes (left, through-right)

- Southbound Taylor Road (TR 169) – two (2) lanes (left, through-right)

The southbound left turn lane was recently lengthened as part of the reconstruction of the north leg to accommodate the new Dairy Queen development in the northeast corner of the intersection.

The cross-sections approaching the intersection is as follows:

- Eastbound Broad Street (SR 16) – five (5) lanes (2 EB through, 2-way left turn lane, 2 WB through)

- Westbound Broad Street (SR 16) – two (2) lanes (1 EB through, 1 WB through)

- Northbound Taylor Road (TR 169) – three (3) lanes (1 NB through, 2-way left turn lane, 1 SB through)

- Southbound Taylor Road (TR 169) – two (2) lanes (1 NB through, 1 SB through)

The westbound Broad Street (SR 16) approach tapers from a four (4) lane cross-section (1 EB through, WB left turn lane, WB through, WB through-right) at the intersection, to a three (3) lane cross-section with left turn lanes at Annette Drive and Summit Ridge Drive, to a two (2) lane cross section at Belmar Drive.

The existing traffic signal is a traditional span wire configuration. Protected/permissive left turn signal phasing is provided in all directions with 5-section signal heads. Signal heads do have reflective backplates except for the 3-section signal heads for northbound and southbound. There are post-mounted lane control signs on all intersection approaches to inform drivers of intended lane usage with additional overhead lane control signs on the eastbound approach. The overhead lane control signs are mounted approximately 260-ft in advance of the eastbound stop line. There are currently culverts that run underneath the north leg (36” lined RCP culvert) and west leg (4-ft x 8-ft box culvert) of the intersection.

## Eastbound Lane Drop into Right Turn Lane

The eastbound Broad Street (SR 16) approach is two (2) through lanes. The right through lane becomes a right turn only lane at about 225 feet from the stop line, resulting in one through lane at the intersection. Due to the long queue lengths during the evening peak, vehicles in the right through lane make abrupt lane changes into the adjacent through lane near the intersection. Some of these vehicles also block the right lane while trying to make this maneuver as shown in Figure 3. These maneuvers contribute to rear end and sideswipe crash scenarios.

FIGURE 3 - EASTBOUND BROAD ST (SR 16) APPROACH

## Eastbound Left Turn Lane Sight Distance

There is limited sight distance for the eastbound left turn lane due to the formation of a second westbound through lane within 200 feet of the intersection. Westbound through vehicles that use the shared right through/right lane are blocked by westbound traffic in the adjacent lane. Eastbound left turn vehicles do not account for the size of the gap needed to avoid oncoming westbound vehicles in the through/right lane.

## Commercial Driveway Access in the Northeast and Southeast Quadrants

Currently there are five (5) commercial driveways in the northeast and southeast quadrants that are within the functional area of the intersection: two (2) in northeast quadrant and three (3) in the southeast quadrant.  Vehicles that enter and exit the driveways make turns crossing multiple lanes. Durning peak periods, turning vehicles have limited sight distance due to extended queues from the signalized intersection, thereby limiting their ability to see free flow vehicles in the lanes that do not have queues. These maneuvers create potential left turn and angle crash scenarios. Refer to Figures 4 and 5 for these commercial driveway access points.

FIGURE 4 - SOUTHEAST QUADRANT DRIVEWAYS (CIRCLE K GAS STATION)

FIGURE 5 - NORTHEAST QUADRANT DRIVEWAYS (DAIRY QUEEN & LICKING LINE PLAZA)

## Pedestrian Accommodations

There is only one painted crosswalk on the south leg of the intersection, and this crosswalk is without pedestrian signal heads, pedestrian phase, or pedestrian push buttons (see Figure 6). There is no existing sidewalk on the north side of the intersection or along Broad Street (SR 16).

FIGURE 6 - SOUTH LEG CROSSWALK

# Crash Data and Analysis

Crash data was obtained using ODOT’s GIS Crash Analysis Tool (GCAT), housed within the ODOT Transportation Information Mapping System (TIMS) tool. 122 crashes were reported over a five-year period between 2018 and 2022. Each crash report documented within the study limits was reviewed to confirm the accuracy and location of the crash. An excerpt of the intersection crash diagram is included as Figure 8 with the complete crash diagram included in Appendix D.

FIGURE 7 - CRASH DIAGRAM

ODOT utilizes AASHTOWare’s Safety Analyst to prioritize safety locations within the State of Ohio and the subject intersection was ranked #131 (District 5 rank #2) in the 2021 Safety Analyst Urban Intersection priority list. The following crash types and/or conditions were determined to be over-represented compared to statewide averages (shown in parenthesis).

- Minor Injury Crash: 14.75% (8.86%)

- Angle Crashes: 21.31% (12.86%)

- Left Turn Crash: 22.95% (15.07%)

There were no fatalities and one serious injury crash reported within the five-year study period. Rear End (50, 40.98%), Left Turn (28, 22.95%), and Angle (26, 21.31%) were the top three crash types. These three crash types account for 85.25% (104 of 122) of all crashes at the study intersection. Figure 9 shows frequency by crash type at the intersection over the five-year period.

FIGURE 8 - FREQUENCY OF CRASHES BY TYPE OF CRASH

Of the top three crash types, the eastbound approach had the most rear end crashes (25 of 50, 50%), the westbound approach and northern Taylor Road (TR 169) Circle K driveway tied for the most left turn crashes (8 of 28, 28.5%), and the northern Taylor Road (TR 169) Circle K driveway had the most angle crashes (16 of 26, 61.5%).

Rear end crashes on the eastbound approach are attributed to high travel speeds and the length of the eastbound queue in the single through lane.  This creates a stop condition for drivers up to a quarter mile away from the Taylor Road (TR 169) intersection. This condition is compounded by the right through lane becoming an exclusive right turn lane at the intersection. Vehicles in the right through lane attempt lane changes into the left through lane to cut the queue. Some of these vehicles also block the right lane while trying to make this maneuver.

The left turn and angle crashes at the northern Taylor Road (TR 169) Circle K driveway are attributed to limited sight distance caused by northbound queues extending from the Broad Street (SR 16) intersection. Turning vehicles into and out of the driveway have limited sight distance due to queues that form in the northbound left turn lane.

## Benefit Cost Analysis

A benefit cost analysis for the proposed countermeasures was prepared using the ODOT Economic Crash Analysis Tool (ECAT). The financial benefits of the recommended safety countermeasures were determined by comparing the net present value of the project construction costs to the safety benefits provided by the long-term proposed recommended countermeasures. Details of the benefit cost analysis are included in Appendix I.

The addition of lanes at the intersection of Broad Street (SR 16) and Taylor Road (TR 169) means that the basic safety performance functions were modified between existing and proposed conditions; therefore, Part C and Part D CMFs were both applied to this project.  Part C CMFs are focused on the addition of an eastbound Broad Street (SR 16) through lane and a new right turn lane on the eastbound Broad Street (SR 16) approach.

The following Part D crash modification factors are included in the ECAT analysis:

- Implement Systemic Signing and Visibility Improvements at Signalized Intersections.

- Presence of Driveway on an Intersection Approach Corner.

Although the Part D CMFs applied may not perfectly match the proposed improvements, they are believed to be conservative based on the expected improvements in operations with the proposed countermeasures.  The Part D CMF to	implement systemic signing and visibility improvements at signalized intersections conservatively estimates that the traffic signal will be rebuilt in 10 years due to a 10-year service life instead of the 20-year life span seen with other improvements. This effectively doubles the net present cost of this countermeasure. Other safety benefits that can be expected but were not documented with the ECAT analysis include:

- Modify change plus clearance intervals to ITE Recommended Practice.  Clearance intervals will be modified due to changes of the stop line locations on all intersection approaches.

- The conversion of protected/permitted left turn traffic signals to flashing yellow arrow left turn traffic signals.  This is an independent medium-term improvement or would also be applicable with the construction of a new traffic signal in the long-term recommended improvement.

- Closure of the western driveway on westbound Broad Street (SR 16) to Licking Line Plaza.  This driveway is more than 50 feet from the intersection and therefore does not meet the criteria of a driveway on an intersection approach corner.  The closure of this entrance to Broad Street (SR 16) will have a safety benefit for the segment of Broad Street (SR 16) between Taylor Road (TR 169) and Annette Drive.

- Access modification to the driveway on eastbound Broad Street (SR 16) to the Circle K Gas Station.  Driveway access will be maintained but modified to include a raised curb island for right-in / right-out movements only.

Construction costs were estimated for the proposed recommended countermeasures concept plan and the future concept plan as shown in Appendix G. A summary of project costs is shown in the Tables below. Detailed cost estimates are included in Appendix H.  Table 1 summarizes the proposed recommended improvements costs, Table 2 summarizes the benefit cost analysis results for the proposed recommended improvements, and Table 3 summarizes the future improvement estimated costs.

Table 1 – Proposed Recommended Improvements Cost Summary

| Description | Cost |
| --- | --- |
| *Construction Cost | $ 1,520,730 |
| Right of Way | $ 25,000 |
| Contingency (30%) | $ 464,000 |
| Design, Environment, and Construction Engineering Costs (30%) | $ 464,000 |
| INITIAL COST OF COUNTERMEASURES | $ 2,473,730 |
| Inflation (19.6%) | $ 484,000 |
| TOTAL COST | $ 2,957,730 |

*Based upon 2023 Construction Costs

Table 2 – Benefit Cost Results

| Description | Value |
| --- | --- |
| Expected annual crash adjustment | 7.083 crashes/year |
| Net present value of project | $3,265,842 |
| Net present value of safety benefit | $2,855,532 |
| BENEFIT/COST RATIO | 0.87 |

Table 3 – Proposed Future Improvements Cost Summary

| Description | Cost |
| --- | --- |
| *Construction Cost | $ 4,362,620 |
| Right of Way | $ 75,000 |
| Contingency (30%) | $ 1,331,000 |
| Design, Environment, and Construction Engineering Costs (30%) | $ 1,331,000 |
| TOTAL COST | $ 7,099,620 |
| Inflation (19.6%) | $ 1,388,000 |
| TOTAL COST | $ 8,487,620 |

*Based upon 2023 Construction Costs

# Recommendations and Prioritization

Countermeasures have been developed that will improve the safety performance of the study area by focusing on crash types with the greatest potential for mitigation. The focus of the countermeasures is identifying factors that contribute to these crash types and providing measures that will mitigate these factors.

## Short-Term Countermeasures

- Signage Improvements. Improving lane control signs for eastbound Broad Street (SR 16) traffic can help alert drivers earlier to give them enough time to make necessary lane changes. This includes the following:

  - Replace existing faded overhead land control signs for eastbound Broad Street (SR 16).

  - Remove existing ‘Right Lane Ends’ (W9-1R) warning sign located 150-ft in advance of overhead lane use sign and replace it with ‘Right Lane Must Turn Right’ (R3-7R) sign in advance of the right turn lane.

Additional signage improvements include installing signs for ¾-access to Circle K off Broad Street (SR 16) and providing separate and larger ‘Do Not Block Driveway’ signs on Taylor Road (TR 169).

- Pavement Marking Improvements. Revise eastbound Broad Street (SR 16) skip dash pavement markings as summarized below:

  - Add dotted line pavement marking up to about 700 feet west of the stop bar to indicate right turn lane drop at Taylor Road (TR 169).

  - Install additional right turn arrow pavement markings further west of the intersection.

  - Provide offset stop bars for the left turn lanes of both eastbound and westbound Broad Street (SR 16) which will help reduce the size of the intersection.

- Traffic Signal Improvements. Add retroreflective backplates to northbound and southbound 3-section signal heads to match the rest of the intersection and to increase the visibility of the signal heads.

## Mid-Term Countermeasures

- Signage Improvements. Add a secondary overhead sign support for westbound Broad Street (SR 16) further west (about 700 feet) to give an extra advance warning to vehicles in the curb lane for the vehicles to have enough time to merge into the adjacent through lane.

- Pavement Marking Improvements. Provide positive offset left turn lanes for both eastbound and westbound Broad Street (SR 16) to provide additional sight distances for left turn vehicles. Extend the right eastbound through lane through the intersection to Annette Drive and provide the lane drop from there. Shift lanes north on the east leg and utilize the current shoulder area to enable the second eastbound through lane to Annette Drive.

- Traffic Signal Improvements. Traffic signal improvements include the following:

  - Provide pedestrian signal heads and push buttons to the existing crosswalk on the south leg.

  - Install flashing yellow arrows on all approaches at the intersection. Flashing yellow arrows provide an option to run left turns as protected only during peak periods and allows for flexibility in handling of left-turn movements. They are recommended as they provide more clarity for signal operations and are becoming more of a standard mode of operating left-turn movements at signalized intersections throughout Ohio.

- Driveway Restrictions. Proposed driveway access restrictions include the following:

  - Close the north driveway (driveway nearest to Broad Street) to the Circle K Gas Station on northbound Taylor Road (TR 169) due to northbound queue blockages. This will reduce crashes caused by vehicles turning in and out of the driveway passing through NB queues.

  - Close the west driveway (driveway nearest to Taylor Road) to Licking Line Plaza on westbound Broad Street (SR 16) at 14952 E. Broad Street. This will limit vehicle interaction at the access points to Licking Line Plaza, Dairy Queen, and westbound through/right turn movements.

  - Modify access to the driveway on eastbound Broad Street (SR 16) to the Circle K Gas Station to include a raised curb island for right-in/right-out movements only.

## Long-Term Countermeasures (Proposed Recommended Improvements)

- Geometric Improvements. Extend two eastbound through lanes through the Tayor Road intersection by providing a 5-lane configuration (2 through lanes in both directions with westbound left turn lane) on the east leg of Broad Street (SR 16) to Annette Drive. Also, install an eastbound right turn lane.

- Traffic Signal Improvements. Traffic signal improvements include the following:

  - Provide eastbound right turn overlap phase (with additional 5-section signal heads) to run concurrent with northbound left turn phase.

  - Install pedestrian accommodations on the north, east, and south legs of the intersection with installation of marked crosswalks, pedestrian signal heads, push buttons, and ADA ramps. Pedestrian accommodations on north and east legs are proposed as a framework for the future sidewalk installation in the area.

  - Traffic signal poles and equipment location will be optimized such that they will not need to be relocated for the proposed future long-term improvements.

- Driveway Restrictions. Proposed driveway access restrictions include the following:

  - Close the north driveway (driveway nearest to Broad Street) to the Circle K Gas Station on northbound Taylor Road (TR 169) due to northbound queue blockages. This will reduce crashes caused by vehicles turning in and out of the driveway passing through NB queues.

  - Close the west driveway (driveway nearest to Taylor Road) to Licking Line Plaza on westbound Broad Street (SR 16) at 14952 E. Broad Street. This will limit vehicle interaction at the access points to Licking Line Plaza, Dairy Queen, and westbound through/right turn movements.

  - Modify access to the driveway on eastbound Broad Street (SR 16) to the Circle K Gas Station to include a raised curb island for right-in/right-out movements only.

A concept plan showing the proposed recommended long-term countermeasures can be found in Appendix G.

## Proposed Future Improvements

The proposed recommended countermeasures in this safety study will provide the foundation for a future improvement desired by the City of Pataskala. This will be the extension of the 5-lane roadway configuration (2 through lanes in both directions with a 2-way left turn lane) on the east leg of Broad Street (SR 16) to Village Gate Boulevard. The additional eastbound through lane will drop east of Village Gate Boulevard, and the additional westbound through lane will start approximately just west of Village Gate Boulevard. Furthermore, right turn lanes will be installed on the south, east, and north legs of the intersection with Taylor Road (TR 169). Traffic signal poles and equipment location will be optimized as part of the recommended long-term countermeasures to be placed such that they will not need to be relocated for proposed future long-term improvements.

The City of Reynoldsburg has identified the resurfacing of Taylor Road (TR 169) from Broad Street (SR 16) south to Main Street (US 40) within the 2018-2022 Capital Improvement Plan (CIP) as CIP Project No. R-10. Both the proposed recommended and proposed future improvements to the south leg of Taylor Road (TR 169) identified within this study would include some of this 2.2-mile proposed resurfacing.

A concept plan showing the proposed future long-term improvements can be found in Appendix G.

# Summary of Supplemental Traffic Studies

## Data Collection

Turning movement counts (TMC) were collected at the study intersection on April 16, 2019, as part of the Broad Street (SR 16) Corridor Study performed by GPD Group for the City of Pataskala, for a 13-hour period from 6:00 AM to 7:00 PM. Traffic count data is included in Appendix B. The morning peak hour at the study intersection was identified from 7:00-8:00 AM and the afternoon peak hour was identified from 4:45-5:45 PM. The 2019 ADT on Broad Street (SR 16) was 25,852 vehicles per day (vpd) west of Taylor Road (TR 169) and 19,518 vpd east of Taylor Road (TR 169) according to the Mid-Ohio Reginal Planning Commission (MORPC) traffic count maps. The 2019 ADT on Taylor Road (TR 169) was 8,356 vpd north of Broad Street (SR 16) and 10,336 vpd south of Broad Street (SR 16) according to the same MORPC traffic count maps. 2019 peak hour counted traffic volumes were projected for Design Year (2029) using MORPC growth rates received on October 6, 2023, with forecasted volumes shown in Figure 9. Refer to Appendix C for Future Traffic Volume Calculations.

FIGURE 9 - 2029 FORECASTED TRAFFIC VOLUMES

## Capacity Analysis

Capacity analyses were performed to determine operational impacts of the proposed recommended countermeasures and traffic signal improvements. Analyses were prepared for the AM and PM peak periods for the 2019 existing, 2029 No Build, and 2029 build condition with proposed recommended safety countermeasures. Results summarized in Table 3 indicate that LOS D or better is maintained for the intersection and all approaches with the proposed recommended safety countermeasures. Details of the capacity analysis procedures and results are included in Appendix F.

Table 3 – Capacity Analysis Summary

| Intersection/ Approach | AM Peak Hour |  |  | PM Peak Hour |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2019 Existing | 2029 No Build | 2029 Build | 2019 Existing | 2029 No Build | 2029 Build |
|  | LOS (Delay/sec) | LOS (Delay/sec) | LOS (Delay/sec) | LOS (Delay/sec) | LOS (Delay/sec) | LOS (Delay/sec) |
| Broad Street & Taylor Road | C (31.3) | D (41.4) | D (40.2) | D (41.1) | E (73.2) | D (42.7) |
| EB Approach | C (21.8) | C (28.2) | C (23.2) | D (36.7) | D (52.0) | D (38.7) |
| WB Approach | C (31.4) | D (35.9) | D (44.3) | C (29.6) | C (34.9) | D (39.5) |
| NB Approach | D (40.1) | E (57.4) | D (53.5) | D (53.8) | F (130.1) | D (54.8) |
| SB Approach | D (36.0) | E (58.5) | D (42.5) | E (57.7) | F (136.9) | D (47.7) |

# APPENDIX A

Existing Conditions Diagram

# APPENDIX B

Existing Traffic Data

# APPENDIX C

Future Traffic Volumes

# APPENDIX D

Crash Diagram

# APPENDIX E

Crash Summary (CAM Tool Output)

# APPENDIX F

Capacity Analyses

# APPENDIX G

Concept Plans

Proposed Recommended Improvements

Proposed Future Improvements

# APPENDIX H

Cost Estimates

Proposed Recommended Improvements

Proposed Future Improvements

# APPENDIX I

Benefit-Cost (ECAT) Analysis

ECAT Summary

Safety Benefit-Cost Analysis

Part D CMF Details