To calculate the elevation along a vertical curve, you'll need the elevation of the Beginning of Vertical Curve (BVC), the entering and exiting grades, and the distance from the BVC to the point of interest. The general formula is: Elevation at Point = E_BVC + G1 * X + A * X^2, where A = (G2 - G1) / (2 * L).

Here's a breakdown of the terms:

- E_BVC: The elevation at the Beginning of Vertical Curve.

- G1: The entering grade (as a decimal, not a percentage).

- X: The distance from the BVC to the point where you want to find the elevation.

- G2: The exiting grade (as a decimal).

- L: The length of the vertical curve.

- A: A coefficient calculated as (G2 - G1) / (2 * L).

In simpler terms:

- Determine the curve's parameters: You need the elevations and stationing (horizontal location) of the BVC and the End of Vertical Curve (EVC), as well as the entering and exiting grades (G1 and G2) and the curve length (L).

- Calculate the coefficient 'A': Use the formula A = (G2 - G1) / (2 * L).

- Calculate the elevation at any point: Use the formula: Elevation at Point = E_BVC + G1 * X + A * X^2.

Example:

Let's say:

- E_BVC = 598

- G1 = 2% (or 0.02 as a decimal)

- G2 = -1% (or -0.01 as a decimal)

- L = 10 stations (meaning the curve length is 10 times the station interval)

- X = 5 stations (distance from BVC)

- Calculate 'A': A = (-0.01 - 0.02) / (2 * 10) = -0.0015.

- Calculate the elevation at X = 5: Elevation = 598 + (0.02 * 5) + (-0.0015 * 5^2) = 598 + 0.1 - 0.0375 = 604.0625.