C-R-S:  HAM US 50 19.80

PID 124627

Scope Narrative

# PDP Phase Included in this Agreement:

# Agreement is for the Preliminary Engineering (PE) and Detail Design.

# Agreement between Consultant and Ohio Department of Transportation.

# This is a Two-Part agreement. Part 1 will complete the Feasibility Study, conceptual MOT plans, and Stage 1 plans. Part 2 will prepare construction plans through final design.

# Study Location:

Rehabilitate Bridges HAM-50-1980 L/R (SFN 3103005 / 3103064) which carries US 50 over Central Railroad of Indiana, CSX Railroad, Flood Wall, and Linn Street.

Map of existing bridge:

The existing bridges are circled in red on the map below.

Study Description:

Purpose & Need:

The primary issues include:

- Bridges HAM-50-1980 L/R (SFN 3103005 / 3103064) US 50 over Central Railroad of Indiana, CSX Railroad, Flood Wall, and Linn Street have deteriorating concrete bridge decks and superstructure paint and are both rated a 5 out of 9 (9 being best and 4 being deficient).

Secondary issues that require consideration are as follows:

- The barrier on the westbound structure also has condition issues and is rated a 5.

- The bridge does not meet current seismic requirements.

- The repairs are needed to the substructures, other secondary components such as bearings, expansion joints, drainage, etc.

- With scheduled closures on US 50 in the eastbound direction and possible the westbound direction, due to the Brent Spence Bridge project, construction is best to be done during already planned closures of US 50.

Feasibility Study:

A detailed feasibility study is not needed for this project, the following information is needed to determine the necessary scope for seismic, fatigue, and concrete placement:

  - Evaluate the structure for seismic compliance per the Bridge Design Manual. In the event the structure does not meet the seismic requirements, at a minimum, the following shall be explored

    - Evaluate extensions to the beam seat width for seismic beam seat requirements at the abutments and Pier 3.

    - Evaluate restraint systems at beam ends located beneath expansion joints.

    - Evaluate replacing the existing cross frames (configuration and location) over the fixed piers (Pier 2 and Pier 5).

    - Evaluate the use of seismic isolation bearings for the fixed piers.

    - Evaluate adding fixed pier locations.

    - Fiber wrap systems

  - Provide details of fatigue retrofits for the cross frame connections.

  - Perform a fatigue analysis to evaluate the existing top flange full penetration field splice over the piers. Provide a recommended fatigue repair based upon the remaining fatigue life and stress.

  - Determine if a deck pouring sequence is required. Provide a recommended deck pouring sequence if needed.

The feasibility study shall include the estimated costs, estimated duration of repair, shoring requirement, preliminary design of bearings sufficient to evaluate available beam seat width and/or beam seat modifications, and impacts to railroad including the number of days the work will impact the railroad. ODOT will provide the flagging cost per day for inclusion with the estimate.

Project Scope: Discipline specific scope items have been identified below.

Structures:

- Design for the removal and replacement of the existing deck with a composite deck, including the approach slabs per AS-1-15, and barriers (outside and median) per SBR-1-20.

  - Use Stay in place (SIP) metal forms in Spans 1, 2, and 5. ODOT will provide latest SIP during plan development.

- Replace the backwalls down to the beam seat (at minimum). Patch deteriorated portions of the abutments and piers with epoxy injection, patching per 519, and galvanic anodes per SS844 for patches larger than 5 square feet. Consultant to sound the existing substructure to determine repair quantities and locations. Show the limits of substructural concrete removal and analyze the substructure unit with the removal limits stated in CMS 519.03 to ensure the support will not become overstressed or unstable during the concrete removal process. Evaluate whether temporary supports are needed at Pier 3 based on the pier’s condition and the results of the concrete removal.

- Replace the abutment bearings and Pier 3 bearings due to condition. Determine if temporary shoring is needed for bearing replacement or seat modifications.

- Include seismic retrofits as determined by the approved Feasibility Study.

- Per the Feasibility Study, include fatigue retrofits as required.

- Replace the existing scuppers with new scuppers per GSD-1-19 and the existing downspouts down to the ground. Establish processes to clean the portions of the conduit below ground to the first manhole or catch basin. Include a note requiring the new conduit to be galvanized steel only. Other material types are not allowed. (homeless encampment fires have occurred under this bridge in the past).

- Show clearing and grubbing of all trees within the R/W.

- Show locations for welding the cross frame stiffeners to the top and bottom flanges of the plate girders to prevent out-of-plan bending cracks (similar to current standard GSD-1-19). (Anticipate to be spans 4 through 8)

- Paint the existing structural steel per 514 OZEU Specifications. Color shall be Federal Color FS 595C-34058 (dark, muted blue-green with a slight gray cast), as requested by the City Cincinnati.

- Seal the concrete per the typical limits defined within the BDM with epoxy urethane sealer 512 specifications, Federal Color 17778 (gray white). Include pay item for the removal of existing sealers.

- Coordinate existing utilities and lighting. Light poles on the bridge will need to be replaced. ODOT has coordinated the lighting requirements with the City of Cincinnati.

- Add structure grounding per standard drawing.

- Update the existing BrR file (1/17/2024) with the proposed work, reanalysis and submit BR100 for both structures per the requirements of the BDM, Section 900.

- ODOT will provide baseline aesthetics.

Materials for structures:

- Concrete: Include macro-fibers and corrosion inhibitor in concrete mix. District to provide notes during design development.

- Reinforcing steel: Use galvanized reinforcing steel for all new reinforcing steel.

Roadway:

No roadway improvements are anticipated.

Traffic:

No traffic sign impacts are anticipated. Pavement markings shall be epoxy.

Railroad:

Amy Shell is District Railroad Coordinator and will handle most of the railroad coordination with Central Railroad of Indiana (CIND) and CSX. A fully executed PE agreement has already been obtained from CIND (milepost 00.75 AARDOT# 524839N). Communication with CSX has been started (AARDOT# 151408N).

ODOT will coordinate with CSX and CIND for track crossings to be used during construction.

Design Consultant will need to provide the following for railroad coordination

- Address comments and provide disposition to railroad.

- Will include railroad site plan in Part 2 if CSX and CIND require.

- Create site access plan to show temporary crossing agreement and pay item. District will provide example (PID 110570).

- Provide details and pay item for railroad track protection. District will provide example (PID 102886).

Lighting:

Per City of Cincinnati, replace existing poles (Davit arm) in kind to match the style of the rest of US 50. Perform lighting analysis to determine pole placement. Located new poles as close to substructure units as possible. Also don’t place new poles near high power utility lines.

Per City of Cincinnati, maintain existing wall pack lights on the bridge piers that illuminate Linn Street.

Traffic Analysis:

No traffic analysis is required.

Geotechnical:

No geotechnical work is anticipated.

Drainage:

See the structures section for drainage on the bridge. No drainage modifications are anticipated off the bridge. District 8 has made contact with City of Cincinnati about existing flood wall under the bridge but don’t expect any issues.

Maintenance:

None required.

Maintenance of Traffic:

Internal Project Coordination:

The ODOT project manager shall coordinate with the Brent Spence Bridge project manager and District Work Zone Traffic Engineer to best align the project schedule including window contracts and project milestones. The Brent Spence Bridge project plans to close EB and WB 50; it should be noted during portions of these closures traffic to/from 5th/6th Streets may still be maintained

The following US 50 MOT summary is the current estimate relative to the US 50 portion over Linn Street.

- Westbound US 50: 2 lanes are maintained beginning in Phase 1A. There are some connectivity changes during Phase 1B Stage 2; however, 2 lanes are still maintained from downtown via 6th Street. US 50 is restored to 3 lanes with Phase 4A.

- Eastbound US 50: 2 lanes are maintained beginning in Phase 1B Stage 1. Eastbound US 50 is completely closed beginning in Phase 2A Stage 2 and reopens to 2 lanes in Phase 3B Stage 2. US 50 is restored to 3 lanes with Phase 4A.

At a high level, cross over construction can be completed with demo the 1st bridge (westbound) beginning winter and constructed the following year. Then in the following winter, demo the 2nd bridge (eastbound) and construct in the following year.

Important Note: A previous version of the MOT scope included specific closures of US 50 based on the Brent Spence Bridge project. Because these dates remain fluid and subject to change, District 8 requests that previously posted dates not be shared or published.

MOT General:

MOT for this project will utilize lane reductions of an adjacent project. MOT will include maintaining 2 lanes of westbound US 50 and complete closure of eastbound US 50. Westbound US 50 is 3 lanes at the exit to Linn Street; the left (high speed) lane will be closed and traffic maintained in the middle and right (slow speed) lanes. Eastbound US 50 will be completely closed and traffic detoured onto Freeman Avenue ramps.

Phasing for this work will be accomplished in 3 phases:

- Maintain 2 lanes in each direction to construct crossovers.

- Close Eastbound US 50 (by others). Maintain 2 lanes of westbound traffic on eastbound pavement. Construct proposed work on the westbound bridge.

- Eastbound US 50 remains closed. Maintain 2 lanes of westbound traffic on westbound pavement. Remove crossovers and restore median barrier. Construct proposed work on the eastbound bridge.

On Linn Street, 2 lanes shall be maintained in each direction. Short-term lane closures and complete overnight closures will be necessary to complete work. Close the western sidewalk and maintain the eastern sidewalk during Phases 2 and 3.

Part 1 Scope:

- Develop conceptual MOT plans to determine/verify the following:

  - Crossovers are assumed to be located between Carr Street/Linn Street and between Linn Street/Gest Street. Determine if crossover geometry will meet a 45-mph MOT design speed. If not, what design speed can be met.

  - Identify any constraints or features that need to be addressed, mitigated, or avoided to provide crossovers.

  - Determine if it is necessary to shift traffic onto the outside shoulder during Phase 1 to construct the crossover.

Part 2 Scope:

- Detailed Design should follow that appropriate for the particular stage being submitted.

- Develop detailed MOT plans for Phase 1 and Phase 2.

- US 50 Eastbound – MOT for road closure and detour routes will be designed and installed by others.

- Innovative contracting methods to be used:

  - Include a Disincentive Table with associated Interim Completion Date Requirements plan note for the deck replacement and painting.

  - Lane Value Contracts for short term lane closures within permitted hours for US 50 and for Linn Street. Include event restrictions note with the table.

- Include the following plan notes: City of Cincinnati Permits, City of Cincinnati Transit Operations, and Adjacent Project Coordination

- Develop a pedestrian detour plan for Linn Street; the pedestrian detour will use the western sidewalk along Linn Street between Daltan Avenue and W 6th Street. Provide details for a pedestrian canopy to provide overhead protection.  Provide temporary crosswalk markings and advance signing for a crosswalk at W 6th Street across Linn Street

Environmental:

ODOT District 8 will complete all environmental work. Asbestos survey to be done by design consultant.

Survey:

The consultant shall provide all necessary survey.

Right-of-Way:

The existing aerial easement allows for the construction, operation, maintenance and replacement of the structure. A temporary easement is needed with the railroad for access on their property. An acquisition is anticipated with 1 to 2 parcels for access to the aerial easement.

Utility Coordination Requirements:

Consultant to try to avoid utility conflicts throughout design while holding to the scope of work.  If utility conflicts cannot be avoided, they should be minimized. Consultant to provide a copy of the OUPS ticket information to ODOT PM (if applicable). Up to date utility contacts shall be used at each plan submission.  Utility contact information can be requested by consultant from ODOT PM. If Ohio 811 (OUPS) are more than two (2) years old, a design non-marking ticket shall be requested to obtain most up to date Utility Members List. The ticket does not need to be submitted to obtain the Utility Members List.

Consultant to provide a utility set of plans with the utility lines shown in color using the most recent version of ODOTcadd_UTPen.tbl at each plan submission. This file is found in the standard ODOTcadd executable file that can be downloaded from the CADD services webpage. Additionally, Consultant to prepare a summary of potential utility conflicts at each plan submission. Summary to be provided to Utility Companies at each plan submission. Summary to include, but not limited to station and offset of conflict, type of conflict (direct, decreased cover, proximity, etc.), utility owner (if known) and utility type. Consultant to use District 8's 'standardized' letter for sending submissions and plans to Utility Companies for review and comment. Consultant to provide the ODOT PM a copy of all Utility Correspondence. Consultant to compile Utility Company responses and forward to the ODOT PM. Final compilation of utility correspondence is due 35 days after plan submission to utilities

A “no response” from a utility on a plan submission review cannot be considered as “no comment”, “no conflicts” and/or “a confirmation of the consultant’s findings” from the utility. A written response (email is sufficient) must be received from the utility verifying that they have no comments, no conflicts and/or they agree with the conflicts identified by the consultant.

Consultant to review the Utility Company responses and evaluate. The evaluation of the responses shall include validating that a conflict does exist or that a utility may remain in place. If a conflict does exist, consultant should provide an evaluation of the feasibility of potential resolutions. A disposition of utility status (i.e. utility to stay in place, utility facility relocation plan in writing or plan format) is required at the Stage 3 submission. This disposition shall be included to the utilities with the Stage 3 plan submission. This disposition shall be formulated based on utility responses from previous plan submissions.

A utility note spreadsheet (District will provide) which can be used by ODOT to draft a utility note through Ellis shall be submitted after evaluation of the Stage 3 utility coordination. The template spreadsheet can be provided by ODOT as needed. The spreadsheet should include discussion about the existing utilities for each utility, if they are staying in place and in service or if they are being relocated. If a utility is relocating, information about the location of their relocation should be included. Additionally, the relocation time frames should be included in the utility note as discussed with the utility companies. Example utility notes can be provided by the District utility coordinator upon request.

Project Management:

This project shall be a Two Part Project. Part 1 will complete the Feasibility Study, conceptual MOT plans, and Stage 1 plans. Part 2 will complete construction plans for the project including detailed design of the seismic retrofit items from the approved Feasibility Study.

Funding:

This project will be funded 50%/50% using Major Investments / District Allocation funding that are both State and Federal.

Design Designations:

|  | HAM US 50 19.80 |
| --- | --- |
| Functional Classification | 02 Other Freeway/Expressway (urban) |
| Design Functional Classification | 02 Other Freeway/Expressway (urban) |
| National Highway System (NHS) | Yes- Intermodal Connector |
| Opening Year ADT (2031) | 40,500 |
| Design Year ADT (2051) | 42,500 |
| Design Hourly Volume | 5,500 |
| Direction Distribution | 0.62 |
| Trucks (24 Hour B&C) | 8% |

Existing Plans and Bridge Inspection Photos:

See the FTP Site for existing plans and photos.

ftp.dot.state.oh.us - /pub/Districts/D08/Programmatics/2026-September/HAM-50-1980 PID 124627/

Schedule:

The Official schedule will be maintained in Ellis. The consultant may propose changes to the schedule that don’t alter the final plan package submittal or sale dates.

ODOT Internal Links:

|  | Arch No | Name | Year | PID | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | 08c2204 | HAM-50-19.76 | 1962 | n/a | Original Construction |
| 2 | 08c1739 | HAM-50-1936 | 1992 | n/a | Minor Rehab |
| 3 | 08c3057 | HAM-50-1903 (Part 2) | 2011 | 20082 | Minor Rehab (P.400 of PDF) |

2025 Insp Photos:

J:\structures\bridges\25 photos\HAM\US50\1980 L

J:\structures\bridges\25 photos\HAM\US50\1980R

PIP photos are available here: J:\structures\bridges\26 photos\HAM\US50\1980R\PIP 02-17-26