Project Name:  CUY-480-17.87/17.90	Mainline Design Speed:   65 mph I-480 & I-77

PID:  110810		Ramp Design Speed:   	55 mph; 45 mph minimum (Per L&D)

Ramp Legal Speed:     N/A mph

Design Year ADT (2046):	16000

Design Functional Classification:  Interstate (Urban) (Ramp)

Roadway:  IR480 WB to IR77 SB Directional Ramp E-S

| ROW # | Key Highway Design Features Requiring Design Exceptions (if not met or exceeded) | Normal Design Criteria |  |  | Project Specific Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Section | Figure | Required Design Criteria |  | Existing | Proposed Design Criteria | Design Exception | Comments |
| 1 | Traveled Way Width | 303.1 | 303-1 | 16’ |  | 16’ | 16’ | No |  |
| 2 | Paved Shoulder Width | 303.1 | 303-1 | 4’ Lt., 6’ Rt. |  | 4’ Lt., 10’ Rt. | 4’ Lt., 10’ Rt. (Roadway) 3.5’ Lt., 7.5’ Rt. (Structure) | Yes* | *For left shoulder width on structure |
| 3 | Graded Shoulder Width | 303.1 | 303-1 | w/ Barr. / Slope > 6:1: 9’ Lt. / 11’ Rt. w/o Barr./ Slope < 6:1: 6’ Lt. / 8’ Rt. |  | w/ Barr./ Slope > 6:1 9’ Lt. / 13’ Rt. w/o Barr./ Slope < 6:1: 6’ Lt. / 10’ Rt | w/ Barr./ Slope > 6:1 9’ Lt. / 15’ Rt. w/o Barr./ Slope < 6:1: 6’ Lt. / 10’ Rt | No |  |
| 4 | Max. Centerline Deflection w/o Horizontal Curve | 202.2 | 202-1 | Varies for Ramp Speed |  | None | None | N/A | No Deflections w/o Horizontal Curve |
| 5 | Maximum Degree of Curve | 202.3 | 202-2 | Mainline (65 mph) 3° 30’ Design Speed (60 mph) 4° 15’ Upper Range (55 mph) 5° 30' Middle Range (45 mph) 9° 00’ Lower Range (30 mph) 24° 45’ |  | 2° 45’ 00” 4° 00’ 00” | 2° 45’ 00” 4° 00’ 00” | No |  |
| 6 | Grade Breaks | 203.3.2 | 203-2 | 0.30% (65 mph-ML) 0.30% (60 mph DS) 0.40% (55 mph-U) 0.55% (45 mph-M) 1.30% (30 mph-L) |  | 0.02% Per Record Plan | 0.40 0.32 0.30 0.30 0.30 | N/A |  |
| 7 | Grades | 203.2, 503.3 | 203-1; (Table 503-1) | 5% upgrade (> 45 mph) 7% downgrade (> 45 mph) 6% upgrade (35-40 mph) 8% downgrade (35-40 mph) 7% upgrade (25-30 mph) 8% downgrade (25-30 mph) |  | 3.66% max. upgrade 2.39% max. downgrade | 3.66% max. upgrade 2.39% max. downgrade | No |  |
| 8 | Stopping Sight Distance | 201.2 | 201-1, 203-3, -4, -6 & -7 | 645’ (65 mph-ML) 570’ (60 mph DS) 495’ (55 mph-U) 360’ (45 mph-M) 200’ (30 mph-L) |  | Design Speed – 55 mph Crest- N/A Sag – 744’ Horiz. 362’ at bridge parapet | Design Speed – 55 mph Crest- N/A ML Range (55 mph): Horiz. 362’ at bridge abutment | No | Horizontal SSD limited due to existing bridge parapet; meets Middle Range criteria |
| 9 | Superelevation Rate | 202.4.1 | 202-8, -10 | 0.06 MAX 2°45’00” curve 0.053 (60 mph DS) 0.030 (45 mph Middle) 4°00’00” curve 0.06 (60 mph DS) 0.046 (45 mph Middle) |  | 2°45’00” curve 0.035 4°00’00” curve 0.066 | 2°45’00” curve 0.035* 4°00’00” curve 0.066** | Yes | * Match existing. Project scope is overlay only. Meets middle range design speed requirement **On bridge. Design exception required |
| 10 | Vertical Clearance | 302.1 | 302-1 | 15.5’ if alternate interstate routing has 16.5’ minimum clearance: 16.5’ if no alternate routing |  | 16.58’ under Ramp 15.05’ over Ramp | 16.50’ under Ramp 15.05’ over Ramp | Yes |  |
| 11 | Structural Capacity | 301.4 (BDM) |  | HL-93 LOADING PLUS FUTURE WEARING SURFACE OF 60 PSF |  | HS20-44 and Interstate Alternate Loading | HL-93 LOADING PLUS FUTURE WEARING SURFACE OF 60 PSF | No |  |