# Appendix C – Sample Plan Notes

| The Sample plan notes included in this Appendix are the most frequently used. Each note is accompanied by a Designer Note to provide guidance on when to use the note and how to estimate quantities for some of the items where the methods for quantity calculations are not obvious. The following note categories are included: Category; Letter Prefix. Drainage Notes; D. Erosion Control Notes; E. Water Quality Notes; W. |
| --- |
| NOTE | NAME |
| D101 | Item 611 – Catch Basin Grate |
| D102 | Item 611 – Conduit Misc.: Internal Joint Seal |
| D103 | Item Special – Fill and Plug Existing Conduit |
| D104 | Crossings and Connections to Existing Pipes and Utilities |
| D105 | Pipe Connections to Corrugated Metal Structures |
| D106 | Item 611 – Tunnel Liner Plate Structure |
| D107 | Farm Drains |
| D109 | Spring Drains |
| D110 | Drainage Discharge Continuance |
| D111 | Unpermitted Drainage Discharge Removal |
| D112 | Item 611 – Conduit Bored or Jacked |
| D113 | Item 611 – Conduit Under Railroad |
| D114 | Review of Drainage Facilities |
| D117 | Manholes, Catch Basins and Inlets Removed or Abandoned |
| D119 | Item Special – Miscellaneous Metal |
| D120 | Item 611 – Slotted Drain |
| D121 | Item Special – Pipe Cleanout |
| D123 | Existing Underdrains |
| D124 | Temporary Drainage Items |
| E101 | Seeding and Mulching |
| E102 | Sodding |
| W100 | Post Construction Storm Water Treatment |
| W101 | Bioretention Cell(s) |
| W102 | Infiltration Trench (or Basin) |
| W103 | Manufactured Water Quality Structure |
| W104 | Vegetated Filter Strip |
| W105 | Vegetated Biofilter |
| W106 | Extended Detention Basin |

| D101 | ITEM 611 - CATCH BASIN GRATE |
| --- | --- |
|  | MODIFY EXISTING CATCH BASINS BY REPLACING THE EXISTING GRATES WITH BICYCLE SAFE GRATES. QUANTITIES AND LOCATIONS ARE SHOWN IN THE PLANS AND ARE PAID FOR AT THE CONTRACT PRICE FOR ITEM 611, CATCH BASIN GRATE, TYPE _____, EACH. Designer Note: Use the above note on projects where existing catch basin grates are not bicycle safe. The size and type of grate to be supplied must be indicated. There may be more than one type and size on a project. If specific locations are not shown in the plan, or additional grates are to be included on a contingency basis, use the following to either replace the second sentence in the note or add to the note: THE FOLLOWING ESTIMATED QUANTITIES HAVE BEEN INCLUDED IN THE GENERAL SUMMARY FOR USE AS DIRECTED BY THE ENGINEER FOR REPLACEMENT OF EXISTING CATCH BASIN GRATES WITH BICYCLE SAFE GRATES: ITEM 611, CATCH BASIN GRATE, TYPE ______ ______EACH |
| D102 | ITEM 611 - CONDUIT MISC.: INTERNAL JOINT SEAL |
|  | PROVIDE AN INTERNAL JOINT SEAL AT THE LOCATIONS INDICATED IN THE PLANS. COMPRESS THE INTERNAL JOINT SEAL AGAINST THE INTERNAL DIAMETER OF THE PIPE COVERING THE JOINT. EXTEND THE INTERNAL JOINT SEAL A MINIMUM OF 4 INCHES FROM EITHER SIDE OF THE JOINT. FURNISH AN INTERNAL JOINT SEAL COMPOSED OF EITHER MECHANICAL EXPANSION BANDS WITH LOCKING ASSEMBLY, A FIBERGLASS BASED RESIN POINT REPAIR, OR AN APPROVED EQUAL. PERFORM THE REPAIRS ON THE FOLLOWING CONDUITS: STA. _+__, ___” CONDUIT STA. _+__, ___” CONDUIT CLEAN AND REMOVE ALL DIRT AND DEBRIS PRIOR TO INSTALLATION OF THE INTERNAL JOINT SEAL. FILL THE VOIDS WITH GROUT. SEALS CAN BE SUPPLIED WITHOUT TEST VALVES. INSTALL THE INTERNAL JOINT SEAL PER THE REQUIREMENTS OF THE MANUFACTURER. THE INTERNAL JOINT SEAL, LABOR, EQUIPMENT AND ANY OTHER INCIDENTALS NECESSARY TO COMPLETE THE WORK ARE INCLUDED IN: ITEM 611, CONDUIT MISC.: INTERNAL JOINT SEAL ______ EACH Designer Note: Use the above note to repair failing joints and prevent soil infiltration on structurally sound culverts and storm sewers. Verify the size, material and shape availability to perform the repair with the manufacturer or the OHE. |
| D103 | ITEM SPECIAL - FILL AND PLUG EXISTING CONDUIT |
|  | THIS ITEM CONSISTS OF THE CONSTRUCTION OF BULKHEADS IN AN EXISTING ____ INCH DIAMETER CONDUIT AND FILLING THE AREA SEALED OFF WITH ITEM 613, SAND OR OTHER MATERIAL APPROVED BY THE ENGINEER. LOCATE THE BULKHEADS AT THE LIMITS OF THE AREA TO BE FILLED, AS INDICATED ON THE PLANS. THE BULKHEADS CONSIST OF BRICK OR CONCRETE MASONRY WITH A MINIMUM THICKNESS OF 12 INCHES. PUMP THE FILL MATERIAL INTO PLACE OR BY OTHER MEANS APPROVED BY THE ENGINEER, SO THAT AFTER SETTLEMENT, AT LEAST 90 PERCENT OF THE CROSS-SECTIONAL AREA OF THE CONDUIT, FOR ITS ENTIRE LENGTH IS FILLED. THE LENGTH OF FILLED AND PLUGGED CONDUIT TO BE PAID FOR IS THE ACTUAL NUMBER OF FEET (MEASURED ALONG THE CENTERLINE OF EACH CONDUIT FROM OUTER FACE TO OUTER FACE OF BULKHEADS) FILLED AND PLUGGED AS DESCRIBED ABOVE. IN LIEU OF FILLING AND PLUGGING THE EXISTING CONDUIT, THE PIPE MAY BE CRUSHED AND BACKFILLED PER 203, OR IT MAY BE REMOVED. THE LENGTH, MEASURED AS PROVIDED ABOVE, WILL BE PAID FOR AT THE CONTRACT PRICE PER FOOT FOR, ITEM SPECIAL, FILL AND PLUG EXISTING CONDUIT. Designer Note: Use the above note when it is desired to abandon an existing conduit by filling and plugging rather than more conventional methods. If the conduit is in shallow fill, the designer may delete the crush and backfill option specified in the fourth paragraph. Add pay item 202E70000 “202, Special – Fill and plug existing conduit, ___ft” to the plans. |
| D104 | CROSSINGS AND CONNECTIONS TO EXISTING PIPES AND UTILITIES |
|  | WHERE PLANS PROVIDE FOR A PROPOSED CONDUIT TO BE CONNECTED TO, OR CROSS OVER OR UNDER AN EXISTING SEWER OR UNDERGROUND UTILITY, LOCATE THE EXISTING PIPES OR UTILITIES BOTH AS TO LINE AND GRADE BEFORE STARTING TO LAY THE PROPOSED CONDUIT. IF IT IS DETERMINED THAT THE ELEVATION OF THE EXISTING CONDUIT, OR EXISTING APPURTENANCE TO BE CONNECTED, DIFFERS FROM THE PLAN ELEVATION OR RESULTS IN A CHANGE IN THE PLAN CONDUIT SLOPE, NOTIFY THE ENGINEER BEFORE STARTING CONSTRUCTION OF ANY PORTION OF THE PROPOSED CONDUIT WHICH WILL BE AFFECTED BY THE VARIANCE IN THE EXISTING ELEVATIONS. IF IT IS DETERMINED THAT THE PROPOSED CONDUIT WILL INTERSECT AN EXISTING SEWER OR UNDERGROUND UTILITY IF CONSTRUCTED AS SHOWN ON THE PLAN, NOTIFY THE ENGINEER BEFORE STARTING CONSTRUCTION OF ANY PORTION OF THE PROPOSED CONDUIT WHICH WOULD BE AFFECTED BY THE INTERFERENCE WITH AN EXISTING FACILITY. PAYMENT FOR ALL THE OPERATIONS DESCRIBED ABOVE IS INCLUDED IN THE CONTRACT PRICE FOR THE PERTINENT 611 CONDUIT ITEM. Designer Note: Use the above note when the designer is unsure of the exact location of a conduit that will require an extension or where the potential for interference between proposed and existing conduits exists. |
| D105 | PIPE CONNECTIONS TO CORRUGATED METAL STRUCTURES |
|  | PROVIDE CONNECTIONS OF PROPOSED LONGITUDINAL DRAINAGE TO CORRUGATED METAL STRUCTURES BY MEANS OF A SHOP FABRICATED OR FIELD WELDED STUB ON THE STRUCTURE. FURNISH A STUB MEETING THE REQUIREMENTS OF 707 WITH A MINIMUM LENGTH OF 2 FEET AND A MINIMUM WALL THICKNESS OF 0.064 INCHES. THE LOCATION AND ELEVATION OF THE STUB ARE TO BE CONSIDERED APPROXIMATE AND MAY BE ADJUSTED BY THE ENGINEER TO AVOID CUTTING THROUGH JOINTS IN THE STRUCTURE. THOROUGHLY CLEAN AND REGALVANIZE OR OTHERWISE SUITABLY REPAIR THE FIELD WELDED JOINT, IF USED. MEET WELDING REQUIREMENTS OF 513.21. PROVIDE A MASONRY COLLAR PER STANDARD CONSTRUCTION DRAWING DM-1.1, TO CONNECT THE LONGITUDINAL DRAINAGE TO THE STUB, WHEN PIPE OTHER THAN CORRUGATED METAL IS USED FOR THE LONGITUDINAL DRAINAGE. PAYMENT FOR CUTTING INTO THE STRUCTURE AND PROVIDING THE CONNECTION DESCRIBED, IS INCLUDED IN THE CONTRACT PRICE FOR ITEM 611 OR 522. Designer Note: Use the above note on all projects where connections to existing corrugated metal conduits are proposed. |
| D106 | ITEM 611 - TUNNEL LINER PLATE STRUCTURE |
|  | PROVIDE GALVANIZED STEEL OR ALUMINUM TUNNEL LINER PLATE MANUFACTURED BY: DSI TUNNELING, LLC; CONTECH ENGINEERED SOLUTIONS, LLC.; OR AN APPROVED EQUAL. THE MINIMUM THICKNESS SHOWN IN THE PLANS IS PROVIDED FOR THE DURABILITY DESIGN. INCLUDE THE PLATE THICKNESS IN THE INSTALLATION PLANS PER CMS 611 FOR THE STRUCTURAL REQUIREMENTS. PROVIDE MATERIAL FOR GALVANIZED STEEL TUNNEL LINER PLATE PER 707.03. PROVIDE MATERIAL FOR ALUMINUM TUNNEL LINER PLATE PER 707.23. PROVIDE CONCRETE FIELD PAVING PER 611.11. FILL THE ANNULAR SPACE BETWEEN THE TUNNEL EXCAVATION AND THE TUNNEL LINER PLATE PER THE REQUIREMENTS OF 837.03 C AND D. FOR ALUMINUM TUNNEL LINER PLATE, PROVIDE AN OUTSIDE COATING PER 837.02 B PRIOR TO GROUTING OPERATIONS. Designer Note: Perform conduit durability design per L&D Vol. 2 – 1002.2.2 to determine minimum plate thickness and/or level of protection. Perform durability using 707.03 Structural Plate Corrugated Steel for Steel Tunnel Liner Plate and 707.23 Aluminum Alloy Structural Plate for Aluminum Tunnel Liner Plate. |
| D107 | FARM DRAINS |
|  | PROVIDE UNOBSTRUCTED OUTLETS TO ALL FARM DRAINS ENCOUNTERED DURING CONSTRUCTION. REPLACE EXISTING COLLECTORS WHICH ARE LOCATED BELOW THE ROADWAY DITCH ELEVATIONS, AND WHICH CROSS THE ROADWAY WITHIN THE (RIGHT OF WAY)( CONSTRUCTION) LIMITS WITH ITEM 611, CONDUIT, TYPE B, ONE COMMERCIAL SIZE LARGER THAN THE EXISTING CONDUIT. OUTLET EXISTING COLLECTORS AND ISOLATED FARM DRAINS, WHICH ARE ENCOUNTERED ABOVE THE ELEVATION OF ROADWAY DITCHES INTO THE ROADWAY DITCH USING ITEM 611, TYPE F CONDUIT. THE OPTIMUM OUTLET ELEVATION IS ONE FOOT ABOVE THE FLOWLINE ELEVATION OF THE DITCH. INTERCEPT LATERAL FIELD TILES WHICH CROSS THE ROADWAY WITH ITEM 611, TYPE E CONDUIT, AND CARRY IN A LONGITUDINAL DIRECTION TO AN ADEQUATE OUTLET OR ROADWAY CROSSING. THE LOCATION, TYPE, SIZE AND GRADE OF REPLACEMENTS IS DETERMINED BY THE ENGINEER AND PAYMENT MADE ON FINAL MEASUREMENTS. PROVIDE EROSION CONTROL PADS AT THE OUTLET END OF ALL FARM DRAINS PER STANDARD CONSTRUCTION DRAWING DM-1.1, EXCEPT WHEN THEY OUTLET INTO A DRAINAGE STRUCTURE. PAYMENT FOR THE EROSION CONTROL PADS AND ANY NECESSARY BENDS OR BRANCHES IS INCLUDED FOR PAYMENT IN THE PERTINENT CONDUIT ITEMS. THE FOLLOWING ESTIMATED QUANTITIES HAVE BEEN INCLUDED IN THE GENERAL SUMMARY FOR THE WORK NOTED ABOVE: ITEM 611, “ CONDUIT, TYPE B ______ FT ITEM 611, “ CONDUIT, TYPE E ______ FT ITEM 611, “ CONDUIT, TYPE F ______ FT ITEM 601, ROCK CHANNEL PROTECTION TYPE C WITH FILTER ______ CY Designer Note: Use the above note where excavation may conflict with existing farm drains. Use of a lateral field interceptor tile located on a temporary easement outside the limited access right of way may be appropriate on limited access facilities. |
| D109 | SPRING DRAINS |
|  | THE FOLLOWING ESTIMATED QUANTITIES HAVE BEEN CARRIED TO THE GENERAL SUMMARY FOR USE AS DIRECTED BY THE ENGINEER FOR DRAINING ANY SPRINGS SHOWN IN THE PLAN OR ENCOUNTERED DURING CONSTRUCTION. THE FOLLOWING PIPE MATERIALS ARE PERMITTED: 707.33, 707.41, 707.42 or 707.45 PERFORATED PER 707.31. CONSTRUCT SPRING DRAINS PER STANDARD CONSTRUCTION DRAWING DM-1.1 AND PAID FOR AT THE CONTRACT PRICE FOR: ITEM 605, 6" UNCLASSIFIED PIPE UNDERDRAINS FOR SPRINGS ______ FT ITEM 605, AGGREGATE DRAINS FOR SPRINGS ______ FT ITEM 611, PRECAST REINFORCED CONCRETE OUTLET ______ EACH Designer Note: Use this only where springs are present in the project area and/or the project area is known to have spring activity. In addition to quantities required to drain springs located by field work, include estimated contingency quantities for draining springs encountered during construction. |
| D110 | DRAINAGE DISCHARGE CONTINUANCE |
|  | FURNISH A DRAINAGE DISCHARGE CONTINUANCE FOR ANY DRAINAGE DISCHARGE DISTURBED BY THE WORK AND NOT SHOWN IN THE PLANS. THE LOCATION, TYPE (CONDUIT OR SWALE), SIZE AND GRADE OF THE DRAINAGE DISCHARGE CONTINUANCE WILL BE AGREED TO BY THE ENGINEER. FURNISH AN INSPECTION WELL AT THE RIGHT OF WAY LINE IN PER STANDARD CONSTRUCTION DRAWING DM-3.1 FOR EACH DRAINAGE DISCHARGE THAT OUTLETS THROUGH A CURB OPENING OR INTO A STORM SEWER OR DRAINAGE STRUCTURE. THE COST IS INCLUDED IN ITEM 611, INSPECTION WELL. FURNISH A WELL GRADED TRANSITION BETWEEN THE DITCH AND THE SWALE WHEN OUTLETTING A SWALE TO A DITCH. THE COST FOR THE GRADED TRANSITION IS INCLUDED IN ITEM 203, EMBANKMENT, AS PER PLAN. FURNISH AN EROSION CONTROL PAD AS SHOWN IN STANDARD CONSTRUCTION DRAWING DM-1.1 WHEN OUTLETTING A CONDUIT TO A DITCH. THE COST FOR THE EROSION CONTROL PAD IS INCLUDED IN ITEM 611, CONDUIT, MISC TYPE _ FOR DRAINAGE DISCHARGE CONTINUANCE. FURNISH A DRILLED HOLE OR A CURB SECTION WHEN OUTLETTING A CONDUIT THROUGH A CURB OPENING. THE COST OF DRILLING OR FURNISHING THE CURB SECTION WITH HOLE IS INCLUDED IN ITEM 611, CONDUIT, MISC TYPE _ FOR DRAINAGE DISCHARGE CONTINUANCE. FURNISH A DRILLED CORE HOLE WHEN OUTLETTING INTO A STORM SEWER OR DRAINAGE STRUCTURE. THE COST OF THE DRILLED CORE HOLE IS INCLUDED IN ITEM 611, CONDUIT, MISC TYPE _ FOR DRAINAGE DISCHARGE CONTINUANCE. DOCUMENTATION PROVIDE WRITTEN DOCUMENTATION TO THE ENGINEER AND TO THE DISTRICT R/W PERMIT OFFICE. THE DOCUMENTATION INCLUDES THE CONSTRUCTION PROJECT NUMBER, PID, COUNTY, ROUTE, SECTION, LATITUDE AND LONGITUDE OF THE DRAINAGE DISCHARGE AT THE R/W, THE NAME OF PROPERTY OWNER WITH ADDRESS, THE DATE THE DRAINAGE DISCHARGE WAS LOCATED, THE DATE THE DRAINAGE DISCHARGE CONTINUANCE WAS FURNISHED, A DETAILED DESCRIPTION OF THE WORK AND PICTURES OF THE DRAINAGE DISCHARGE CONTINUANCE (IN PDF OR JPEG FORMAT). THE DOCUMENTATION IS INCLUDED IN ITEM 611, CONDUIT, MISC TYPE _ FOR DRAINAGE DISCHARGE CONTINUANCE OR ITEM 203, EMBANKMENT, AS PER PLAN DRAINAGE DISCHARGE CONTINUANCE REMOVAL THE ENGINEER MAY REQUIRE THE NEWLY INSTALLED DRAINAGE DISCHARGE CONTINUANCE TO BE REMOVED. REMOVE THE NEWLY INSTALLED CONDUIT AND ANY EXISTING CONDUIT TO THE RIGHT OF WAY LINE. FOR CONDUIT THAT OUTLETS THROUGH THE CURB, RESTORE THE CURB BY FILLING THE HOLE WITH CLASS QC 1 CONCRETE OR REPLACE THE CURB SECTION. FOR CONDUIT THAT OUTLETS TO A STORM SEWER OR DRAINAGE STRUCTURE LEAVE 6 INCHES PROTRUDING OUTSIDE OF THE CONDUIT. PLUG THE PROTRUDING CONDUIT WITH EITHER A MANUFACTURED CAP OR CLASS QC 1 CONCRETE. FOR CONDUIT THAT OUTLETS TO THE DITCH REMOVE THE EROSION CONTROL PAD. RESTORE ALL AREAS AS REQUIRED. PLUG THE EXISTING CONDUIT REGARDLESS OF SIZE AT THE RIGHT OF WAY LINE WITH CLASS QC 1 CONCRETE AND RESTORE ALL AREAS AS REQUIRED. ALL COSTS ARE INCLUDED IN ITEM 202, REMOVAL MISC.: CONDUIT. DAM THE SWALE THAT OUTLETS TO THE DITCH AT THE R/W AS DIRECTED BY THE ENGINEER. ALL COSTS ARE INCLUDED IN ITEM 203, EMBANKMENT AS PER PLAN REMOVE THE INSPECTION WELL AND RESTORE ALL AREAS AS REQUIRED. THE COST IS INCLUDED IN ITEM 202, REMOVAL MISC.: INSPECTION WELL. CONDUIT MATERIAL TYPES THE FOLLOWING CONDUIT MATERIAL TYPES ARE PERMITTED: 707.33, 707.41 NON-PERFORATED, 707.42, 707.43, 707.45, 707.46, 707.47, AND 707.51. PAY ITEMS EACH OF THE PAY ITEMS LISTED BELOW FOR CONDUIT MISCELLANEOUS TYPES B, C, E AND F FOR DRAINAGE DISCHARGE CONTINUANCE INCLUDE CONDUIT SIZES 2 INCH TO 10 INCH. THERE IS NO COST DIFFERENTIATION FOR SIZE IN THESE PAY ITEMS. THE FOLLOWING ESTIMATED QUANTITIES HAVE BEEN INCLUDED IN THE GENERAL SUMMARY FOR USE AS DIRECTED BY THE ENGINEER IN MAKING THE ABOVE DRAINAGE DISCHARGE CONTINUANCE: ITEM 611, INSPECTION WELL ______ EACH ITEM 611, CONDUIT, MISC.: TYPE C FOR DRAINAGE DISCHARGE CONTINUANCE ______ FT ITEM 611, CONDUIT, MISC.: TYPE E FOR DRAINAG DISCHARGE CONTINUANCE ______ FT ITEM 611, CONDUIT, MISC.: TYPE F FOR DRAINAGE DISCHARGE CONTINUANCE ______ FT ITEM 202, REMOVAL MISC.: CONDUIT ______ FT ITEM 202, REMOVAL MISC.: INSPECTION WELL ______ EACH ITEM 203, EMBANKMENT, AS PER PLAN ______ CY Designer Note: Make a complete investigation within the project limits for the presence of any drainage discharge of: - Treated septic, treated wastewater, treated curtain/gradient drains, treated foundation floor drains - Sanitary wastewater, Sanitary curtain/gradient drains, or Sanitary foundation floor drains - Roof drains, footer drains, or yard drains Send written documentation to the District R/W Permit Office for all drainage discharge discovered by the investigation. Include the PID, county, route, section, latitude and longitude of the drainage discharge location at the R/W, the name and address of the property owner, and a detailed description of the inspection including date and pictures of the drainage discharge in pdf or jpeg format. The District R/W Permit Office will direct the designer as to the continuance or removal and plugging of each drainage discharge discovered by the investigation. If the District R/W Permit Office requires the drainage discharge be removed and plugged, include plan note D111. Add note D110 only if any drainage discharge locations are discovered during the investigation and if the investigation yields any possibility that additional drainage discharge locations could be discovered during construction. Furnish a minimum of 5 feet for each lane mile of project length rounded up to the next 5 foot increment for each listed type of conduit above. Furnish a minimum of 1 inspection well for each 20 feet of conduit estimated above. Furnish a minimum of 25% of the conduit estimated above rounded up to the next 5 foot increment to determine the length of Item 202, Removal Misc.: Conduit. Furnish a minimum of 10% of the above estimated Item 611, Inspection Well for Item 202, Removal Misc.: Inspection Well, rounded up to the next whole number. Furnish 50 cubic yard for Item 203, Embankment, As Per Plan. |
| D111 | UNPERMITTED DRAINAGE DISCHARGE REMOVAL |
|  | AT THE LOCATIONS SHOWN IN THE PLANS, REMOVE THE CONDUIT REGARDLESS OF SIZE TO THE RIGHT OF WAY LINE AND PLUG THE CONDUIT WITH CLASS QC 1 CONCRETE. FOR CONDUIT THAT OUTLETS THROUGH THE CURB, RESTORE THE CURB BY FILLING THE HOLE WITH CLASS QC 1 CONCRETE OR REPLACE THE CURB SECTION. FOR CONDUIT THAT OUTLETS TO A STORM SEWER OR DRAINAGE STRUCTURE, LEAVE 6 INCHES OF THE CONDUIT PROTRUDING. PLUG THE PROTRUDING CONDUIT WITH EITHER A MANUFACTURED CAP OR CLASS QC 1 CONCRETE. FOR CONDUIT THAT OUTLETS TO THE DITCH, REMOVE THE EROSION CONTROL PAD. RESTORE ALL AREAS AS REQUIRED. PLUG THE EXISTING CONDUIT REGARDLESS OF SIZE AT THE RIGHT OF WAY LINE WITH CLASS QC 1 CONCRETE. ALL COSTS ARE INCLUDED IN ITEM 202, REMOVAL MISC. CONDUIT. AT THE LOCATIONS SHOWN IN THE PLANS, DAM THE SWALE THAT OUTLETS TO THE DITCH AT THE R/W AS DIRECTED BY THE ENGINEER. ALL COSTS ARE INCLUDED IN ITEM 203, EMBANKMENT, AS PER PLAN. AT THE LOCATIONS SHOWN IN THE PLANS, REMOVE THE INSPECTION WELL AND RESTORE ALL AREAS AS REQUIRED. THE COST IS INCLUDED IN ITEM 202, REMOVAL MISC. INSPECTION WELL. THE FOLLOWING ESTIMATED QUANTITIES HAVE BEEN INCLUDED IN THE GENERAL SUMMARY: ITEM 202, REMOVAL MISC.: CONDUIT ______ FT ITEM 202, REMOVAL MISC.: INSPECTION WELL ______ EACH ITEM 203, EMBANKMENT, AS PER PLAN ______ CY Designer Note: Use this note when the District R/W Permit Office has identified specific locations for the removal of unpermitted drainage discharges. Furnish the total length of conduit for the length of Item 202 Removal Misc. Conduit. If a conduit size greater than 10 inch is known furnish that size removal item in the plans. Furnish 50 cubic yard for Item 203, Embankment, As Per Plan. Furnish the total number of Inspection Wells required for removal for Item 202, Removal Misc.: Inspection Well. |
| D112 | ITEM 611 - CONDUIT BORED OR JACKED |
|  | WHERE IT IS SPECIFIED THAT A CONDUIT BE INSTALLED BY THE METHOD OF BORING OR JACKING, NO TRENCH EXCAVATION IS PERMITTED WITHIN _____ FEET OF THE (EDGE OF PAVEMENT) (NEAREST RAIL). PROVIDE A STEEL CASING PIPE CONFORMING TO 748.06. JOINTS WITH A CIRCUMFERENCIAL FULLY PENETRATING B-U4b WELD THAT IS PERFORMED BY A CERTIFIED WELDER FOR WELDING CODE AMERICAN WELDING SOCIETY (AWS) D1. 1 OR MACHINED INTERLOCKING JOINTS ARE PERMITTED. THE INSTALLED CASING PIPE IS THE STORM WATER CONVEYANCE CARRIER UNLESS OTHERWISE SPECIFIED IN THE PLANS. HYDROSTATIC TESTING IS NOT REQUIRED FOR THE CASING PIPE. Designer Note: provide the following pay item 611, Conduit Bored or Jacked, _____“, Type _____ , _____ Ft. in the General Summary. Where a conduit is installed by this method under a railroad, coordinate with the Rail Company to determine the allowable distance from the nearest rail and add note D113 to the plans. Extend the casing pipe past the edge of pavement or where access will be limited to the joint. Specify a concrete masonry collar per DM-1.1 between the casing pipe and adjacent conduit material if the casing pipe is used as the final carrier pipe. Follow SGE guidance for exploratory soil borings when considering this method of installation. |
| D113 | ITEM 611 – CONDUIT UNDER RAILROAD |
|  | THE DEPARTMENT WILL PAY TO THE RAIL COMPANY ALL COSTS FOR WATCHMEN OR FLAGGERS DEEMED NECESSARY BY THE RAIL COMPANY DURING THE INSTALLATION OF CONDUIT UNDER THE RAILROAD. ANY COSTS FOR WATCHMEN OR FLAGGERS REQUIRED BY AN ALTERNATE METHOD OF INSTALLATION ARE THE RESPONSIBILITY OF THE CONTRACTOR. THE COSTS FOR WATCHMEN OR FLAGGERS DUE TO THE NEGLIGENCE OF THE CONTRACTOR, OR ANY SUB-CONTRACTOR, IN CONNECTION WITH THE INSTALLATION OF THE CONDUIT MUST BE PAID BY THE CONTRACTOR. TRACK SUPPORTS REQUIRED BY THE RAIL COMPANY IN CONNECTION WITH THE INSTALLATION OF THE CONDUIT ARE INCLUDED IN THE COMPANY FORCE ACCOUNT WORK AND PAID BY THE DEPARTMENT. THE COST OF ANY TRACK SUPPORTS REQUIRED BY AN ALTERNATE METHOD OF INSTALLATION OF CONDUIT ARE THE RESPONSIBILITY OF THE CONTRACTOR. THE CONTRACTOR IS RESPOSIBLE TO SECURE APPROVAL OF OPERATIONS FROM THE DEPARTMENT AND THE RAIL COMPANY. THE RAIL COMPANY WILL PERFORM AN ENGINEERING REVIEW OF METHODS OF OPERATIONS AND ENGINEERING SUPERVISION OF CONSTRUCITON WITHOUT COST TO THE CONTRACTOR. PRIOR TO BIDDING, COORDINATE WITH THE RAIL COMPANY TO AGREE UPON THE REQUIREMENTS OF WATCHMEN AND FLAGGERS TO PROTECT RAILROAD TRAFFIC DURING THE CONTRACTOR’S OPERATIONS. EXECUTE A BOND IN FAVOR OF BOTH THE STATE AND THE COMPANY AS REQUIRED BY SECTION 5525.16 OF THE REVISED CODE OF OHIO. COORDINATE WITH THE RAIL COMPANY CONCERNING WORK ADJACENT TO RAILROAD TRACKS, IN ORDER TO AVOID DELAY TO, OR INTERFERENCE WITH RAILROAD TRAFFIC, AND NOTIFY THE RAIL COMPANY ____ HOURS IN ADVANCE OF CONSTRUCTION OPERATIONS. Designer Note: Provide this note when placing pipe culverts, sewers, or water lines under railroads. Coordinate with the rail company to determine the number of hours required for pre-construction notification. |
| D114 | REVIEW OF DRAINAGE FACILITIES |
|  | PRIOR TO THE START OF WORK AND AGAIN BEFORE FINAL ACCEPTANCE, PERFORM AN INSPECTION WITH REPRESENTATIVES OF THE DEPARTMENT, CONTRACTOR AND LOCALS OF ALL EXISTING DRAINAGE FACILITIES THAT ARE TO REMAIN IN SERVICE WHICH MAY BE AFFECTED BY THE WORK. THE CONDITION OF THE EXISTING CONDUITS AND THEIR APPURTENANCES IS DETERMINED FROM FIELD OBSERVATIONS. RECORDS OF THE INSPECTION ARE MAINTAINED BY THE DEPARTMENT. CONFIRM ALL EXISTING SEWERS INSPECTED INITIALLY BY THE ABOVE-MENTIONED PARTIES ARE MAINTAINED AND LEFT IN A CONDITION COMPARABLE TO THAT DETERMINED BY THE ORIGINAL INSPECTION. THE CONTRACTOR IS RESPONSIBLE TO CORRECT ANY CHANGE IN THE CONDITION RESULTING FROM THEIR OPERATIONS AS DIRECTED AND APPROVED BY THE ENGINEER. PAYMENT FOR ALL OPERATIONS DESCRIBED ABOVE IS INCLUDED IN THE CONTRACT PRICE FOR THE PERTINENT 611 CONDUIT ITEMS. Designer Note: Provide this note on projects where existing drainage facilities are to remain in service. |
| D117 | MANHOLES, CATCH BASINS AND INLETS REMOVED OR ABANDONED |
|  | CAREFULLY REMOVE AND STORE ALL CASTINGS WITHIN THE RIGHT OF WAY FOR SALVAGE BY (DEPARTMENT) (CITY) (VILLAGE) (COUNTY) FORCES. PAYMENT FOR ALL OF THE ABOVE IS INCLUDED IN THE CONTRACT PRICE FOR THE PERTINENT 202 ITEM. Designer Note: Only use this note when the owner desires to retain the existing castings. |
| D119 | ITEM SPECIAL- MISCELLANEOUS METAL |
|  | EXISTING CASTINGS MAY PROVE TO BE UNSUITABLE FOR REUSE, AS DETERMINED BY THE ENGINEER. THE CONTRACTOR IS RESPONSIBILE TO PROVIDE THE CASTINGS OF THE REQUIRED TYPE, SIZE AND STRENGTH (HEAVY OR LIGHT DUTY) FOR THE PARTICULAR STRUCTURE IN QUESTION. FURNISH MATERIALS PER 611 WITH PRIOR APPROVAL OF THE ENGINEER. THE FOLLOWING ESTIMATED QUANTITY HAS BEEN CARRIED TO THE GENERAL SUMMARY FOR USE AS DIRECTED BY THE ENGINEER. ITEM SPECIAL, MISCELLANEOUS METAL ______ LB REPLACE EXISTING CASTINGS DAMAGED BY CONTRACTOR NEGLIGENCE, AS DETERMINED BY THE ENGINEER, AT THE EXPENSE OF THE CONTRACTOR. Designer Note: Use this note if existing castings are to be reused and which may be unsuitable. |
| D120 | ITEM 611 - ( )”, SLOTTED DRAIN, TYPE ( ) |
|  | THIS ITEM CONSISTS OF ____ INCH DIAMETER SLOTTED DRAIN ALUMINUM COATED STEEL CONDUIT 707.01 WITH 6 INCH TRAPEZOIDAL GALVANIZED SOLID BAR GRATE AS APPROVED BY THE ENGINEER. ALL COSTS FOR LABOR AND MATERIALS, INCLUDING TYPE 2 BEDDING, AND BACKFILLING AS DETAILED ON STANDARD CONSTRUCTION DRAWING DM-1.3 IS INCLUDED IN THE PRICE BID PER FT FOR ITEM 611, ____ “ SLOTTED DRAIN,TYPE ____ . Designer Note: Use this note in conjunction with Standard Construction Drawing DM-1.3. Outlet slotted drain pipe into a catch basin. |
| D121 | ITEM SPECIAL - PIPE CLEANOUT |
|  | THIS WORK CONSISTS OF REMOVING SEDIMENT AND DEBRIS FROM THE EXISTING DRAINAGE CONDUITS SPECIFIED IN THE PLANS. DISPOSE OF ALL MATERIAL PER 105.16 AND 105.17. CLEAN OUT TO THE APPROVAL OF THE ENGINEER. CLEANOUT OF THE PIPE IS PAID FOR AT THE UNIT PRICE BID FOR ITEM SPECIAL, PIPE CLEANOUT. THIS PRICE INCLUDES THE COST FOR MATERIAL, EQUIPMENT, LABOR, AND ALL INCIDENTALS REQUIRED TO COMPLETE THE CLEANOUT. THE FOLLOWING ESTIMATED QUANTITIES HAVE BEEN INCLUDED IN THE GENERAL SUMMARY FOR THE WORK NOTED ABOVE: ITEM SPECIAL, PIPE CLEANOUT, 24” AND UNDER ______ FT ITEM SPECIAL, PIPE CLEANOUT, 27” TO 48” ______ FT ITEM SPECIAL, PIPE CLEANOUT, OVER 48” ______ FT Designer Note: This item may not be eligible for federal participation. |
| D123 | EXISTING SUBSURFACE DRAINAGE |
|  | PROVIDE UNOBSTRUCTED OUTLETS FOR ALL EXISTING UNDERDRAINS OR AGGREGATE DRAINS ENCOUNTERED DURING CONSTRUCTION. PROVIDE AN OUTLET PER STANDARD CONSTRUCTION DRAWING DM-1.1 FOR ALL UNDERDRAINS THAT OUTLET TO A SLOPE. UNDERDRAINS THAT CAN BE CONNECTED TO THE NEW OR EXISTING UNDERDRAINS AT THE END OF THE PROJECT LIMITS AS WELL AS ALL NECESSARY BENDS OR BRANCHES REQUIRED FOR CONNECTION ARE INCLUDED IN THE BASIS OF PAYMENT FOR UNCLASSIFIED PIPE UNDERDRAINS. THE FOLLOWING ESTIMATED QUANTITIES HAVE BEEN INCLUDED IN THE GENERAL SUMMARY FOR THE WORK NOTED ABOVE: ITEM 601, TIED CONCRETE BLOCK MAT, TYPE 1 ______ SY ITEM 605, AGGREGATE DRAINS ______ FT ITEM 611, ____” CONDUIT, TYPE F ______ FT ITEM 611, PRECAST REINFORCED CONCRETE OUTLET ______ EACH ITEM 605, ____” UNCLASSIFIED PIPE UNDERDRAINS ______ FT Designer Note: Use the note on projects if there are existing underdrains or aggregate drains within the project limits that are to remain. Perform an investigation for the presence of existing underdrain outlet locations or potential conflict areas within the project limits and show them on the plan view sheets. |
| D124 | TEMPORARY DRAINAGE ITEMS |
|  | TEMPORARY DRAINAGE ITEMS LABELED ON THE MAINTENANCE OF TRAFFIC PLAN ARE ITEMIZED ON THE MOT PLANS AND CARRIED TO THE GENERAL SUMMARY. Designer Note: Provide this note when temporary drainage items are required per section 1009 of this manual. Furnish drainage items for each phase of the maintenance of traffic operations. Removal items may be required between individual phases. Utilize drainage structures furnished for final drainage design where feasible. |
| E101 | SEEDING AND MULCHING |
|  | THE FOLLOWING QUANTITIES ARE PROVIDED TO PROMOTE GROWTH AND CARE OF PERMANENT SEEDED AREAS: ITEM 659, SOIL ANALYSIS TEST ______ EACH ITEM 659, TOPSOIL ______ CY ITEM 659, SEEDING AND MULCHING ______ SY ITEM 659, REPAIR SEEDING AND MULCHING ______ SY ITEM 659, INTER-SEEDING ______ SY ITEM 659, COMMERCIAL FERTILIZER ______ TON ITEM 659, LIME ______ ACRES ITEM 659, WATER ______ MGAL ITEM 659, MOWING ______ MSF APPLY SEEDING AND MULCHING S TO ALL AREAS OF EXPOSED SOIL BETWEEN THE RIGHT-OF-WAY LINES AND WITHIN THE CONSTRUCTION LIMITS FOR AREAS OUTSIDE THE RIGHT-OF-WAY LINES COVERED BY WORK AGREEMENT OR SLOPE EASEMENT. QUANTITY CALCULATIONS FOR SEEDING AND MULCHING ARE BASED ON THESE LIMITS. Designer Note: Use the above quantities on all projects that require grading work. The following is a basic guideline for estimating quantities for the above items. These quantities may be omitted from the note if they are itemized elsewhere in the plans. Show calculations for all items in the plans. 659, Soil Analysis Test (EACH) - Soil Analysis Tests are used to field adjust the rate of Lime based on soil conditions. - Soil Analysis Test is not specified. Use the standard rate for Lime without adjustment. - Soil Analysis Test is specified. If specified, minimum of two tests. If no Topsoil to be placed - One test per 10 Acres (one test per 48400 Sq. Yd.) of permanent seeded area and sodded area. If placing Topsoil - One test per 10000 Cu. Yds. of Topsoil. 659, Topsoil (CY) - 111 Cu. Yds. per 1000 Sq. Yd. of permanent seeded area. Topsoil is optional. However, it is recommended, especially for projects involving A4 silty materials, granular embankment or granular materials due to severe erosion problems. 659, Seeding and Mulching (SY) - This quantity is usually calculated by the end width method using the cross sections. On short projects, seeding quantities may be determined by other methods. For example, the area for seeding may be estimated by calculating an area per Plan & Profile sheet determined by multiplying an average width (based on construction limits or right-of -way lines) by the distance on each sheet, and then deducting for paved surface areas. A deduction should be taken for 660 and 670 items. 659, Repair Seeding and Mulching (SY) - 5 % of the permanent seeding and mulching area. 659, Inter-seeding (SY) - 5% of the permanent seeding and mulching area. 659, Commercial Fertilizer (TON) - 30 pounds per 1000 Sq. Ft. (one Ton per 7410 Sq. Yd.) of permanent seeded area. This rate includes 20 pounds per 1000 Sq. Ft. for the first application and 10 pounds per 1000 Sq. Ft. for the second application. If Inter-seeding is provided, use an additional 20 pounds per 1000 Sq. Ft. of commercial fertilizer for the Inter-seeding area. 659 Lime (ACRE) - Apply over permanent seeded area. 659, Water (MGAL) - Two applications each at 300 Gallons per 1000 Sq. Ft. (0.0027 M Gallons per Sq. Yd.) of permanent seeded area. The above rate is for a single application. If Inter-seeding is provided, use an additional 300 Gallons per 1000 Sq. Ft. of water for the Inter-seeded area. 659, Mowing (MSF) - 25 % of the permanent seeded area for projects expected to last more than one construction season. |
| E102 | SODDING |
|  | THE FOLLOWING QUANTITIES ARE PROVIDED TO PROMOTE GROWTH AND CARE OF PERMANENT SODDED AREAS. ITEM 659, SOIL ANALYSIS TEST ______ EACH ITEM 659, TOPSOIL ______ CY ITEM 659, COMMERCIAL FERTILIZER ______ TON ITEM 659, LIME ______ ACRE ITEM 659, WATER ______ MGAL ITEM 660, SODDING, UNSTAKED, STAKED, REINFORCED ______ SY Designer Note: Provide the above quantities on all projects that have pay item(s) for permanent sodding. The following is a basic guideline for estimating quantities for the above items. These quantities may be omitted from the note if they are itemized elsewhere in the plans. Show calculations for all items in the plans. 659, Soil Analysis Test (EACH) - Soil Analysis Tests are used to field adjust the rate of Lime based on soil conditions. - Soil Analysis Test is not specified. Use the standard rate for Lime without adjustment. - Soil Analysis Test is specified. If specified, minimum of two tests. If no Topsoil to be placed - One test per 10 Acres (one test per 48400 Sq. Yd.) of permanent seeded area and sodded area. If placing Topsoil - One test per 10000 Cu. Yds. of Topsoil. 659, Topsoil (CY) - 111 Cu. Yds. per 1000 Sq. Yd. of permanent seeded area. Topsoil is optional. However, it is recommended, especially for projects involving A4 silty materials, granular embankment or granular materials due to severe erosion problems. 659, Commercial Fertilizer (TON) - 30 pounds per 1000 Sq. Ft. (one Ton per 7410 Sq. Yd.) of permanent seeded area. This rate includes 20 pounds per 1000 Sq. Ft. for the first application and 10 pounds per 1000 Sq. Ft. for the second application. If Inter-seeding is provided, use an additional 20 pounds per 1000 Sq. Ft. of commercial fertilizer for the Inter-seeding area. 659 Lime (ACRE) - Apply over permanent seeded area. 659, Water (MGAL) - Two applications each at 300 Gallons per 1000 Sq. Ft. (0.0027 M Gallons per Sq. Yd.) of permanent seeded area. The above rate is for a single application. If Inter-seeding is provided, use an additional 300 Gallons per 1000 Sq. Ft. of water for the Inter-seeded area. 660, Sodding (SY) - This is the actual number of Sq. Yds. of permanent sodded area. |
| W100 | pOST cONSTRUCTION STORM WATER treatment |
|  | THIS PLAN UTILIZES STRUCTURAL BEST MANAGEMENT PRACTICES (BMP’S) FOR POST CONSTRUCTION STORM WATER TREATMENT. Designer Note: Use this note on all projects that have post construction storm water management BMP’s. Follow this note by the notes below if applicable. |
| W101 | BIORETENTION cELL(s) |
|  | construct The bioretention cell(s) AFTER all contributing drainage areas are stabilized as shown on the Contract Plans. DO NOT OPERATE Heavy equipment within the perimeter of a bioretention CELL. Use all suitable excavated material in the work. Alternatively, legally use, recycle, or dispose of all excavated materials according to 105.16 and 105.17. excavate The bioretention cell to the dimensions, WITH VERTICAL sideS, TO THE elevations SPECIFITIED. minimize The compaction of the bottom of the bioretention CELL. EXCAVATION WILL be measured and paid as item 203, EXCAVATION, as per PLAN. tHE BIORETENTION CELL CONSISTs OF FOUR DiSCRETE LAYERS: BIORETENTION PLANTING SOIL LAYER, fINE aGGREGATE LAYER, COARSE AGGREAGE NO. 78 LAYER, AND COARSE AGGREGATE NO. 57 LAYER AND AN UNDERDRAIN SYSTEM. THE MATERIALS AND VOLUMES FOR EACH LAYER ARE AS SHOWN: BIORETENTION CELL; PROJECT QUANTITY (CY). BIORETENTION PLANTING soil LAYER PLUS 3 INCH COVER. cOMPOSITION BY VOLUME. 5; PARTS SAND – CMS FINE AGGREGATE PER 703.20. 1; PART TOPSOIL – CMS 659.05. 2; PARTS COMPOST – CMS 659.06. FINE AGGREGATE PER CMS 703.20. Coarse aggreage size NO. 78 PER 703.20. coarse aggreage size no. 57 PER 703.20. TOTAL CUBIC YARDS. CONSTRUCT The underdrain system AS SPECIFIED. PLACE THE BIORETENTION PLANTING SOIL IN 12 INCH LIFTS. the bioretention planting soil LAYER PLUS 3 INCH COVER IS 3 inches greater than the depth SPECIFIED to account for expected settling of the uncompacted soil. The bioretention PLANTING soil must be a uniform mix that is free of stones, stumps, roots, or any other object larger than two inches. The soil may consist of existing soil, furnished soil, or a combination of both provided that THE PH IS BETWEEN 5.2 – 8.0 and meets the composition requirements listed above. Phosphorus concentrations OF the planting soil must fall between 15 and 60 mg/kg (ppm) aNd determined by the mehlich iii test. THOROUGHLY mix The bioretention PLANTING soil prior to placement. place observation well and cleanout where SPECIFIED. CONNECT The observation well and cleanout to the perforated underdrain with the appropriate manufactured connections. extend The observation well and cleanout 4 inches above the SURFACE elevation. CAP THE observation well and cleanout with a threaded screw cap. CAP The ends of perforated underdrain pipes not terminating in an observation well and cleanout or connected to other conduits. PLACE seed, turf, Trees, shrubs, or other plant materials for Bioretention Facilities as specified. Plant materials will be measured and paid for per CMS Item(s) 659, 660, or 661 depending on the plant materials SPECIFIED. APPLY NO Pesticides, herbicides, LIME, and fertilizers. install ITEM 611 AS SPECIFIED. INSTALL TEMPORARY EROSION CONTROL MAT TYPE A, B, C, OR E PER cms 671 with either straw mulch or compost OR AS SPECIFIED IN THE PLANS. Bioretention Cells ARE paid for as ITEM 601, Bioretention Cell, cy AND ITEM 601, TIED CONCRETE MAT, SY. EXCAVATION FOR BIORETENTION CELLS is FOR VERTICAL SIDES ONLY, AS SPECIFIED AND PAID FOR AS ITEM 203, EXCAVATION, as per plan, CY. PERFORATED UNDERDRAINS, OBSERVATION WELLS, AND ASSOCIATED FITTINGS AND COUPLERS are PAID FOR AS ITEM 605, UNDERDRAIN, as per plan. nON PEFORATED OUTLET PIPES FOR are PAID FOR AS ITEM 611. sEEDING AND MULCHING for the bioretentioN cell are paid for as item 659 seeding and mulching, sy EROSION CONTROL MATS are PAID FOR AS ITEM 671, EROSION CONTROL MATS, SY. Designer Note: Use this note on all projects that have bioretention cell(s) identified in the plan. Add a note that states: “ITEM 203, EXCAVATION, AS PER PLAN, VERTICAL SIDES ONLY” on plan sheets showing bioretention cell cross sections. |
| W102 | infiltration trench (or basin) |
|  | THIS PLAN UTILIZES INFILTRATION FOR POST CONSTRUCTION STORM WATER TREATMENT. CONSTRUCT The completed INFILTRATION TRENCH(Es) (and or Basin(s)) AFTER all contributing drainage areas are stabilized as shown in the Contract Plans and to the satisfaction of the Engineer. DO NOT USE INFILTRATION devices as temporary sediment control facilities during construction. DO NOT OPERATE Heavy equipment within the perimeter of aN INFILTRATION device during excavation OR backfilling of the facility. Designer Note: Use this note on all projects that have infiltration trenches and or basins identified in the plan. Embankment work to create the impoundment will be constructed and paid for as Item 203 Embankment, using natural soils, 703.16.A. |
| W103 | Manufactured Water Quality Structure |
|  | This plan utilizes manufactured water quality structures for water quality treatment. areas ARE shown in the plans for placement of an off-line system. Payment for these devices is made at the contract unit price for Item 895, Manufactured Water Quality Structure, Type ____. Designer Note: Use this note on all projects that have manufactured water quality structures identified in the plan. If more than one manufactured water quality structure is provided in the plans, provide a table to indicate the location and type of each structure used. Supplemental specification 895 outlines the different types of structures (1-4). Manufactured systems may not be installed under the roadway or downstream of a connecting pipe more than ten feet deep without approval OHE. |
| W104 | Vegetated filter strip |
|  | THIS PLAN UTILIZES vegetated filter strip(s) FOR POST CONSTRUCTION STORM WATER TREATMENT. Place either Item 660 SODding or ITEM 659 SeeDing and Mulching with a 4-inch lift of topsoil and item 670, slope erosion protection to All disturbed areas designated as vegetated filter strips, the edge of shoulder, and the foreslope as specified in the plans. Designer Note: Use this note on all projects that have vegetated filter strips identified in the plan. Pay for grass planting and topsoil as Item 659 or Item 660 and include with quantities for the rest of the project. Pay for erosion control mat as Item 670, slope erosion protection and include with quantities for the rest of the project. |
| W105 | Vegetated biofilter |
|  | THIS PLAN UTILIZES vegetated biofilter(s) FOR POST CONSTRUCTION STORM WATER TREATMENT. place either item 660 sodding or item 659 seeding and mulching with a 4-inch lift of topsoil as shown in the plans to any disturbed area on the shoulder and foreslope draining to a vegetated biofilter. The ditch for each vegetated biofilter is trapezoidal, as shown in the plan cross sections. Provide Item 670 as Specified in the plans. Designer Note: Use this note on all projects that have vegetated biofilters identified in the plan. Pay for grass planting and topsoil as Item 659 or Item 660 and include with quantities for the rest of the project. Pay for erosion control mat as Item 670, ditch erosion protection and include with quantities for the rest of the project. |
| W106 | Extended detention basin |
|  | THIS PLAN UTILIZES extended detention basin(s) for POST CONSTRUCTION STORM WATER TREATMENT. detention basins may be used as sediment control devices during construction. Following stabilization of the tributary area, CONFIRM THE final grading of the detention basin matchES the plans. REMOVE The detention basin outlet structure for construction sediment control and CONFIRM the outlet structure matchES the design shown in the plans. Designer Note: Use this note on all projects that have extended detention basins identified in the plan. This note may be modified for retention basins or constructed wetlands, if those are included in the plans. |