# General Information

General Information - Table of Contents

Purpose	1

Application	2

Format	2

Preparation	2

Revisions	3

Map of Districts	4

OHIO Counties	6

Glossary of Terms	8

Abbreviations	15

References	19

## Purpose

This Drainage Design Manual has been prepared as a guide for the hydraulic design of highway drainage facilities. Drainage criteria and design outlined in this manual reflect the maximum standard achievable that is based on new project development for traditional design, bid, and build projects. Existing conditions represent the minimum standard, which should always be evaluated against the cost of achieving the maximum standard and the purpose and need of the project. The goal is to make improvements proportionate with the purpose and need of the project at minimized project costs without negative impacts to safety. Coordinate any proposed deviations from the maximum standard with the Department prior to incorporation into the design. Drainage facilities account for approximately one quarter of the total construction cost for most roadway projects. This cost justifies a careful and scientific hydraulic analysis.

## Application

Design drainage facilities following the recommended design procedures noted in this manual to minimize the following: • Damage of private property due to flooding • Inconvenience to the motorist during moderate to heavy rainfall • Disturbance to the environment Drainage Design Aid Charts have been assembled to assist the Drainage Design Engineer with the hydraulic analysis. Other design charts are available in Hydraulic Engineering Circulars and Hydraulic Design Series prepared by the Federal Highway Administration. Reference is made to those charts as required. This manual is not a substitute for engineering knowledge, experience, or judgment. Its purpose is to provide uniform procedures for implementing drainage design decisions and assure quality and continuity in drainage of highways in Ohio. Although the manual is considered the primary source of reference for drainage design in Ohio, some of the suggested practices may be inappropriate for projects because of fiscal limitations or other justifiable reasons. Consideration must also be given to reasonable hydraulic design standards adopted by city, county, or other local governments when designing facilities under their jurisdiction.

## Format

Sections 1000 and 1100 of the LD2 are written in a two-column format. The left column represents ODOT drainage design specifications, and the right column represents commentary. The commentary may include background information, design guidance or other supplemental information related to a corresponding specification.

## Preparation

The Drainage Design Manual has been developed by the Office of Hydraulic Engineering. Errors or omissions should be reported to the Administrator, Office of Hydraulic Engineering. A list of contacts can be found on the OHE website.

## Revisions

Updating the manual is intended to be a continuous process. Revisions are issued periodically by OHE and will be available at the ODOT Publications Webpage. All revisions are shown in highlighted text. The date of the revision is shown in the document header.

## Map of Districts

## OHIO Counties

County; Code; District; County; Code; District. Adams Allen Ashland Ashtabula Athens Auglaize Belmont Brown Butler Carroll Champaign Clark Clermont Clinton Columbiana Coshocton Crawford Cuyahoga Darke Defiance Delaware Erie Fairfield Fayette Franklin Fulton Gallia Geauga Greene Guernsey Hamilton Hancock Hardin Harrison Henry Highland Hocking Holmes Huron Jackson Jefferson Knox Lake Lawrence; ADA ALL ASD ATB ATH AUG BEL BRO BUT CAR CHP CLA CLE CLI COL COS CRA CUY DAR DEF DEL ERI FAI FAY FRA FUL GAL GEA GRE GUE HAM HAN HAR HAS HEN HIG HOC HOL HUR JAC JEF KNO LAK LAW; 9 1 3 4 10 7 11 9 8 11 7 7 8 8 11 5 3 12 7 1 6 3 5 6 6 2 10 12 8 5 8 1 1 11 2 9 10 11 3 9 11 5 12 9; Licking Logan Lorain Lucas Madison Mahoning Marion Medina Meigs Mercer Miami Monroe Montgomery Morgan Morrow Muskingum Noble Ottawa Paulding Perry Pickaway Pike Portage Preble Putnam Richland Ross Sandusky Scioto Seneca Shelby Stark Summit Trumbull Tuscarawas Union Van Wert Vinton Warren Washington Wayne Williams Wood Wyandot; LIC LOG LOR LUC MAD MAH MAR MED MEG MER MIA MOE MOT MRG MRW MUS NOB OTT PAU PER PIC PIK POR PRE PUT RIC ROS SAN SCI SEN SHE STA SUM TRU TUS UNI VAN VIN WAR WAS WAY WIL WOO WYA; 5 7 3 2 6 4 6 3 10 7 7 10 7 10 6 5 10 2 1 5 6 9 4 8 1 3 9 2 9 2 7 4 4 4 11 6 1 10 8 10 3 2 2 1.

## Glossary of Terms

Aggregate Drain – A trench filled with granular material extending laterally from the pavement base or subbase layer to an outlet on the roadway foreslope with the intent of draining surface and/or ground water away from the pavement base and/or subbase.

Annual Exceedance Probability (AEP) – The probability that an event will occur, or be exceeded, in any given year, normally given in percentage.

Anti-seep Collar – Device that prevents the flow of water through the surrounding soil around a conduit that is used as an outlet for an infiltration, retention, or detention basin.

Apron – Paving at a pipe inlet or outlet, or upstream of a catch basin, constructed along the channel bottom to prevent scour.

Backwater Analysis – The determination of water surface profiles measured at specific locations upstream from a constriction causing an increased flow depth upstream.

Bankfull Discharge – The flow or stage of a stream corresponding to the highest level of active deposition. It is the discharge that, on average, fills a main channel to the point of overflowing. For simplicity, it is generally considered to be approximately the 50% AEP discharge.

Bankfull Width – The width of a stream or river channel at the elevation where it typically overflows into the floodplain, also known as the channel-forming flow

Base Flood – (FEMA) The flood having a one percent chance of being equaled or exceeded in any given year. This is the regulatory standard also referred to as the "100-year flood" or the “1% AEP flood.” The base flood is the national standard used by the National Flood Insurance Program (NFIP) and all Federal agencies for the purposes of requiring the purchase of flood insurance and regulating new development. Base Flood Elevations (BFEs) are typically shown on Flood Insurance Rate Maps (FIRMs)

Base Flood Elevation – (FEMA) The computed elevation to which floodwater is anticipated to rise during the base flood. Base Flood Elevations (BFEs) are shown on Flood Insurance Rate Maps (FIRMs) and on the flood profiles.

Body of Water – A body of water is any significant accumulation of water, generally on a planet's surface. The term most often refers to large rivers, and lakes, but it includes smaller pools of water such as ponds, or wetlands.

Bridge – Structure that has a span greater than or equal to 10 feet as measured in a parallel direction to the roadway centerline.

Camber – A slight convex curve constructed into the bottom of a pipe to overcome anticipated settlement problems.

Cast-in-place Structure – A concrete drainage structure which is placed in forms and cured at its final location. Precast beams on cast-in-place foundations are considered cast-in-place structures.

Catch Basin – A structure for intercepting flow from a gutter or ditch and discharging the water through a conduit.

Channel – The bed and banks that confine the surface flow of a waterway under normal flow conditions.

Coefficient of Runoff (C) – A value, varying with the ground and ground cover, which is used in the Rational formula to determine the amount of a rainfall which is directed to streams and not absorbed into the ground.

Conduit – A closed structure such as a pipe that has a span less than 10 feet as measured in a parallel direction to the roadway centerline.

Corner Bearing Pressure – The pressure generated at the corners of pipe arch structures.

Cover – Distance measured from the top of the conduit vertically upward.

Culvert – A structure used to convey surface runoff through embankments, typically designed hydraulically to take advantage of submergence at the inlet to increase hydraulic capacity. A structure, as distinguished from a bridge, which is usually covered with embankment and is composed of structural material around the entire perimeter, although some are supported on footings with the stream bed serving as the bottom.

Cutoff Wall – A wall that extends downward from the end of a structure to below the expected scour depth, or to a scour-resistant material.

Design Discharge (Q) – The peak rate of flow for which a drainage facility is designed. Usually given in cubic feet per second (cfs).

Design Service Life – The average usable life of a pipe or structure. A conduit is conserved to have reached the end of its service life when the General Appraisal rating is equal to 4.

Design Storm – A given rainfall amount, areal distribution, and a time distribution, used to estimate runoff. The rainfall amount is either a given storm (10% AEP, 4% AEP, 2% AEP, etc.) or a specific large value.

Detention Basin – A structure that holds water for a short period of time before releasing it to the natural water course.

Discharge – The volume of water that passes through a given cross-section per unit time; commonly measured in cubic feet per second (cfs).

Diversion Dike – An embankment to control or to deflect water away from a soil bank.

Drainage Area – The total surface area contributing discharge to a channel or catchment at a given point.

Drainage Basin – See Watershed.

Drop-down Entrance (Drop inlet) – A type of inlet which conveys the water from a higher elevation to a lower elevation smoothly without a free fall at the inlet.

Elliptical Pipe – Pipe which is manufactured with a span greater than rise to be utilized in shallow cover situations.

Ephemeral Stream – A stream or reach of stream that does not flow for parts of the year. As used here, the term includes intermittent streams with flow less than perennial. It is located above the water table year-round. Ground water is not a source of water supply.

Feasible – Term used to define BMP practicability. A BMP must be: technically feasible, implemented within the procured highway right-of-way, safe for the traveling public and ODOT maintenance personnel, cost effective as compared to the benefit, and will be legal at the State, Federal, and Local levels.

Flood Fringe or Floodway Fringe – The portion of the floodplain outside of the floodway.

Flood Hazard Evaluation – The act of determining if flood levels within a watercourse for a 100-year flood, or other recurrence interval floods have a significantly increased detrimental impact on upstream property.

Flood Insurance Rate Map (FIRM) – The official map of a community on which FEMA has delineated both the special hazard areas and the risk premium zones applicable to the community.

Flood Insurance Study (FIS) – A book with information regarding flooding in a community that is developed in conjunction with the FIRM. It discusses the engineering methods used to develop the FIRMs.

Floodplain – An area of land adjacent to a watercourse which experiences flooding during periods of high discharge.

Floodplain Culverts – Relief culverts that are placed in addition to a bankfull culvert at a higher elevation across the floodplain to allow multiple outlets for floodwaters.

Floodplain Study – A more extensive analysis of the effects of flood levels on upstream property than the Flood Hazard Evaluation. This analysis is to be used when upstream properties appear to have been subjected to a significantly increased detrimental effect from the flood flows.

Floodway – The channel and portion of the floodplain, which is effective in carrying flow, within which this carrying capacity must be preserved and where the flood hazard is generally highest.

Flowline – see Thalweg

Forebay – Depressed area that offers pretreatment of sediment laden storm water prior to a retention, detention, or infiltration basin.

Freeboard – the vertical clearance of the lowest structural member of a bridge superstructure above the water surface elevation.

Friction Slope – The slope of the energy grade line.

Granular Material – A term relating to the uniform size of grains or crystals in rock, larger than sand or pea gravel.

Grate – A type of screen made from sets of bars used to allow the interception of flow and to cover an area for pedestrian or vehicular traffic.

Headwall – The structural appurtenance placed at the open end of a pipe to control an adjacent highway embankment and protect the pipe end from undercutting.

Headwater – That depth of water impounded upstream of a culvert due to the influence of the culvert constriction, friction, and configuration.

Height of Cover – Distance measured from the top of the conduit vertically upward to the pavement surface or finished grade for conduits not under pavement.

Highest Known Water Elevation – The highest known flood water in record.

Hydraulic Grade Line – A line coinciding with the level of flowing water in an open channel. In a closed conduit operating under pressure, a line representing the distance water would rise in a pitot tube at any point along a pipe. The hydraulic grade line is equal to the pressure head (P/γ) along the pipe.

Hydraulic Gradient – The slope of the hydraulic grade line for a storm sewer or culvert.

Hydraulics – The science concerned with the behavior and flow of liquids, especially in conduits and open channels.

Hydrology – The science concerned with the occurrence, distribution and circulation of water on the earth, including precipitation, runoff and groundwater.

Idealized Channel Geometry – Physical, geometric, and hydraulic characteristics of a channel determined from empirical relationships.

Impervious Surface – Hardened pavement surface.

Infiltration Rate – The rate at which water penetrates the surface of the soil at any given instant. The rate can be limited by the infiltration capacity of the soil or the rate at which water is applied.

Inlet – A structure for capturing concentrated surface flow. May be located along the roadway, in a gutter, in the highway median, or in the field.

Inlet Control – The situation where the culvert hydraulic performance is controlled by the entrance geometry only.

Intermittent Stream – A stream that is dry for part of the year, ordinarily more than 3 months.

Manhole – A structure by which one may access a closed drainage system.

Micropool –Depressed area providing a settling pool located at the water quality outlet structure of a detention basin.

MS4 Phase II Regulated Area – Area that has been designated by the Ohio EPA that requires a storm water management plan to discharge storm water.

Multiple Cell Culvert – A culvert with more than one barrel.

New Development Project – Projects that change the land use of a site from undeveloped to developed characteristics.

Ordinary High Water Mark – The line on the shore established by the fluctuation of water and indicated by physical characteristics such as: a clear natural line impressed on the bank, shelving, changes in the character of soil, destruction of terrestrial vegetation, or other appropriate means that consider the characteristics of the surrounding areas. This elevation is lower than the highest known water.

Outlet Control – The situation where the culvert hydraulic performance is determined by the controlling water surface elevation at the outlet, the slope, length and roughness of the culvert barrel, as well as the entrance geometry.

Overland Flow – Water which travels over a surface and reaches a stream.

Perennial Stream – A stream that flows continuously for all or most of the year. The water table is located above the stream bed for most of the year.

Permeability – The quality of the soil that enables water to move downward through the soil profile. It is measured in units of inches per hour.

pH – The reciprocal of the negative logarithm of the Hydrogen ion concentration. Neutral water has a pH value of 7. A measure of the acidity of a substance, if less than 7; alkalinity if greater than 7.

Pipe Arch – Pipe which is manufactured with a span greater than rise (semicircular crown, small-radius corners, and large radius invert) to be utilized in shallow cover situations.

Pipe Underdrain – A longitudinal subsurface drainage system composed of a perforated pipe at the bottom of a narrow trench filled with permeable material and lined with a geotextile in erodible soils, with the intent of draining surface and/or ground waters away from the pavement base and/or subbase.

Porosity – The volume of voids divided by the total volume and multiplied by 100.

Prefabricated Edge Drain – A longitudinal underdrain system utilizing a narrow trench and a vertically elongated, perforated water carrier with the intent of draining surface and/or ground water away from the pavement base and/or subbase.

Prefabricated Structure – Any drainage structure which is manufactured off site and transported to the location of intended use. It may be of various materials, including concrete, clay, metal, thermoplastics, etc. It may be of various shapes including circular, elliptical, rectangular, arched, etc.

Premium Joints – Watertight joints.

Pretreatment – Preliminary filtering of sediment laden storm water prior to secondary treatment through a structural best management practice.

Rainfall Intensity (i) – The amount of rainfall occurring in a unit of time, normally given in inches per hour.

Reference Reach – A length of channel with stable geometric, physical, and hydraulic characteristics. A representation of the desired outcome of a restored channel.

Regulatory Floodway – (FEMA) The channel of a river or other watercourse and the adjacent land areas that must be reserved in order to discharge the base flood without cumulatively increasing the water surface elevation more than a designated height.

Retention Basin – A structure that holds water on a permanent basis.

Roughness Coefficient (n) – The measure of texture on the surface of channels and conduits. Usually represented by the n-value coefficient used in Manning’s open channel flow equation.

Runoff – That part of the precipitation which runs off the surface of a drainage area after all abstractions are accounted for.

Sanitary Sewer – A conduit or pipe system which carries household and/or industrial wastes. Sanitary sewers do not convey storm water.

Scupper – A device used to drain water through a bridge deck, parapet or barrier.

Sediment Basin – A basin or tank in which stormwater containing settleable solids is retained, to remove by gravity or filtration a part of the suspended matter.

Sediment Dam – A dam that is designed to allow suspended sediment to settle out of flowing water in a controlled area.

Short-circuiting – The act of storm water bypassing the intended route.

Soil Bioengineering – The use of live and dead plant materials, in combination with natural and synthetic support materials, for slope stabilization, erosion reduction, and vegetative establishment.

Spring Line – The locus of the horizontal extremities of a transverse section of a conduit.

Standard Temporary Discharge (STD) – The flow corresponding to twice the maximum mean monthly flow. The rate of flow for which a TAF facility is designed. Usually given in cubic feet per second (cfs).

Step Backwater or Standard Step Method – An iterative use of the energy equation for determining the water surface profile of an open channel.

Storm Sewer – A conduit or pipe drainage system that conveys storm water, subsurface water, condensate, or similar discharge, but not household or industrial wastes.

Thalweg – The lowest bed elevation in a natural channel cross section. Also used in reference to the profile line extending down a channel along the lowest bed elevation.

Tailwater – The depth of flow in the stream directly downstream of a drainage facility, measured from the invert at the culvert outlet. Often calculated for the discharge flowing in the natural stream without the highway constriction. Term is usually used in culvert design and is the depth measured from the downstream flow line of the culvert to the water surface.

Temporary Access Fill (TAF) –A structure or earth fill that allows a contractor access to work on roads or bridges located within or over bodies of water.

Time of Concentration (tc) – Time required for water to flow from the most distant point on a drainage area to the measurement or collection point.

TMDL (Total Maximum Daily Load) Regulated Stream – An Impaired water body as defined by the Ohio EPA that can still meet water quality standards if the daily maximum pollutant load is regulated.

Two Stage Channel – A channel that contains a cross sectional area for low and high discharges.

Water of The United States – Water bodies subject to Army Corps of Engineers jurisdiction through Section 404 of the Clean Water Act. They include all interstate waters such as lakes, rivers, streams (including intermittent streams) and wetlands. Ephemeral streams are included if they have a clearly defined channel.

Watershed – An area confined by drainage divides, and often having only one outlet for discharge.

Waterway – Any stream, river, pond, or lake. (See Channel)

Waterway Opening – Area of a bridge or drainage structure opening measured normal to the principal flow direction.

## Abbreviations

AASHTO American Association of State Highway and Transportation Officials ADT Average Daily Traffic AEP Annual Exceedance Probability AOP Aquatic Organism Passage ASTM American Society for Testing and Materials AVFS Amended Vegetated Filter Strip AWWA American Water Works Association BDM Bridge Design Manual (ODOT) BFE Base Flood Elevation BMP Best Management Practice C&MS Construction and Material Specifications (ODOT) CB Catch Basin CDSS Drainage Design Software for Culverts Ditches and Storm Sewers (ODOT) CIPP Cured-In-Place Pipe CFN Culvert File Number (ODOT) CFR Code of Federal Regulations cfs Cubic Feet per Second (ft3/s) CLOMR Conditional Letter Of Map Revision CMM Conduit Management Manual (ODOT) CMP Corrugated Metal Pipe DEC District Environmental Coordinator (ODOT) EBW Enhanced Bankfull Width EDA Earth Disturbing Activity EDV Extended Detention Volume EPA Environmental Protection Agency FEMA Federal Emergency Management Agency FHWA Federal Highway Administration, Department of Transportation FIRM Flood Insurance Rate Map FIS Flood Insurance Study FMP Floodplain Management Program fps Feet per Second GIS Geographic Information Systems GDM Geotechnical Design Manual HDPE High Density Polyethylene HDS Hydraulic Design Series HEC Hydraulic Engineering Circular HEC-RAS Hydrologic Engineering Center-River Analysis System H&H Hydrology and Hydraulic (Report)(ODOT) HSG Hydrologic Soil Group HSR Hard Shoulder Running (lanes) HUC Hydraulic Unit Code HW Headwater or Headwall HWI Headwater Inlet Control HWO Headwater Outlet Control LD1 Location & Design Manual, Volume 1 – Roadway Design (ODOT) LD2 Location & Design Manual, Volume 2 – Drainage Design (ODOT) LD3 Location & Design Manual, Volume 3 – Highway Plans (ODOT) LD4 Location & Design Manual, Volume 4 – Survey, Mapping & Subsurface Utility Location Services Specifications (ODOT) LFE Limiting Flow Elevation LOMR Letter Of Map Revision LPA Local Public Agency LRFD Load and Resistance Factor Design LRW Limited Resource Waters MH Manhole MOT Maintenance Of Traffic MS4 Municipal Separate Storm Sewer System Program MSE Mechanically Stabilized Earth (Retaining Wall) NFIP National Flood Insurance Program NOI Notice Of Intent NPDES National Pollutant Discharge Elimination System OCCS Ohio County Coordinate System ODNR Ohio Department of Natural Resources ODOT Ohio Department of Transportation OES Office of Environmental Services (ODOT) OGE Office of Geotechnical Engineering (ODOT) OHE Office of Hydraulic Engineering (ODOT) OHWM Ordinary High Water Mark OMM Office of Material Management (ODOT) ORC Ohio Revised Code OSE Office of Structural Engineering (ODOT) OSIP Ohio Statewide Imagery Program PBPD Performance Based Practical Design PDP Project Development Process PE Polyethylene PDR Permit Determination Request RCP Rock Channel Protection R/W Right-of-Way SAPL Spray Applied Pipe Liner SCD Standard Construction Drawing (ODOT) SFE Structure Foundation Exploration (Report)(ODOT) SFHA Special Flood Hazard Area SFN Structure File Number SGE Specifications for Geotechnical Exploration (ODOT) SIR Scientific Investigations Report (USGS Report) STD Standard Temporary Discharge STS Structure Type Study SWPPP Storm Water Pollution Prevention Plan SS### Supplemental Specification TAF Temporary Access Fill TMDL Total Maximum Daily Load TIMS Transportation Information Mapping System TRB Transportation Research Board TSEC Temporary Sediment and Erosion Control TSS Total Suspended Solids USDA United States Department of Agriculture USDOT United States Department of Transportation USGS United States Geological Survey VBF Vegetated Bio Filter VFS Vegetated Filter Strip WRI Water Resources Investigation (USGS Report) WQF Water Quality Flow WQV Water Quality Volume WSE Water Surface Elevation

## References

Code of Federal Regulations

23 CFR 650A - Location and Hydraulic Design of Encroachments on Flood Plains. Code of Federal Regulations: Title 23 - Highways; Chapter I - Federal Highway Administration, Department of Transportation; Subchapter G - Engineering and Traffic Operations; Part 650 - Bridges, Structures, and Hydraulics; Subpart A - Location and Hydraulic Design of Encroachments on Flood Plains.

40 CFR 122.26(b)(15)(i) - Storm water discharges (applicable to State NPDES programs, see § 123.25). Code of Federal Regulations: Title 40 - Protection of Environment; Chapter I - Environmental protection Agency (continued); Subchapter D - Water Programs; Part 122 - EPA Administered Permit Programs: The National Pollutant Discharge Elimination System; Subpart B - Permit Application and Special NPDES Program Requirements; Section 122.26 - Storm water discharges (applicable to State NPDES programs, see § 123.25); (b) Definitions; (15)(i)

Publications

American Association of State Highway and Transportation Officials (AASHTO). (2005). Model Drainage Manual.

American Association of State Highway and Transportation Officials (AASHTO). (2014). Drainage Manual Volume 1 – Policy.

Arneson, L.A.; Zevenbergen, L.W.; Lagasse, P.F.; Clopper, P.E. (2012). Hydraulic Engineering Circular No. 18: Evaluating Scour at Bridges, Fifth Edition (FHWA-HIF-12-003)(HEC-18). U.S. Department of Transportation, Federal Highway Administration.

Brown, S.A.; Schall, J.D.; Morris, J.L; Doherty, C.L.; Stein, S.M.; Warner J.C. (2009). Hydraulic Engineering Circular No. 22, Third Edition: Urban Drainage Design Manual (FHWA-NHI-10-009)(HEC-22). U.S. Department of Transportation, Federal Highway Administration.

California Department of Transportation (CALTRANS). (2002). Caltrans Storm Water Quality Handbook - Project Planning and Design Guide.

Che, D.; Khoury, I.; White, K.; Rothwell, D. (2022). Efficient and Effective Ways to Manage Water through ODOT’s Temporary Fills during Construction (FHWA/ODOT-2022/18). Ohio Department of Transportation.

Federal Aviation Administration. (1970) Airport Drainage (USDOT, FAA Advisory Circular 150/5320-5B). U.S. Department of Transportation.

Federal Emergency Management Administration. (2005). National Flood Insurance Program (NFIP) Floodplain Management Requirements, A study Guide and Desk Reference for Local Officials - FEMA 480.

Federal Interagency Stream Restoration Working Group (FISRWG). (1995) (Revised 2001). Stream Corridor Restoration: Principles, Practices and Processes (USDA, Natural Resources Conservation Service, National Engineering Handbook Part 653) (H_210_NEH_653 – Part 653). U.S. Department of Agriculture.

Great Lakes - Upper Mississippi River Board of State and Provincial Public Health and Environmental Managers (GLUMRB). (2014). 10 State Standards, Recommended Standards for Wastewater Facilities.

Great Lakes - Upper Mississippi River Board of State and Provincial Public Health and Environmental Managers (GLUMRB). (2022). 10 State Standards, Recommended Standards for Water Works.

Harrison, L.J.; Morris, J.L; Norman, J.M.; Johnson, F.L. (1972). Hydraulic Engineering Circular No. 13: Hydraulic Design of Improved Inlets for Culverts (FHWA/EO-72-130)(HEC-13). U.S. Department of Transportation, Federal Highway Administration.

Hurd, J.O. (1991). Field Performance of Precast Reinforced Concrete Box Culverts (TRB Transportation Research Record No. 1315, p. 53-57). Transportation Research Board.

Hurd, J.O. (1991). Information Needs for the Proper Application of Hydrologic Regional Regression Equations (TRB Transportation Research Record No. 1319, p. 126-130). Transportation Research Board.

Kilgore, R.T.; Cotton, G.K. (2005). Hydraulic Engineering Circular No. 15: Design of Roadside Channels with Flexible Linings, Third Edition (FHWA-NHI-05-114)(HEC-15). U.S. Department of Transportation, Federal Highway Administration.

Koltun, G.F.; Roberts, J.W. (1990). Techniques for Estimating Flood-Peak Discharges of Rural, Unregulated Streams in Ohio (USGS Water-Resources Investigations Report 89-4126) (FHWA/OH-90/01). U.S. Geological Survey.

Koltun, G.F. (2003). Techniques for Estimating Flood-Peak Discharges of Rural, Unregulated Streams in Ohio, Second Edition (USGS Water-Resources Investigations Report 03-4164) (FHWA/OH-2003/013). U.S. Geological Survey.

Koltun, G.F.; Kula, S.P.; Puskas, B.M. (2006). A Streamflow Statistics (StreamStats) Web Application for Ohio (USGS Scientific Investigations Report 2006-5312). U.S. Geological Survey.

Koltun, G.F. (2019). Flood-Frequency Estimates for Ohio Streamgages Based on Data through Water Year 2015 and Techniques for Estimating Flood-Frequency Characteristics of Rural, Unregulated Ohio Streams (USGS Water Resources Investigations Report 2019-5018). U.S. Geological Survey.

Lagasse, P.F.; Clopper, P.E; Zevenbergen, Ruff, J.F. (2006). Riprap Design Criteria, Recommended Specifications, and Quality Control (NCHRP Report 568). Transportation Research Board.

Lagasse, P.F.; Clopper, P.E; Zevenbergen, Girard, L.G. (2007). Countermeasures to Protect Bridge Piers from Scour (NCHRP Report 593). Transportation Research Board.

Lagasse, P.F.; Clopper, P.E; Pagán-Ortiz, J.E.; Zevenbergen, L.W.; Arneson, L.A.; Schall, J.D.; Girard, L.G. (2009). Hydraulic Engineering Circular No. 23: Bridge Scour and Stream Instability Countermeasures Experience, Selection, and Design Guidance, Third Edition, Volumes 1 and 2 (FHWA-NHI-09-111) (FHWA-NIH-09-112)(HEC-23). U.S. Department of Transportation, Federal Highway Administration.

Lagasse, P.F.; Zevenbergen, L.W.; Spitz, W.J.; Arneson, L.A. (2012). Hydraulic Engineering Circular No. 20: Stream Stability at Highway Structures, Fourth Edition (FHWA-HIF-12-004)(HEC-20). U.S. Department of Transportation, Federal Highway Administration.

Masada, T. (2017). Structural Benefits of Concrete Field Paving of Steel Culvert Inverts (FHWA/OH-2017/21). Ohio Department of Transportation.

McCuen, R.H; Johnson, P.A.; Robert M. Ragan, R.M. (2002). Hydraulic Design Series Number 2: Highway Hydrology, Second Edition (FHWA-NHI-02-001)(HDS-2). U.S. Department of Transportation, Federal Highway Administration.

Mitchell, G.F.; Riefler, R.G.; Russ, A. (2010). Vegetated Biofilter for Post Construction Storm Water Management for Linear Transportation Projects (FHWA/OH-2010/7). Ohio Department of Transportation.

National Corrugated Steel Pipe Association (NCSPA). (2008). Corrugated Steel Pipe Design Manual.

Ohio Environmental Protection Agency. (2019). Technical Memorandum: Sediment Storage Design for Post-Construction Practices.

Ohio Environmental Protection Agency. (2019). Technical Memorandum: Revision to minimum filter bed size and ponding depth in Rainwater & Land Development Practice.

Ohio Department of Natural Resources Division of Soil and Water Conservation; Mathews, J. (2006) (Updated 2014). Rainwater and Land Development: Ohio’s Standards for Stormwater Management Land Development and Urban Stream Protection, Third Edition 2006. Ohio Department of Natural Resources.

Richardson, E.V.; Simons, D.B.; Lagasse, P.F. (2001). Hydraulic Design Series Number 6: River Engineering for Highway Encroachments Highways in the River Environment (FHWA-NHI-01-004)(HDS-6). U.S. Department of Transportation, Federal Highway Administration.

Sargand, S.; Hurd, J. White, K.; Masada, T.; Garcia-Ruiz, J.; Colorado-Urrea, G. (2016). Assessment of ODOT’s Conduit Service Life Prediction Methodology (FHWA/OH-2016/16). Ohio Department of Transportation.

Schall, J.D.; Thompson, P.L; Zerges, S.M.; Kilgore, R.T.; Morris, J.L. (2012). Hydraulic Design Series Number 5: Hydraulic Design of Highway Culverts, Third Edition (FHWA-HIF-12-026)(HDS-5). U.S. Department of Transportation, Federal Highway Administration.

Sherwood, J.M. (1993). Estimation of Peak-Frequency Relations, Flood Hydrographs, and Volume-Duration-Frequency Relations of Ungaged Small Urban Streams in Ohio (USGS Open-File Report 93-135) (FHWA/OH-93/008). U.S. Geological Survey.

Sherwood, J.M. (1993). Estimation of Flood Volumes and Simulation of Flood Hydrographs for Ungaged Small Rural Streams in Ohio (USGS Water-Resources Investigations Report 93-4080) (FHWA/OH-93/007). U.S. Geological Survey.

Sherwood, J.M. (1994). Estimation of Peak-Frequency Relations, Flood Hydrographs, and Volume-Duration-Frequency Relations of Ungaged Small Urban Streams in Ohio (USGS Water-Supply Paper 2432). U.S. Geological Survey.

Sherwood, J.M.; Huitger, C.A. (2005). Bankfull Characteristics of Ohio Streams and Their Relation to Peak Streamflows (USGS Scientific Investigations Report 2005-5153) (FHWA/OH-2005/004). U.S. Geological Survey.

Simon, A.L.; Sarikelle, S. (1984). Internal Energy Dissipators for Culverts (FHWA/OH-84/007). Ohio Department of Transportation.

Tangirala, A.; Kerns, J.; M.S. Consultants. (2022). Assessment of Existing and Potential Volume Reduction for Post Construction Stormwater Management (FHWA/OH-2022-10). Ohio Department of Transportation.

U.S. Department of Agriculture. (1996) Chapter 16, Streambank and Shoreline Protection (USDA, Natural Resources Conservation Service, Engineering Field Handbook).

Water Environment Federation; American Society of Civil Engineers. (1998). Urban Runoff Quality Management (ASCE Manual and Report on Engineering Practice No. 87, WEF Manual of Practice No. 23).

Young, G.K.; Walker, S.E.; Chang, F. (1993). Hydraulic Engineering Circular No. 21: Design of Bridge Deck Drainage (FHWA-SA-92-010)(HEC-21). U.S. Department of Transportation, Federal Highway Administration, Federal Highway Administration.

Zevenbergen, L.W. Arneson, L.A.; Hunt, J.H.; Miller, A.C. (2012). Hydraulic Design Series Number 7: Hydraulic Design of Safe Bridges (FHWA-HIF-12-018)(HDS-7). U.S. Department of Transportation, Federal Highway Administration.

Manuals, Standards & Specifications

ODOT Bridge Maintenance Manual

ODOT Conduit Management Manual

ODOT Construction and Material Specifications

ODOT Geotechnical Design Manual

ODOT Location & Design Manual Volume 1

ODOT Location & Design Manual Volume 3

ODOT Location & Design Manual Volume 4

ODOT Ordinary High Water Mark Identification Manual

ODOT Standard Construction Drawings

Websites

FHWA Ultra Urban BMP webpage

USEPA National Pollutant Discharge webpage

Ohio EPA

(THIS PAGE INTENTIONALLY LEFT BLANK)