STATE OF OHIO

DEPARTMENT OF TRANSPORTATION

SUPPLEMENT 1128

REQUIREMENTS FOR NON-TRACKING TACK APPROVAL

January 16, 2026

1128.01	Introduction

1128.02	Approval and Certification of New Non-Tracking Tack Product

1128.03	Procedure to Regain Certification of a Non-Tracking Tack Product

1128.01 	Introduction.  The supplier’s product must be listed on the Supplement 1032 Certified List prior to use on a project.  New non-tracking tack products not listed on the Supplement 1032 Certified List may be used with permission of OMM for demonstration purposes according to 1128.02 C.  Once a supplier’s product is listed on the Supplement 1032 Certified List, follow the requirements of Supplement 1032.

1128.02 	Approval and Certification of New Non-Tracking Tack Product.

- The supplier will initiate a request for authorization to ship a new non-tracking tack product under Supplement 1032 certification by:

  - Submit a written request to OMM to have the new non-tracking tack product evaluated for certification.

  - Provide a letter to OMM detailing where the supplier’s product has been used successfully and list states, counties, cities, etc. where the product has been approved or certified for use.

  - Provide a technical data sheet showing the minimum and/or maximum test requirements for the supplier’s product including requirements in Table 702.12-1.  Testing requirements on residual properties will be per supplier’s requirements.  Testing requirements will follow AASHTO procedures.  If no AASHTO procedure is available, then an ASTM procedure will be used.  Technical data sheet will indicate the expected break and set times for the product to become non-tracking.  The expected times will be 30 minutes or less.

  - Provide a safety data sheet (SDS).

  - Provide a Quality Control Plan as outlined in Supplement 1032.

  - Submit five one-quart (1.25 L) samples of supplier’s product to OMM including supplier’s test results for that same batch or lot of material.

- Once samples are tested at OMM and results verify compliance with the supplier’s product specifications, the supplier must schedule a demonstration on a non-Department project using the following requirements:

  - The demonstration project will be at a location in Ohio the supplier determines. Locations outside Ohio may be allowed at the discretion of OMM. The demonstration may be on a project (city, county, etc.) or at a supplier facility. Inform OMM of proposed location at least one week in advance of the demonstration.

  - The demonstration must have a minimum pavement temperature of 85 °F (29 °C).

  - The product must meet the requirements of supplier’s technical data sheet.

- If the results of the demonstration in 1128.02 B. are acceptable, OMM will allow supplier’s product to be demonstrated on a Department project.  The supplier will be responsible for finding an interested contractor and Department project to perform the demonstration. No additional compensation will be given from the Department to the contractor for using the supplier’s product.  Include the following requirements:

  - The demonstration must have a minimum pavement temperature of 85 °F (29 °C).

  - The product must meet the requirements of supplier’s technical data sheet.

- If results of both demonstrations are acceptable, the supplier must follow S1032.02 A. and B. After five consecutive passing samples, the supplier’s product will be certified and listed on the Supplement 1032 Certified List.

1128.03 	Procedure to Regain Certification of a Non-Tracking Tack Product.

- In the event the product fails to meet test requirements or perform (tracking, not setting, etc.) to the satisfaction of the Department, the supplier will be removed from the Supplement 1032 Certified List. Failure to meet non-tracking requirements in actual field use will result in supplier removal from certification. The “supplier” is any original manufacturer and any subsequent secondary supplier of the same material. Any secondary supplier may be allowed to provide the same product from another S1032-certified manufacturer.  The supplier may perform the following items in order to be considered for recertification:

  - Submit in writing to OMM the reason(s) why the product failed to perform and detail changes that will be made to eliminate the cause(s) of failure, and

  - Propose changes to the product’s specifications, and

  - Submit five one-quart (1.25 L) samples of the redeveloped product to OMM for testing to the new specifications, and

  - Successfully perform demonstrations on a non-Department project and then on an Department project as outlined in S1128.02 B. and C.

- Upon successful completion of 1128.03 A. the Department may consider the product for re-certification.  The supplier must follow S1032.02 A. and B. to obtain certification on the Supplement 1032 Certified List.

- If a product is removed two or more times due to performance (tracking, adhesion issues, etc.) in a five-year span, the product removal will be a minimum of 180 days. Upon completion of the 180 days, the supplier must show documentation of successful performance of the product prior to re-applying for certification. The supplier must follow S1032.02 A. and B. to obtain certification on the Supplement1032 Certified List.