Roadway high point: Rear abutment side towards the end of the approach slab (where pile driving equipment could be setup. Elevation 910.12’

This roadway high point is approximately 2,030 feet away from the edge of Neil Armstrong Airport.

Airport elevation from www.airport-data.com : 913 feet. Runway length is 5500 feet.

Z= C/X + A-B: where Z is amount of clearance, A is airport elevation, B is project elevation and C is distance between project and airport. X is 100:1 slope so x is 100 in this case.

Z= 2030/100 + 913-910.12 = 23.18 feet. Anything taller than 23.18’ will go through the notification surface.

1407-3 in L&D Vol 3 suggests large bridges need 100 feet, pile driving needs 50 feet and small bridges need 60 feet. I would consider this a small bridge and it will require pile driving. 60 feet should be height used.

23.18’<60 feet. Notification is necessary, unless surrounded by structures or objects greater equal in height (Vol 3 1407.1.4).