CONSYS Inputs and calcs

Geometry of slab: 3 spans of 26, 32.5, and 26 feet. 17” thick. 20” thick deck edge @ 4 feet wide.

Moment of Inertia: 1/12 bh^3 = 1/12 x 12in x 17^3 = 4913 in^4

Modulus of Elasticity = 57000sqrt(concrete psi) = 57000sqrt(4500) = 3823676.24 /1000 = 3823.7 ksi

*Moment of Inertia and Modulus are for calculating deflection, so not necessary for pile loading*

Dead loads on the structure:

Slab: calc area from ORD or manually [(17/12 x 36’) + (3/12 x 4 x 2)]/36 [*.150 kcf] = 0.2208 klf

TST rail: per std TST-2-21 [0.08 klf x 2 sides]/36’ = 0.0044 klf

Total DC loading: 0.2208 klf + 0.0044 klf = 0.2252 klf

Future Wearing Surface: 0.06 ksf per BDM 401.3 or 0.06 klf

PW link to Consys span file with dead and live loads

Proposed 3 span concrete slab.CSP

Consys Results

Abutment loading reactions:

DC dead load = 2.178 klf

DW dead load = 0.58 klf

Design Lane = 7.597 kip

Design Truck = 43.823 kip

Design Tandem = 45.250 kip

Design Double Truck = 44.484 kip

Design Double Tandem = 45.250 kip (N/A for bridge per LRFD C3.6.1.3.1)

Service 1 for Abutments = 55.604 kip

Pier loading reactions:

DC dead load = 7.345 klf

DW dead load = 1.955 klf

Design Lane = 22.159 kip

Design Truck = 61.296 kip

Design Tandem = 49.512 kip

Design Double Truck = 61.296 kip

Design Double Tandem = 62.472 kip (N/A for bridge per LRFD C3.6.1.3.1)

Service 1 for Piers = 93.932 kip

Abutment

Skew of 15 degrees

Abutment length is 54.16 feet

Diaphragm width is approximately 40.55’

Moment of Inertia abutment – 1/12 x 27 x 60^3 = 486000 in^4 (Assuming abutment height is 5 feet)

Modulus of Elasticity = 57000sqrt(concrete psi) = 57000sqrt(4500) = 3823676.24 /1000 = 3823.7 ksi

Number of lanes that can physically fit on the bridge 36’/12 foot lanes = 3 lanes

Multiple presence factors from Table 3.6.1.1.2-1 LRFD: 1.2 for single lane, 1.0 for 2 lanes and 0.85 for 3 lanes.

Abutment Loading

DC slab and rail = 2.178 klf x 36’ = 78.41 kip

DC diaphragm = [2 ends x 1.55’ x 1.67’ x 2.25’]+ [1.65’ x 2.25’ x 40.55’] x .150 kcf = 24.33 kip

DC approach slab = 22.53 sf from ORD x 36 ft width x 1/3 reaction to abutment x .150 kcf = 40.55 kip

Total superstructure loading = 143.29/40.5’ = 3.538 klf

DC wingwall = 15.30 sf x 2.25’ + 17.16 sf x 2.25’ x .150 kcf = 10.96 kip

DC abutment = 2.25’ x 2.35’ x 54.16’ x .150 kcf = 42.96 kip

DC footing = 3’ x 3’ x 54.16’ x .150 kcf = 73.12 kip

Total substructure loading = 10.96+42.96+73.12 = 127.04 kip/54.16’ = 2.346 klf

DW = 0.58 klf x 36’ = 20.88 kip/40.5’ = 0.516 klf

Design Lane = 7.597 kip/10’ = 0.760 klf

Design Truck = 43.823 kip/2 = 21.912 k per wheel

Design Tandem = 45.250 kip/2 = 22.625 k per wheel

Consys output values (Iteration on location of loads to find max pile loading. Use influence lines to guide placement of loads)

Case: 1 lane centered on 3rd pile. Single truck loads.

Dead load on pile 3 = 40.923 kip

Future wearing surface on pile 3 = 3.589 kip

Design truck wheels = 29.568 kip force on abutment pile 3

Design lane = 5.689 kip force on abutment pile 3

Total load: 40.923 + 3.589 + 1.2(29.568 + 5.689) = 86.82 kip

Case: 1 lane centered on 3rd pile. Tandem truck loads.

Dead load on pile 3 = 40.923 kip

Future wearing surface on pile 3 = 3.589 kip

Design truck wheels = 30.53 kip force on abutment pile 3

Design lane = 5.689 kip force on abutment pile 3

Total load: 40.923 + 3.589 + 1.2(30.53 + 5.689) = 87.97 kip

Pier

Skew of 15 degrees

Pier length is 35.79 feet

Moment of Inertia abutment – 1/12 x 36 x 24^3 = 41,472 in^4

Modulus of Elasticity = 57000sqrt(concrete psi) = 57000sqrt(4500) = 3823676.24 /1000 = 3823.7 ksi

Number of lanes that can physically fit on the bridge 36’/12 foot lanes = 3 lanes

Multiple presence factors from Table 3.6.1.1.2-1 LRFD: 1.2 for single lane, 1.0 for 2 lanes and 0.85 for 3 lanes.

Pier Loading

DC slab and rail = 7.345 klf x 36’ = 264.42 kip

DC Pier cap = 105.36 sf x 2’ + keyway (7.42 sf x 3’) x .150 kcf = 34.95 kip

Superstructure total = [264.42 + 34.95]/35.79 = 8.365 klf

DW (future wearing surface) = [1.955 klf x 36’]/35.79 = 1.966 klf

Design Lane = 22.159 k/10’ = 2.216 klf

Design truck = 61.296/2 = 30.65 kip per wheel

CONSYS output

Worst case single truck over pier pile 2:

Dead loads = 59.83 kip

Future wearing = 14.062 kip

Design lane = 15.385 kip

Design single truck = 41.609 kip

Total load = 59.83 + 14.062 + 1.2(15.385 + 41.609) = 142.285 kip