PID 118002, CHP-29-1569: QUANTITY CALCULATIONS

Calculated by:  Matt Blythe, P.E., Date: 11/22/22

Checked by: N/A: 11/22/22

Revised by: JUN XIE: 12/28/22

Roadway

- Item 606 – Anchor Assembly Removed, Type E (ea)

  - Replacing the damaged EB end assembly only = 1ea

- Item 606 – Anchor Assembly, MGS Type E, MASH 2016 (ea)

  - Replacing the damaged EB end assembly only = 1ea

Erosion Control

- Item 659 – Seeding & Mulching (sy)

  - Estimated area of exposed soil due to equipment access (Microstation) = 10’x197’ = 1970sf/9 = 220sy

- Item 659 – Commercial Fertilizer (ton)

  - Designer Note: 40#/1,000sf x1,970sf = 79#/2000 = 0.04ton

- Item 659 – Lime (acre)

  - Over perm seeded area = 1970sf/43560 = 0.05acre

- Item 659 – Water (Mgal)

  - Designer Note: 2ea x 300gal/1,000sf x 1970sf = 1,182gal = 1Mgal

- Item 832-Erosion Control (EA)

  - Use Min = $1,000

Structures

- Item 202 – Portions of Structure Removed, Over 20 Foot Span, As Per Plan (LS)

  - TST post & anchors = 5ea

  - TST tube w/ splice = 41.67’ + 35.42’ = 77.09’

  - Remove BTA hardware for reuse.

  - Replace tube splices & hardware.

  - Superstructure concrete: 1.5’x 31’ x 1.75’/27 = 3.0134cy = 3cy

- Item 509 – Replacement of Existing Concrete Reinforcement, As Per Plan (lb)

  - Minimal expected, assume = 50#

- Item 511 - Class QC2 Concrete, Superstructure (cy)

  - Total = 1.5’x 31’ x 1.75’/27 = 3.0134cy = 3cy

- Item 512 – Sealing of Concrete Surfaces (sy)

  - Total = 31’ x (1.75’ + 0.5’) = 69.75sf /9 = 8sy

- Item 517 – Bridge Railing Rebuilt, As Per Plan (ft)

  - Top length is pay length = 42’

- Item 518 – Steel Drip Strip

  - Total = 31’ + 5ea x 1.5’ + 2ea x 1.5’ = 42’

Maintenance of Traffic

- Item 614-Law Enforcement Officer with Patrol Car for Assistance (HOUR)

  - Total = 0 hrs (matching 99855 approach)

- Item 614-Work Zone Traffic Signal (ea)

  - Total = 0 ea.

  - Included with LS. Not to be a separate pay item.

- Item 614-Work Zone Impact Attenuator, 24” Wide Hazards, (Bidirectional) (EA)

  - Total = 2ea.

- Item 614-Barrier Reflector, Type 1 (ea)

  - TEM 642-51: Not needed since not within 5’ of edge of travel lane.

  - TEM 605-19.2: Max spa = 50’

    - (180 ft/1) x (# of Spaces/50ft)=3.6 Spaces, 4 spaces = 5 ea

- Item 614-Object Marker, Two Way (ea)

  - TEM 642-51: Not needed since not within 5’ of edge of travel lane.

  - TEM 605-19.2: Max spa = 50’

    - (180 ft/1) x (# of Spaces/50ft)=3.6 Spaces, 4 spaces = 5 ea

- Item 614-Work Zone Edge Line (ft)

  - Approx. 190 ft/(5280ft/mile) = 0.05 mile

- Item 614-Work Zone Stop Line (ft)

  - Total = 2ea x 10’ = 20’

- Item 622-Portable Barrier, Unanchored (ft)

  - 2 parallel sections: 2 x 10’ =20’

  - 1st section = 29’, use 30’

  - 2nd section = 32’, use 40’

  - 3rd section = 97’, use 100’

  - Total = 20’ + 30’ + 40’+100’ = 190’, Adopt 200’

Incidentals

- Item 614-Maintaining Traffic (LS)

  - Includes signal.

- Item 623-Construction Layout Stakes and Surveying (LS)

  - For staking OHWM.

- Item 624-Mobilization (LS)

END OF CALCULATIONS