# HY-8 Culvert Analysis Report

## Crossing Discharge Data

Discharge Selection Method: Specify Minimum, Design, and Maximum Flow

Minimum Flow: 100.00 cfs

Design Flow: 317.00 cfs

Maximum Flow: 442.00 cfs

### Table 1 - Summary of Culvert Flows at Crossing: CLA-40-0737 Proposed

| Headwater Elevation (ft) | Total Discharge (cfs) | Stone Arch Discharge (cfs) | Roadway Discharge (cfs) | Iterations |
| --- | --- | --- | --- | --- |
| 1006.21 | 100.00 | 100.00 | 0.00 | 1 |
| 1007.03 | 134.20 | 134.20 | 0.00 | 1 |
| 1007.82 | 168.40 | 168.40 | 0.00 | 1 |
| 1008.57 | 202.60 | 202.60 | 0.00 | 1 |
| 1009.33 | 236.80 | 236.80 | 0.00 | 1 |
| 1010.10 | 271.00 | 271.00 | 0.00 | 1 |
| 1011.02 | 317.00 | 308.85 | 7.89 | 19 |
| 1011.05 | 339.40 | 309.73 | 29.00 | 6 |
| 1011.08 | 373.60 | 310.72 | 62.26 | 5 |
| 1011.10 | 407.80 | 311.55 | 96.03 | 5 |
| 1011.12 | 442.00 | 312.28 | 129.34 | 4 |
| 1011.00 | 308.21 | 308.21 | 0.00 | Overtopping |

### Rating Curve Plot for Crossing: CLA-40-0737 Proposed

# Culvert Data: Stone Arch

### Table 1 - Culvert Summary Table: Stone Arch

| Total Discharge (cfs) | Culvert Discharge (cfs) | Headwater Elevation (ft) | Inlet Control Depth (ft) | Outlet Control Depth (ft) | Flow Type | Normal Depth (ft) | Critical Depth (ft) | Outlet Depth (ft) | Tailwater Depth (ft) | Outlet Velocity (ft/s) | Tailwater Velocity (ft/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100.00 cfs | 100.00 cfs | 1006.21 | 3.21 | 1.974 | 6-FFt | 1.82 | 1.96 | 2.19 | 1.63 | 6.86 | 6.62 |
| 134.20 cfs | 134.20 cfs | 1007.03 | 4.03 | 2.521 | 6-FFt | 2.21 | 2.34 | 2.46 | 1.90 | 8.17 | 7.19 |
| 168.40 cfs | 168.40 cfs | 1007.82 | 4.82 | 3.113 | 6-FFt | 2.60 | 2.70 | 2.70 | 2.14 | 9.35 | 7.66 |
| 202.60 cfs | 202.60 cfs | 1008.57 | 5.57 | 3.878 | 6-FFc | 2.99 | 3.03 | 3.03 | 2.35 | 10.05 | 8.05 |
| 236.80 cfs | 236.80 cfs | 1009.33 | 6.33 | 4.710 | 6-FFc | 3.39 | 3.35 | 3.35 | 2.54 | 10.68 | 8.40 |
| 271.00 cfs | 271.00 cfs | 1010.10 | 7.10 | 5.608 | 6-FFc | 3.80 | 3.65 | 3.65 | 2.72 | 11.28 | 8.71 |
| 317.00 cfs | 308.85 cfs | 1011.02 | 8.02 | 6.655 | 6-FFc | 4.35 | 3.94 | 3.94 | 2.94 | 11.98 | 9.08 |
| 339.40 cfs | 309.73 cfs | 1011.05 | 8.05 | 6.679 | 6-FFc | 4.36 | 3.95 | 3.95 | 3.04 | 12.00 | 9.24 |
| 373.60 cfs | 310.72 cfs | 1011.08 | 8.08 | 6.707 | 6-FFc | 4.38 | 3.95 | 3.95 | 3.19 | 12.02 | 9.48 |
| 407.80 cfs | 311.55 cfs | 1011.10 | 8.10 | 6.731 | 6-FFc | 4.39 | 3.96 | 3.96 | 3.32 | 12.03 | 9.70 |
| 442.00 cfs | 312.28 cfs | 1011.12 | 8.12 | 6.802 | 6-FFt | 4.40 | 3.96 | 4.02 | 3.46 | 11.92 | 9.91 |

## Culvert Barrel Data

Culvert Barrel Type Straight Culvert

Inlet Elevation (invert): 1003.00 ft,

Outlet Elevation (invert): 1002.44 ft

Culvert Length: 56.00 ft,

Culvert Slope: 0.0100

### Culvert Performance Curve Plot: Stone Arch

### Water Surface Profile Plot for Culvert: Stone Arch

## Site Data - Stone Arch

Site Data Option: Culvert Invert Data

Inlet Station: 0.00 ft

Inlet Elevation: 1003.00 ft

Outlet Station: 56.00 ft

Outlet Elevation: 1002.44 ft

Number of Barrels: 1

## Culvert Data Summary - Stone Arch

Barrel Shape: User Defined

Barrel Span: 7.00 ft

Barrel Rise: 5.50 ft

Barrel Material: Concrete

Embedment: 0.00 in

Barrel Manning's n: 0.0200 (top and sides)

Manning's n: 0.0200 (bottom)

Culvert Type: Straight

Inlet Configuration: Thin Edge Projecting (Ke=0.9)

Inlet Depression: None

## Tailwater Data for Crossing: CLA-40-0737 Proposed

### Table 2 - Downstream Channel Rating Curve (Crossing: CLA-40-0737 Proposed)

| Flow (cfs) | Water Surface Elev (ft) | Velocity (ft/s) | Depth (ft) | Shear (psf) | Froude Number |
| --- | --- | --- | --- | --- | --- |
| 100.00 | 1004.63 | 1.63 | 6.62 | 0.68 | 1.06 |
| 134.20 | 1004.90 | 1.90 | 7.19 | 0.80 | 1.08 |
| 168.40 | 1005.14 | 2.14 | 7.66 | 0.89 | 1.10 |
| 202.60 | 1005.35 | 2.35 | 8.05 | 0.98 | 1.11 |
| 236.80 | 1005.54 | 2.54 | 8.40 | 1.06 | 1.12 |
| 271.00 | 1005.72 | 2.72 | 8.71 | 1.14 | 1.13 |
| 317.00 | 1005.94 | 2.94 | 9.08 | 1.23 | 1.14 |
| 339.40 | 1006.04 | 3.04 | 9.24 | 1.27 | 1.15 |
| 373.60 | 1006.19 | 3.19 | 9.48 | 1.33 | 1.15 |
| 407.80 | 1006.32 | 3.32 | 9.70 | 1.39 | 1.16 |
| 442.00 | 1006.46 | 3.46 | 9.91 | 1.44 | 1.16 |

## Tailwater Channel Data - CLA-40-0737 Proposed

Tailwater Channel Option: Trapezoidal Channel

Bottom Width: 6.00 ft

Side Slope (H:V): 2.00 (_:1)

Channel Slope: 0.0067

Channel Manning's n: 0.0200

Channel Invert Elevation: 1003.00 ft

## Roadway Data for Crossing: CLA-40-0737 Proposed

Roadway Profile Shape: Constant Roadway Elevation

Crest Length: 1000.00 ft

Crest Elevation: 1011.00 ft

Roadway Surface: Paved

Roadway Top Width: 40.00 ft