# HY-8 Culvert Analysis Report

## Crossing Discharge Data

Discharge Selection Method: User Defined

### Table 1 - Summary of Culvert Flows at Crossing: Crossing 1

| Headwater Elevation (ft) | Discharge Names | Total Discharge (cfs) | Culvert 1 Discharge (cfs) | Roadway Discharge (cfs) | Iterations |
| --- | --- | --- | --- | --- | --- |
| 1011.45 | Design | 317.00 | 12.46 | 305.65 | 7 |
| 1012.74 | Flood | 442.00 | 10.59 | 431.41 | 3 |
| 1011.00 | Overtopping | 205.07 | 205.07 | 0.00 | Overtopping |

### Rating Curve Plot for Crossing: Crossing 1

# Culvert Data: Culvert 1

### Table 1 - Culvert Summary Table: Culvert 1

| Discharge Names | Total Discharge (cfs) | Culvert Discharge (cfs) | Headwater Elevation (ft) | Inlet Control Depth (ft) | Outlet Control Depth (ft) | Flow Type | Normal Depth (ft) | Critical Depth (ft) | Outlet Depth (ft) | Tailwater Depth (ft) | Outlet Velocity (ft/s) | Tailwater Velocity (ft/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Design | 317.00 cfs | 12.46 cfs | 1011.45 | 0.78 | 7.455 | 4-FFf | 0.37 | 0.46 | 5.00 | 8.45 | 0.36 | 1.64 |
| Flood | 442.00 cfs | 10.59 cfs | 1012.74 | 0.70 | 8.744 | 4-FFf | 0.33 | 0.41 | 5.00 | 9.74 | 0.30 | 1.78 |

## Culvert Barrel Data

Culvert Barrel Type Straight Culvert

Inlet Elevation (invert): 1004.00 ft,

Outlet Elevation (invert): 1003.00 ft

Culvert Length: 56.01 ft,

Culvert Slope: 0.0179

### Culvert Performance Curve Plot: Culvert 1

### Water Surface Profile Plot for Culvert: Culvert 1

## Site Data - Culvert 1

Site Data Option: Culvert Invert Data

Inlet Station: 100.00 ft

Inlet Elevation: 1004.00 ft

Outlet Station: 156.00 ft

Outlet Elevation: 1003.00 ft

Number of Barrels: 1

## Culvert Data Summary - Culvert 1

Barrel Shape: Concrete Box

Barrel Span: 7.00 ft

Barrel Rise: 5.00 ft

Barrel Material: Concrete

Embedment: 0.00 in

Barrel Manning's n: 0.0200

Culvert Type: Straight

Inlet Configuration: Square Edge (90º) Headwall (Ke=0.5)

Inlet Depression: None

## Tailwater Data for Crossing: Crossing 1

### Table 2 - Downstream Channel Rating Curve (Crossing: Crossing 1)

| Flow (cfs) | Water Surface Elev (ft) | Velocity (ft/s) | Depth (ft) | Shear (psf) | Froude Number |
| --- | --- | --- | --- | --- | --- |
| 317.00 | 1011.45 | 8.45 | 1.64 | 3.53 | 0.13 |
| 442.00 | 1012.74 | 9.74 | 1.78 | 4.07 | 0.13 |

## Tailwater Channel Data - Crossing 1

Tailwater Channel Option: Trapezoidal Channel

Bottom Width: 6.00 ft

Side Slope (H:V): 2.00 (_:1)

Channel Slope: 0.0067

Channel Manning's n: 0.2000

Channel Invert Elevation: 1003.00 ft

## Roadway Data for Crossing: Crossing 1

Roadway Profile Shape: Constant Roadway Elevation

Crest Length: 1000.00 ft

Crest Elevation: 1011.00 ft

Roadway Surface: Paved

Roadway Top Width: 40.00 ft