# HY-8 Culvert Analysis Report

## Crossing Discharge Data

Discharge Selection Method: Recurrence

### Table 1 - Summary of Culvert Flows at Crossing: CLA-40-0732 Proposed

| Headwater Elevation (ft) | Discharge Names | Total Discharge (cfs) | Stone Arch Discharge (cfs) | Roadway Discharge (cfs) | Iterations |
| --- | --- | --- | --- | --- | --- |
| 1009.88 | 25 year | 262.00 | 262.00 | 0.00 | 1 |
| 1010.59 | 100 year | 364.00 | 282.24 | 81.63 | 11 |
| 1010.50 | Overtopping | 279.61 | 279.61 | 0.00 | Overtopping |

### Rating Curve Plot for Crossing: CLA-40-0732 Proposed

# Culvert Data: Stone Arch

### Table 1 - Culvert Summary Table: Stone Arch

| Discharge Names | Total Discharge (cfs) | Culvert Discharge (cfs) | Headwater Elevation (ft) | Inlet Control Depth (ft) | Outlet Control Depth (ft) | Flow Type | Normal Depth (ft) | Critical Depth (ft) | Outlet Depth (ft) | Tailwater Depth (ft) | Outlet Velocity (ft/s) | Tailwater Velocity (ft/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 year | 262.00 cfs | 262.00 cfs | 1009.88 | 7.23 | 7.105 | 7-H2t | -1.00 | 3.45 | 3.45 | 2.68 | 11.35 | 8.63 |
| 100 year | 364.00 cfs | 282.24 cfs | 1010.59 | 7.94 | 7.443 | 7-JH2c | -1.00 | 3.60 | 3.60 | 3.15 | 11.79 | 9.42 |

## Culvert Barrel Data

Culvert Barrel Type Straight Culvert

Inlet Elevation (invert): 1002.65 ft,

Outlet Elevation (invert): 1002.64 ft

Culvert Length: 56.00 ft,

Culvert Slope: 0.0002

### Culvert Performance Curve Plot: Stone Arch

### Water Surface Profile Plot for Culvert: Stone Arch

## Site Data - Stone Arch

Site Data Option: Culvert Invert Data

Inlet Station: 0.00 ft

Inlet Elevation: 1002.65 ft

Outlet Station: 56.00 ft

Outlet Elevation: 1002.64 ft

Number of Barrels: 1

## Culvert Data Summary - Stone Arch

Barrel Shape: User Defined

Barrel Span: 7.00 ft

Barrel Rise: 5.00 ft

Barrel Material: Concrete

Embedment: 0.00 in

Barrel Manning's n: 0.0400 (top and sides)

Manning's n: 0.0150 (bottom)

Culvert Type: Straight

Inlet Configuration: Mitered to Conform to Slope (Ke=0.7)

Inlet Depression: None

## Tailwater Data for Crossing: CLA-40-0732 Proposed

### Table 2 - Downstream Channel Rating Curve (Crossing: CLA-40-0732 Proposed)

| Flow (cfs) | Water Surface Elev (ft) | Velocity (ft/s) | Depth (ft) | Shear (psf) | Froude Number |
| --- | --- | --- | --- | --- | --- |
| 262.00 | 1005.28 | 2.68 | 8.63 | 1.12 | 1.13 |
| 364.00 | 1005.75 | 3.15 | 9.42 | 1.31 | 1.15 |

## Tailwater Channel Data - CLA-40-0732 Proposed

Tailwater Channel Option: Trapezoidal Channel

Bottom Width: 6.00 ft

Side Slope (H:V): 2.00 (_:1)

Channel Slope: 0.0067

Channel Manning's n: 0.0200

Channel Invert Elevation: 1002.60 ft

## Roadway Data for Crossing: CLA-40-0732 Proposed

Roadway Profile Shape: Constant Roadway Elevation

Crest Length: 1000.00 ft

Crest Elevation: 1010.50 ft

Roadway Surface: Paved

Roadway Top Width: 40.00 ft