November 24, 2014

Mr. Stefan Spinosa, P.E.

District Planning and Engineering Administrator

ODOT District 8

505 S. State Route 741

Lebanon, Ohio  45036

Attn:	Scott Brown

Re:	BUT-TR 134-8.45 (Yankee Road), Butler County, Ohio
PID #95124
Middletown Regional/Hook Field Municipal (MWO)
Airway/Highway Clearance Analysis

Dear Mr. Spinosa:

LJB, Inc. has reviewed the above project and has determined that the project will not encroach into the 100:1 and 50:1 FAA notification surfaces for permanent and temporary conditions. This determination is based on an airport runway elevation of 651 feet and the attached project elevations and distances (between the runway’s safety area and the project locations). The attached table provides an analysis of the study showing the available height above the project to the notification surfaces.

Based on this analysis, plan note R118A will be required with a stated maximum operating height of 94’.

G118A

THIS PROJECT HAS BEEN IDENTIFIED AS BEING WITHIN THE INFLUENCE AREA OF A PUBLIC USE AIRPORT

OR HELIPORT. NO TEMPORARY STRUCTURES OR CONSTRUCTION EQUIPMENT AT MAXIMUM OPERATING

HEIGHT SHALL EXCEED A HEIGHT OF 50 FT. IF ANY TEMPORARY STRUCTURES OR CONSTRUCTION

EQUIPMENT WILL EXCEED THIS HEIGHT, FURTHER COORDINATION WITH THE FEDERAL AVIATION

ADMINISTRATION (FAA), AND THE ODOT OFFICE OF AVIATION, WILL BE NECESSARY PRIOR TO ERECTING

SUCH TEMPORARY STRUCTURES OR OPERATING SUCH EQUIPMENT ON THE PROJECT. THE CONTRACTOR

WILL BE REQUIRED TO FILE A NEW FAA FORM 7460-1, ADVISING THE FAA THAT AERONAUTICAL STUDY

NO.____ IS BEING RESUBMITTED AND THAT AN ALTERATION TO THE ORIGINAL SUBMISSION IS REQUESTED.

NOTIFY THE ODOT OFFICE OF AVIATION WHEN RESUBMITTING FAA FORM 7460-1. NO TEMPORARY

STRUCTURES OR CONSTRUCTION EQUIPMENT SHALL EXCEED THE PERMISSIBLE HEIGHT, UNTIL A COPY OF

THE FAA APPROVAL AND THE ODOT OFFICE OF AVIATION PERMIT HAS BEEN FURNISHED TO THE PROJECT

ENGINEER.

FAA APPROVAL MAY TAKE UP TO 45 DAYS. ALL SUBMISSIONS SHALL BE DIRECTED TO THESE OFFICES:

Express Processing Center

The Federal Aviation Administration Ohio Department of Transportation

Southwest Regional Office Office of Aviation

Air Traffic Airspace Branch ASW-520 2829 West Dublin-Granville Road

2601 Meacham Blvd. Columbus, Ohio 43235

Fort Worth, TX 76137-4298 614-387-2358

Designer’s Note: Use note G118A when the construction equipment penetrates the notification surface

of a public-use airport or heliport. The omitted height shall be that which is determined by the FAA in

its letter of response, or approval to Form 7460-1, regarding the elevations that were submitted. If the

FAA approves the original submission, then the maximum height included in the original submission shall

be entered in the note. However, if the FAA approves contingent upon a lesser elevation, then that

elevation shall be entered. The Aeronautical Study No. is set by the FAA when the Form 7460-1 is

originally submitted.

Attached are the following:

- Spreadsheet of notification surface calculations.

- USGS mapping of the project area.

- Project mapping showing the location of points of project used for this analysis.

- Copy of the Ohio Airport Directory (2010-2012) page for Middletown Regional/Hook Field Municipal.

- Copy of the Ohio Aeronautical Chart (2012-2013) showing the Middletown Regional/Hook Field Municipal Airport.

If you have any questions please or need additional information, call me at (937) 259-5177.

Sincerely,

LJB Inc.

Laurence A. Sack, P.E.

Transportation Planner

/tmp/in/input.doc