January 28, 2026

Daniel J. Lorenz, P.E.

LPA Manager

ODOT District 11

2201 Reiser Ave SE

New Philadelphia, Ohio 44663

RE:	Airway/Highway Clearance Analysis

120272 COL-SR9-15.244

Dear Mr. Lorenz:

We have reviewed the above-mentioned project and have determined that the project will not encroach into the 50:1 notification surface. This determination is based on an airport elevation of 1327’ (A) for Koons Airport and 1,162’ (A) for Salem Airpark, a project elevation of 1,282’ (B), with a distance of 11,959’ (C) for Koons Airport and 15,734’ (C) for Salem Airpark between the airport and the nearest/highest point located in the project. The attached diagram provides a detailed analysis of the study. The study has also determined that a height of 234.2’ (Z) exists between the (50:1) notification surface and the project for Koons airport and 144.7’ (Z) for Salem Airpark. Therefore, based on this height and the heights from Figure 1407-3, plan note G118B will be required.

Sincerely,

OHM Advisors

__________________________________

Christopher Dekle, P.E.

Project Manager

FAA Airway/Highway Clearance Process

COL-SR9-15.244

PID# 120272

Step # 1:  LOCATE AIRPORTS IN PROJECT CORRIDOR

Use the following link to locate all airports close to the project corridor (within 20,000 horizontal feet).  This helpful link illustrates and locates both public and private airports.
http://viewer.nationalmap.gov/viewer/

https://ourairports.com/countries/US/OH/

http://www.tollfreeairline.com/ohio.htm

https://skyvector.com/

PID# 120272:  Potentially Impacted Airports:

- Koons Airport

- Salem Airpark

Step #2:  DETERMINE THE GROUND SURFACE ELEVATIONS, “A” & “B”, & LATITUDE/LONGITUDE COORDINATES FOR BOTH THE AIRPORT AND THE POINT ON THE PROJECT CLOSEST-HIGHEST TO THE AIRPORT

Using the same link cited above, right click on the icon “Spot Elevation” (button right next to the Zoom-In (+) button in the tool bar.).  Then right click on the point on the map which defines the end of the airport runway closest to the project.  A box will open up providing the coordinates (latitude, longitude and elevation points) as described in the L&D Manual, Volume 3, Figure 1404-7.  Repeat the process on the corresponding highest-closest point within the project corridor. When choosing where to click, zoom in close enough to ensure that the chosen location is within 50 feet horizontally and 20 vertically, per the L&D Manual, Volume 3, Section 1404.1.7

PID# 120272:  Latitude, Longitude and Elevation points:

| POINTS | LATITUDE (DMS) | LONGITUDE (DMS) | ELEVATION (FT.) “A” & “B” |
| --- | --- | --- | --- |
| Koons Airport | 40° 52’ 58” | -80° 52’ 52” | 1327 |
| Point on project closest-highest to Koons Airport “B” | 40° 54’ 21” | -80° 51’ 01” | 1282 |
| Salem Airpark | 40° 56’ 53” | -80° 51’ 43” | 1162 |
| Point on project closest-highest to Salem Airpark “B” | 40° 54’ 21” | -80° 51’ 01” | 1282 |

Step #3: DETERMINE THE DISTANCE, “C”, IN PLAN VIEW BETWEEN THE COORDINATES ASSOCIATED WITH EACH RESPECTIVE AIRPORT AND THE COORDINATES ASSOCIATED WITH EACH RESPECTIVE HIGH POINT CLOSEST TO THE PROJECT LIMITS.

Use the following link to determine the distance between each set of coordinates:

https://d11ww001.dot.state.oh.us/asp_production/LatLong/latlong_menu.asp

Click on the “Distance” tab, then type in each latitude and longitude coordinate and then hit “go”.  The shortest distance between the airport runway and the project will be generated, as described in the L&D Manual, Volume 3, Figure 1407-7.

PID# 120272:  Distances measured between the point on the airport runway closet to the project:

| POINTS | LATITUDE (DMS) | LONGITUDE (DMS) | DISTANCE (MILES/FT.) “C” |
| --- | --- | --- | --- |
| Koons Airport | 40° 52’ 58” | -80° 52’ 52” | 2.265 mi. 11,959 ft. |
| Point on project closest-highest to Koons Airport “B” | 40° 54’ 21” | -80° 51’ 01” |  |
| Salem Airpark | 40° 56’ 53” | -80° 51’ 43” | 2.98 mi. 15,734 ft. |
| Point on project closest-highest to Salem Airpark “B” | 40° 54’ 21” | -80° 51’ 01” |  |

Step #4:  DETERMINE THE HIGHEST POINT IN ELEVATION REACHED ASSOCIATED WITH THE CONSTRUCTION OF THE PROJECT.

Depending upon what type of project is being constructed, and what type of equipment will be used during the construction of the project, a controlling criteria point must be chosen for each impacted airport within the project corridor.  Per the L&D Manual, Volume 3, Figure 1404-3, based upon the work type being performed near each airport, the height of the controlling criteria, as defined by the equipment used, is chosen.  Use the following link to determine what height should be used for a controlling height at each airport location.

http://www.dot.state.oh.us/Divisions/Engineering/CaddMapping/volume3/Volume3/1400_0714.pdf

PID# 120272:  Work Type, Controlling Criteria & Height, Per the L&D Manual, Volume 3, Figure 1404-3

| WORK TYPE BEING PERFORMED NEAR AIRPORTS WITHIN INFLUENCE AREA (20,000 FEET) | CONTROLLING CRITERIA | HEIGHT (FT.) |
| --- | --- | --- |
| Traffic Signals | Cherry Picker | 50 |
| THE GREATEST HEIGHT ABOVE PROJECT ELEVATION (B) IS 60 FT. |  |  |

Step #5:  USING THE DISTANCE DETERMINED IN STEP 3, DETERMINE IF FAA NOTIFICATION IS REQUIRED BASED ON FAA’S OBSTRUCTION EVALUATION/AIRPORT AIRSPACE ANALYSIS (OE/AAA).

NOT APPLICABLE

Step #6:  USING THE DISTANCE DETERMINED IN STEP 3, DETERMINE IF FAA NOTIFICATION IS REQUIRED BASED ON INTERFERENCE WITH THE NOTIFICATION SLOPE (notification slope, “X;1”, as described in the L&D Manual, Volume 3, Figure 1404-7).

NOT APPLICABLE

Step #7:  USING THE DIMENSIONS PROVIDED IN THE L&D MANUAL, VOL. 3, FIGURES 1404-5 & 1404-6 DETERMINE IF FAA NOTIFICATION IS REQUIRED BASED ON INTERFERENCE WITH THE RUNWAY PROTECTION ZONES (RPZ).

NOT APPLICABLE

Step #8: PERFORM ANALYSIS

NOT APPLICABLE

Step #9: INCLUDE APPROPRIATE NOTES INTO PLANS

NOT APPLICABLE

AIRWAY/HIGHWAY CLEARANCE FOR PRIVATE AIRPORTS

THIS PROJECT HAS BEEN IDENTIFIED AS BEING WITHIN THE INFLUENCE AREA OF TWO (2) PRIVATE USE AIRPORTS LISTED IN THE TABLE BELOW.

| POTENTAILLY IMPACTED AIRPORTS | AIRPORT ELEVATION “A” | PROJECT ELEVATION “B” + 60 feet (Controlling Criteria) | DISTANCE BETWEEN AIRPORT & PROJECT “C” | NOTIFICATION SLOPE X:1 | USE TYPE | AMOUNT OF CLEARANCE ABOVE NOTIFICATION SLOPE “Z” |
| --- | --- | --- | --- | --- | --- | --- |
| Koons Airport | 1327 ft. | 1282+50=1332 ft. | 11,959 ft. | 50:1 | Public | +234.18 ft. |
| Salem Airpark | 1162 ft. | 1282+50=1332 ft. | 15,734 ft. | 50:1 | Public | +144.68 ft. |

NO TEMPORARY STRUCTURES OR CONSTRUCTION EQUIPMENT, AT MAXIMUM OPERATING HEIGHT, SHALL EXCEED THE HEIGHTS (Z) PROVIDED IN THE TABLE ABOVE. IF ANY TEMPORARY STRUCTURES OR CONSTRUCTION EQUIPMENT WILL EXCEED THIS HEIGHT, COORDINATION WITH THE AIRPORT OWNER AND THE ODOT OFFICE OF AVIATION WILL BE NECESSARY PRIOR TO ERECTING SUCH TEMPORARY STRUCTURES OR OPERATING SUCH EQUIPMENT ON THE PROJECT. FOR PRIVATE USE AIRPORTS, COORDINATION SHALL BE WITH THE AIRPORT OWNER AND THE ODOT OFFICE OF AVIATION. NO TEMPORARY STRUCTURES OR CONSTRUCTION EQUIPMENT SHALL EXCEED THE PERMISSIBLE HEIGHT, UNTIL COORDINATION IS MET AND DOCUMENTATION HAS BEEN FURNISHED TO THE PROJECT ENGINEER. IF COORDINATION IS NOT OBTAINED, THEN THE PROJECT ENGINEER WILL HAVE THE AUTHORITY TO PROVIDE RESTRICTIONS AS REQUIRED.

Owner/Manager:    	Salem Airpark					Koons Airport

Michael Pidgeon/Chuck Whitman		David & Harry Koons

11718 Salem Warren Road			1760 State Route 9

Salem, Ohio 44460-9667 			Salem, Ohio 44460-9583

(330) 332-4400					(330) 337-7303

Designer’s Note: Use of note G118B will be required.

Step #10: COMPILE/SUBMIT NOTIFICATION LETTER TO THE PRIVATE AIRPORT OWNER AND TO THE ODOT OFFICE OF AVIATION.

NOT APPLICABLE