PID 115329, MOT-BH-FY24: QUANTITY CALCULATIONS

Calculated by: Parker Brown, Date: 2/22/23

Checked by:

Revised by:

Plan Splits: Parker Brown, Date: 6/26/23

- 01/IMS/47

  - SHE-75-0997L/R

- 03/STR/47

  - CLA-40-2465L/R

  - LOG-245-0209

  - MIA-202-0600

Roadway

- 201E11000 Clearing and Grubbing (LS)

  - SHE-75-0996 =

Erosion Control

- 659E10000 Seeding and Mulching

  - CLA-40-2465L/R = 117+117+1172+1152+ 986+1293 = 4837SF/9 = 538SY

  - LOG-245-0211 = 1400SF + 1173SF = 2573SF/9 = 286SY

  - MIA-202-0600 = 2855SF/9 = 317SY

  - SHE-75-0996 = 2832+1373 = 4205SF/9 = 468SY

  - 01/IMS/47 = 468SY

    - SHE-75-0997L/R

  - 03/STR/47 = 1141SY

    - CLA-40-2465L/R

    - LOG-245-0209

    - MIA-202-0600

- 659E14000 Repair Seeding and Mulching

  - 5% of total

  - 01/IMS/47 = 468SY x 0.5 = 24SY

    - SHE-75-0997L/R

  - 03/STR/47 = 434SY x 0.05 =58SY

    - CLA-40-2465L/R = 538SY x 0.5 = 27SY

    - LOG-245-0209 = 286SY x 0.5 = 15SY

    - MIA-202-0600 = 317SY x 0.5 = 16SY

- 659E20000 Commercial Fertilizer

  - 01/IMS/47 = 4205SF x (30LBS/1000SF) = 126.15LBS/(2000LBS/TON) = 0.063TONS = 0.07TONS

    - SHE-75-0997L/R

  - 03/STR/47 = 0.17tons

    - CLA-40-2465L/R = 4837SF x (30LBS/1000SF) = 145.11LBS/(2000LBS/TON) = 0.073TONS = 0.08TONS

    - LOG-245-0209 = 2573SF x (30LBS/1000SF) = 77.19LBS/(2000LBS/TON) = 0.039TONS = 0.04TONS

    - MIA-202-0600 = 2855SF x (30LBS/1000SF) = 85.65LBS/(2000LBS/TON) = 0.043TONS = 0.05TONS

- 659E35000 Water

  - 01/IMS/47 = 2 apps of 0.0027M.Gal/1SY : 2(468SY x (0.0027M.Gal/1SY)) = 2.52 = 3 M.Gal

    - SHE-75-0997L/R

  - 03/STR/47 = 7 M. Gal

    - CLA-40-2465L/R : 2(538SY x (0.0027M.Gal/1SY)) = 2.9M.Gal = 3 M.Gal

    - LOG-245-0209 : 2(286SY x (0.0027M.Gal/1SY)) = 1.54M.Gal = 2 M.Gal

    - MIA-202-0600 : 2(317SY x (0.0027M.Gal/1SY)) = 1.71M.Gal = 2 M.Gal

- 832E30000 Erosion Control

  - 1000 each for each structure (per Dan)

  - 01/IMS/47 = 2000

    - SHE-75-0997L/R

  - 03/STR/47 = 4000

    - CLA-40-2465L/R

    - LOG-245-0209

    - MIA-202-0600

  - Total = 6000

Traffic Control

- 646E10200 Center Line

  - 114+82 - 109+25= 557’

  - 110+00 - 109+25 = 75’

  - Total = 632ft = 0.12mies

Structure: CLA-40-2465L

- 503E11100 Cofferdams and Excavation Bracing, LS

- 512E10600 Concrete Repair by Epoxy Injection, FT

  - Rear Abut = 5’

  - Fwd Abut = 3’+5.5’ = 8.5’ =9’

  - Total = 14FT

- 844E10001 Concrete Patching with Galvanic Anode Protection, APP, SF

  - Rear Abut = 9SF +8SF  of scc = 17SF

  - Fwd Abut = 163SF+30SF+2SF+2SF=197SF

  - Total = 214SF

Structure: CLA-40-2465R

- 202E11203 Portions of Structure Removed, Over 20 Foot Span, As Per Plan, LS

- 503E11100 Cofferdams and Excavation Bracing, LS

- 509E10000 Epoxy Coated Steel Reinforcement, LB

  - Total = 188LB

- 510E10000 Dowel Holes with Nonshrink, Nonmetallic Grout, EACH

  - WW1 = 6

  - WW2 = 10

  - WW3 = 10

  - WW4 = 10

  - Total = 36 EACH

- 511E50210 Class QC1 Concrete, Abutment, CY

  - WW1 = (1.75’x0.69’)x3’ = 3.62CF

  - WW2 = (1.75’x0.69’)x2.83’ + (1.75’x1’)x4.83’ = 11.87CF

  - Rear Total = 15.49cf/27 = 0.57CY = 1CY

  - WW3 = (1.75’x0.69’)x3’+ (1.75’x1’)x5’ = 12.37CF

  - WW4 = (1.75’x0.69’)x2.83’ + (1.75’x1’)x4.83’ = 11.87CF

  - Rear Total = 24.24cf/27 = 0.90CY = 1CY

  - Total = 2CY

- 512E10100 Sealing of Concrete Surfaces (Epoxy-Urethane), SY

  - Patching = 283SF

  - WWs = 3[2(3’x0.69’)+2(5’x1’)+(1’x1.75’)+(0.69’x1.75’)+(3’x1.75’)+(2’x1.75)]+[2(3’x0.69’)+( 0.69’x1.75’)+( 3’x1.75’)] = 88.14SF

  - Total = 371.14SF/9 = 41.23SY = 42SY

- 512E10600 Concrete Repair by Epoxy Injection, FT

  - Rear Abut = 10’

  - Fwd Abut = 2’+8’+6’ = 16’

  - Total = 26FT

- 516E13600 1” Preformed Expansion Joint Filler

  - WW1 = 0.69’x1.75’ = 1.2SF

  - WW2 = 1.69’x1.75’ = 2.96SF

  - WW3 = 1.69’x1.75’ = 2.96SF

  - WW4 = 1.69’x1.75’ = 2.96SF

  - Total = 10.1sf = 11sf

- 844E10001 Concrete Patching with Galvanic Anode Protection, APP, SF

  - Rear Abut = 14SF+12SF+6SF+16SF+1SF+66SF = 115SF

  - Fwd Abut = 4.5SF+50.5SF+19SF+28SF+63SF+3SF = 168SF

  - Total = 283SF

Structure: LOG-245-0209

- 202E11203 Portions of Structure Removed, Over 20 Foot Span, As Per Plan, LS

- 202E38603 Bridge Railing Removed for Reuse, FT

  - 100ft

- 503E11100 Cofferdams and Excavation Bracing, LS

- 503E21300 Unclassified Excavation, LS

- 509E10000 Epoxy Coated Steel Reinforcement, LB

  - Total = 2194LB

- 511E34410 Class QC2 Concrete, superstructure

  - Left Edge = (2’x1.46’)x93.72’ = 273.66CF/27 = 10.14CY = 11CY

- 511E45720 Class SCC Concrete, Abutment, CY

  - Rear Abut = (2SF+8SF+9SF+2SF)x0.333’ = 7CF

  - Fwd Abut = (29SF + 2SF)x0.333’ = 10.32CF

  - Total = 17.32CF/27 = 0.64CY = 1CY

- 511E81300 Concrete, Misc.: Embedded Galvanic Anode (EGA), EACH

  - Deck Edge Total = 47 EACH

- 512E10400 Treating of Concrete Bridge Deck with SRS, SY

  - Deck = 93.72’x34.5’ = 3233.34SF

  - Approach Slabs = 2(20’x34.5’) = 1380SF

  - Total = 4613.34SF/9 = 512.59SY = 513SY

- 513E21599 Structural Steel for Rehabilitation LB

  - Total from Table = 965LB

- 516E31010 2” Deep Joint Sealer

  - Total = 4ft

- 517E75601 Deep Beam Bridge Retrofit Railing, AAP, FT

  - Total = 2x93.72’ =187.44FT = 188FT

- 517E76302 Railing, Misc.: Type B Anchors, EACH

  - Right Edge = 15posts x 4 anchors = 60EACH

- 518E22300 Special – Steel Drip Strip, FT

  - Total = (15posts x 1.5’) + 94’ = 116.5ft = 117ft

- 844E10001 Concrete Patching with Galvanic Anode Protection, APP, SF

  - Rear Abut = 8+2+9+2 = 21SF

  - Fwd Abut = 29+2 = 31SF

  - Total = 52sf

Structure: MIA-202-0600

- 503E11100 Cofferdams and Excavation Bracing, LS

- 503E21300 Unclassified Excavation, LS

- 507E71200 Special – Pile Encasement, FT

  - Total from Table = 135.8FT = 136ft

Structure: SHE-75-0997 L

- 503E11100 Cofferdams and Excavation Bracing, LS

- 503E21300 Unclassified Excavation, LS

- 509E25000 Uncoated Steel Reinforcement, LB

  - Total = 323LB

- 510E10000 Dowel Holes with Nonshrink, Nonmetallic Grout, EACH

  - Total From Rebar Table = 68 each

- 513E10201 Structural Steel for Rehabilitation, APP LB

  - Total from Table = 2680LB

- 514E27700 Field Painting, Misc.: Zinc Primer, SF

  - Total From Table = 542SF+111SF = 653SF

- 516E13200 1/2” Preformed Expansion Joint Filler, SF

  - Rear Abut = 24’x0.5’ = 12SF

  - Fwd Abut = 18’x0.5’ = 9SF

  - Total = 21SF

- 530E00600 Special- Structures: Abutment Refacing, SF

  - Rear Abut = [(4.92’+4.167’)/2]x24’ = 109.04SF

  - Fwd Abut = [(4.08’+4.66’)/2]x18’ = 78.66SF

  - Total = 187.70SF = 188SF

- 843E50000 Patching Concrete Structures with Trowelable Mortar, SF

  - Rear = (18’x2.5’) = 45SF

  - Fwd = (24’x2.5’) = 60SF

  - West Edge = (6SF+7+9+6+12+16.5+4+7.5+7.5+7.5) = 143sf

  - East Edge = (6SF+8+4+8+8+6) = 40sf

  - Total = 45SF+60SF + 143sf + 40sf= 288SF

Structure: SHE-75-0997 R

- 513E10201 Structural Steel for Rehabilitation APP, LB

  - Total from Table = 2680LB

- 514E27700 Field Painting, Misc.: Zinc Primer, SF

  - Total From Table = 542SF+111SF = 653SF

- 843E50000 Patching Concrete Structures with Trowelable Mortar, SF

  - West Edge = (24SF+6+28+12+5+5) = 80sf

  - East Edge = (8SF+47+6) = 61sf

  - Rt Fwd Scupper = 19SF

  - TOTAL = 160SF

Maintenance of Traffic

- Item 614 Work Zone Impact attenuator, 24” Wide Hazards, (Bidirectional), EACH

  - Total= 2 EACH

- Item 614 Barrier Reflector, Type 1 (Two way), EACH

  - Total = 220FT of barrier / 50ft spacing = 6 each

- Item 614 Object Marker, Two way, EACH

  - Total = 220FT of barrier / 50ft spacing =6 each

- Item 614 Work Zone Edge Line, Class 1, 4” 740.06, Type 1, FT

  - Total = 54FT+231FT+173FT = 458FT

  - 458ft/5280 = 0.0867 = 0.09MILES

- Item 614 Work Zone Stop Line, Class 1, 4” 740.06, Type 1, FT

  - WZSL-1 = 15FT

  - WZSL-2 = 11FT

  - WZSL-3 = 15FT

  - WZSL-4 = 15FT

  - Total = 56FT

- Item 614 Work Zone Dotted Line, Class 1, 4” 740.06, Type 1, FT

  - Total = 153FT

- Item 615 Portable Barrier, Unanchored, FT

  - Total 220FT

Incidentals

- Item 614-Maintaining Traffic

  - Lump Sum (LS)

- Item 623-Construction Layout Stakes and Surveying

  - Lump Sum (LS)

- Item 624-Mobilization

  - Lump Sum (LS)

END OF CALCULATIONS