PID 116934, D07 BP FY26: QUANTITY CALCULATIONS

Calculated by: Jun Xie, Date: 05/05/25

Checked by:

Revised by: Jun Xie, Date: 04/06/26

Plan Splits:

- 01/NHS

(NHS: Improvement Code:13-Bridge Rehabilitation)

  - DAR-49-1410

- 02/STR

(STP Rural: Improvement Code:13-Bridge Rehabilitation)

  - AUG-197-0510

- 03/IMS

(Interstate: Improvement Code:13-Bridge Rehabilitation)

  - AUG-75-1047

  - AUG-75-1234

Erosion Control

(01/NHS)

- Item 659E10000 SEEDING AND MULCHING

- DAR-49-1410

    - Phase 1

      - Northbound outside shoulder closure (8’ shoulder + 12’ wide construction pad off EOP)

        - 12’ x 250’ (If painting equipment is positioned in a single line, estimated length) = 3000sf

    - Phase 2

      - Southbound inside shoulder closure (4’ shoulder + 16’ wide construction pad in the median)

        - 16’ x 250’ = 4000sf

    - Phase 3

      - Southbound outside shoulder closure: (8’ shoulder + 12’ wide construction pad off EOP)

        - 12’ x 250’ = 3000sf

    - Total: 3000sf + 4000sf + 3000sf = 10000sf = 1112sy

- Item 659E14000 REPAIR SEEDING AND MULCHING

- DAR-49-1410:

    - 1112sy x 5% = 56sy

  - Total: 56sy

- Item 659E20000 COMMERCIAL FERTILIZER

- DAR-49-1410:

    - 1112sy / (7410 sy/ton) = 0.15ton

  - Total: 0.15ton

- Item 659E35000 WATER

- DAR-49-1410:

    - 2 applications x 1112sy x 0.0027MGal/sy = 6.0MGal

- Total: 6.0MGal

- Item 832E30000 EROSION CONTROL

- DAR-49-1410:

    - [SS 832 Appendix F] Project Identified Earth Disturbing Activity (EDA) <5 acres, construction seeding and mulching unit price 1.21 x $1 = $1.21

    - Total Cost: 1112sy (seeding area) x $1.21 x 1.5 x 1 season = $1868

    - Erosion Control QTY: 1900 EA

(03/IMS)

- Item 659E10000 SEEDING AND MULCHING

- AUG-75-1047

    - Phase 1

      - Southbound outside shoulder of closure (10’ shoulder + 10’ wide construction pad off EOP)

        - 10’ x 250’ (If painting equipment is positioned in a single line, estimated length) = 2500sf

    - Phase 2

      - Southbound inside shoulder closure (4’ shoulder + 16’ wide construction pad in the median)

        - 16’ x 250’ = 4000sf

    - Phase 3

      - Northbound outside shoulder closure: (10’ shoulder + 10’ wide construction pad off EOP)

        - 10’ x 250’ = 2500sf

    - Total: 2500sf + 4000sf + 2500sf = 9000sf = 1000sy

- AUG-75-1234

    - Phase 1

      - Southbound outside shoulder of closure (10’ shoulder + 10’ wide construction pad off EOP)

        - 10’ x 250’ (If painting equipment is positioned in a single line, estimated length) = 2500sf

    - Phase 2

      - Southbound inside shoulder closure (7’ shoulder + 13’ wide construction pad in the median)

        - 13’ x 250’ = 3250sf

    - Phase 3

      - Northbound outside shoulder closure: (8’ shoulder + 12’ wide construction pad off EOP)

        - 12’ x 250’ = 3000sf

    - Total: 2500sf + 3250sf + 3000sf = 8750sf = 973sy

c. Grand Total: 1000sy + 973sy = 1,973sy

- Item 659E14000 REPAIR SEEDING AND MULCHING

- AUG-75-1047:

    - 1000sy x 5% = 50sy

- AUG-75-1234:

    - 973sy x 5% = 49sy

c. Total: 99sy

- Item 659E20000 COMMERCIAL FERTILIZER

- AUG-75-1047:

    - 1000sy / (7410 sy/ton) = 0.14ton

- AUG-75-1234:

    - 973sy / (7410 sy/ton) = 0.13ton

  - Total: 0.14ton + 0.13ton = 0.27ton

- Item 659E35000 WATER

- AUG-75-1047:

    - 2 applications x 1000sy x 0.0027MGal/sy = 5.4MGal

- AUG-75-1234:

    - 2 applications x 973sy x 0.0027MGal/sy = 5.3MGal

  - Total: 5.4MGal + 5.3MGal = 10.7MGal

- Item 832E30000 EROSION CONTROL

[SS 832 Appendix F] Project Identified Earth Disturbing Activity (EDA) <5 acres, construction seeding and mulching unit price 1.21 x $1 = $1.21

(03/IMS)

- AUG-75-1047:

    - Total Cost: 1000sy (seeding area) x $1.21 x 1.5 x 1 (season) = $1815

    - Erosion Control QTY: 1850 EA

- AUG-75-1234:

    - Total Cost: 973sy (seeding area) x $1.21 x 1.5 x 1 (season) = $1766

    - Erosion Control QTY: 1800 EA

- Total: 1,850 EA + 1,800 EA = 3,650 EA

Structure: AUG-75-1047

(03/IMS)

- Item 514E00050 SURFACE PREPARATION OF EXISTING STRUCTURAL STEEL

  - AUG-75-1047:

    - Total painting quantities:

      - W30x173 Beam: (beamlines: 4) x (parameter: 2 x 30.4” - 2 x 1.07” + 3 x 15” - 2 x 0.655”) x (length: 56’ x 2 + 70’ x 2 – 2 x 1.131’) = 4 x 8.53’ x 249.74’ = 8521 ft2

      - 3 x 3 x 5/16 Cross Frames: (parameter: 4 x 3” = 1’) x (length: 2 x 8.71’ + 1 x 8.42’) x (units: 16 x 3 + 1 + 1) =   1292 ft2

      - (8521 ft2 + 1292 ft2) x 1.02 (misc. for bolts and splices) = 10010 ft2

- Item 514E00056 FIELD PAINTING OF EXISTING STRUCTURAL STEEL, PRIME COAT

  - AUG-75-1047:

    - Total painting quantities:

      - 10010 ft2

- Item 514E00060 FIELD PAINTING STRUCTURAL STEEL, INTERMEDIATE COAT

  - AUG-75-1047:

    - Total painting quantities:

      - 10010 ft2

- Item 514E00066 FIELD PAINTING STRUCTURAL STEEL, FINISH COAT

  - AUG-75-1047:

    - Total painting quantities:

      - 10010 ft2

- Item 514E00504 GRINDING FINS, TEARS, SLIVERS ON EXISTING STRUCTURAL STEEL

  - AUG-75-1047:

Per Ohio DOT BDM 2020 (07/18/2025) section 404.1.11, estimating 1-minute MNHR for each linear foot of beam/girder to be coated.

    - Total Man Hour:

      - 249.74ft x 4 (beam line) x (1 min/ft) x (1 hr/60 min) = 16.65 hours.

      - Round to 17 hours.

- Item 514E10000 FINAL INSPECTION REPAIR

  - AUG-75-1047:

    - Total Man Hour:

Per CMS 514.21.A, final inspection repair (web/flange remove: 1 location per 300 ft beam line; cross assemblies destructive testing: 2.5% of all cross assemblies)

      - 249.74ft x 4 (beam line) / 300ft + 0.025 x [ 50 (# of intermediate cross frame assemblies) + 0 (# of end cross frame assemblies) = 4.54 each

      - Round up to 5 each.

Structure: AUG-75-1234

(03/IMS)

- Item 514E00050 SURFACE PREPARATION OF EXISTING STRUCTURAL STEEL

  - AUG-75-1234:

    - Total painting quantities:

      - W27x178 Beam: (beamlines: 7) x (parameter: 2 x 27.8” - 2 x 1.19’ + 3 x 14.1” - 2 x 0.725”) x (length: 232.25’ - 2 x 1.5’) = 7 x 7.84’ x 229.25’ = 12582 ft2

      - 3 x 3 x 5/16 Cross Frames: (parameter: 1’) x (length: 2 x 8.97’ + 1 x 8.75’) x (units: 14 x 4) + (parameter: 1’) x (length: 2 x 10.19’ + 1 x 10’) x (units: 14 x 2) = 2346 ft2

      - (12582 ft2 + 2346 ft2) x 1.02 (misc. for bolts and splices) = 15227 ft2

      - Round up to 15230 ft2

- Item 514E00056 FIELD PAINTING OF EXISTING STRUCTURAL STEEL, PRIME COAT

- AUG-75-1234:

    - Total painting quantities:

      - 15230 ft2

- Item 514E00060 FIELD PAINTING STRUCTURAL STEEL, INTERMEDIATE COAT

- AUG-75-1234:

    - Total painting quantities:

      - 15230 ft2

- Item 514E00066 FIELD PAINTING STRUCTURAL STEEL, FINISH COAT

- AUG-75-1234:

    - Total painting quantities:

      - 15230 ft2

- Item 514E00504 GRINDING FINS, TEARS, SLIVERS ON EXISTING STRUCTURAL STEEL

- AUG-75-1234:

Per Ohio DOT BDM 2020 (01/17/2025) section 404.1.11, estimating 1-minute MNHR for each linear foot of beam/girder to be coated.

    - Total Man Hour:

      - 229.25ft x 7 (beam line) x (1 min/ft) x (1 hr/60 min) = 26.75 hours.

      - Round to 27 hours.

- Item 514E10000 FINAL INSPECTION REPAIR

- AUG-75-1234:

Per CMS 514.21.A, final inspection repair (web/flange remove: 1 location per 300 ft beam line; cross assemblies destructive testing: 2.5% of all cross assemblies)

    - 229.25ft x 7(beam line) / 300ft + 0.025 x [84 (# of intermediate cross frame assemblies) + 0 (# of end cross frame assemblies) = 7.45 each

- Round up to 8 each.

Structure: AUG-197-0510

(02/STR)

- Item 514E00050 SURFACE PREPARATION OF EXISTING STRUCTURAL STEEL

  - AUG-197-0510:

    - Total painting quantities:

      - W33x152 Beam: (beamlines: 5) x (parameter: 2 x 33.5” - 2 x 1.06” + 3 x 11.6” - 2 x 0.635”) x (length: 182.83’) = 5 x 8.20’ x 182.83’ = 7496 ft2

      - 3x3 x 5/16 Cross Frames: (parameter: 1’) x (length: 2 x 8.26’ + 1 x 7.88’) x (units: 15 x 4) = 1464 ft2

      - 4x4 x 5/16 End Frames:

        - End cross Frames: (parameter: 4 x 4” = 1.33’) x (length: 2 x 4.65’ + 1 x 7.88’) x (units: 2 x 4) = 183 ft2

        - End Plate: 2 sides x (length: 1.5’) x (height: 1’) x (units: 2 x 4) = 24 ft2

        - Total: 183 ft2 + 24 ft2 = 207 ft2

      - (7496 ft2 + 1464 ft2 + 207 ft2) x 1.02 (misc. for bolts and splices) = 9350 ft2.

- Item 514E00056 FIELD PAINTING OF EXISTING STRUCTURAL STEEL, PRIME COAT

- AUG-197-0510:

    - Total painting quantities:

      - 9350 ft2

- Item 514E00060 FIELD PAINTING STRUCTURAL STEEL, INTERMEDIATE COAT

- AUG-197-0510:

    - Total painting quantities:

      - 9350 ft2

- Item 514E00066 FIELD PAINTING STRUCTURAL STEEL, FINISH COAT

- AUG-197-0510:

    - Total painting quantities:

      - 9350 ft2

- Item 514E00504 GRINDING FINS, TEARS, SLIVERS ON EXISTING STRUCTURAL STEEL

- AUG-197-0510:

Per Ohio DOT BDM 2020 (01/17/2025) section 404.1.11, estimating 1-minute MNHR for each linear foot of beam/girder to be coated.

    - Total Man Hour:

      - 182.83ft x 5 (beam line) x (1 min/ft) x (1 hr/60 min) = 15.24 hours.

      - Round to 16 hours.

- Item 514E10000 FINAL INSPECTION REPAIR

- AUG-197-0510:

Per CMS 514.21, final inspection repair (1 location per 300 ft beam line and 2.5% of all cross assemblies)

    - 182.83ft x 5(beam line) / 300ft + 0.025 x [60 (# of intermediate cross frame assemblies) + 8 (# of end cross frame assemblies) = 4.75 each

- Round up to 5 each.

Structure: DAR-49-1410

(01/NHS)

- Item 514E00050 SURFACE PREPARATION OF EXISTING STRUCTURAL STEEL

  - DAR-49-1410:

    - Total painting quantities:

      - W33x118 Beam: (beamlines: 6) x (parameter: 2 x 32.9” - 2 x 0.74” + 3 x 11.5” - 2 x 0.550”) x (length: 0.75’ + 52.75’+12.5’) x 2 = 6 x 8.15’ x 132’ = 6455 ft2

      - W33x141 Beam: (beamlines: 6) x (parameter: 2 x 33.3” + 4 x 11.5” - 2 x 0.605”) x (length: 57.5’ + 32.5’) = 6 x 9.28’ x 90’ = 5012 ft2

      - 3x3 x 5/16 Cross Frames: (parameter: 1’) x (length: 2 x 8.77’ + 1 x 8.42’) x (units: 15 x 5) = 1947 ft2

      - 4x4 x 5/16 End Frames:

        - End cross Frames: (parameter: 4 x 4” = 1.33’) x (length: 1 x 8.42’ + 2 x 3.74’ + 2 x 2.84’) x (units: 2 x 5) = 287 ft2

        - End Plate: 2 sides x (length: 1’) x (height: 0.75’) x (units: 2 x 5 x 3) = 45 ft2

        - Total: 287 ft2 + 45 ft2 = 332 ft2

      - 5926 ft2 + 5012 ft2 + 1947 ft2 + 332 ft2= 13217 ft2. Round up to 13500 ft2

- Item 514E00056 FIELD PAINTING OF EXISTING STRUCTURAL STEEL, PRIME COAT

- DAR-49-1410:

    - Total painting quantities:

      - 13500 ft2

- Item 514E00060 FIELD PAINTING STRUCTURAL STEEL, INTERMEDIATE COAT

- DAR-49-1410:

    - Total painting quantities:

      - 13500 ft2

- Item 514E00066 FIELD PAINTING STRUCTURAL STEEL, FINISH COAT

- DAR-49-1410:

    - Total painting quantities:

      - 13500 ft2

- Item 514E00504 GRINDING FINS, TEARS, SLIVERS ON EXISTING STRUCTURAL STEEL

- DAR-49-1410:

Per Ohio DOT BDM 2020 (01/17/2025) section 404.1.11, estimating 1-minute MNHR for each linear foot of beam/girder to be coated.

    - Total Man Hour:

      - 197 ft x 6 (beam line) x (1 min/ft) x (1 hr/60 min) = 19.7 hours.

      - Round to 20 hours.

- Item 514E10000 FINAL INSPECTION REPAIR

- DAR-49-1410:

Per CMS 514.21, final inspection repair (1 location per 300 ft beam line and 2.5% of all cross assemblies)

    - 197ft x 6 (beam line) / 300ft + 0.025 x [75 (# of intermediate cross frame assemblies) + 10 (# of end cross frame assemblies) = 6.07 each

- Round to 6 each.

Maintenance of Traffic

(01/NHS)

- Item 614E11110 LAW ENFORCEMENT OFFICER WITH PATROL CAR FOR ASSISTANCE

  - DAR-49-1410: 175 Hours

- Item 614E12380 WORK ZONE IMPACT ATTENUATOR, 24" WIDE HAZARDS, (UNIDIRECTIONAL)

  - DAR-49-1410: 3 phases x 1 attenuator/phase = 3 EA

- Item 614E13310 BARRIER REFLECTOR, TYPE 1 (ONE WAY)

  - DAR-49-1410: [MT-101.70] 50’ max intervals.

    - Phase 1: (1 reflector / 50’ portable barrier) x 330’ (total PB length) = 6.6 EA. Round to 7 EA

    - Phase 2: (1 reflector / 50’ portable barrier) x 300’ (total PB length) = 6 EA

    - Phase 3: (1 reflector / 50’ portable barrier) x 330’ (total PB length) = 6.6 EA. Round to 7 EA

  - Total = 7 + 6 + 7 = 20 EA

- Item 614E13350 OBJECT MARKER, ONE WAY

  - DAR-49-1410:

    - Phase 1: (1 marker / 50’ portable barrier) x 330’ (total PB length) = 6.6 EA. Round to 7 EA

    - Phase 2: (1 marker / 50’ portable barrier) x 300’ (total PB length) = 6 EA

    - Phase 3: (1 marker / 50’ portable barrier) x 330’ (total PB length) = 6.6 EA. Round to 7 EA

  - Total = 7 + 6 + 7 = 20 EA

- Item 622E41100 PORTABLE BARRIER, UNANCHORED

  - DAR-49-1410:

    - (Phase 1) NB Outside shoulder closure: 330’

      - PID 105405 Jackie email here (link)

        - If painting equipment is positioned in a single line, length 225’- 240’. Assume 250’ for estimate.

        - Per Traffic SDC MT-101.75, recover area behind portable barrier has a dimension of 75’(long) and 20’(wide).

        - Total single line portable barrier length: 250’+75’ = 325’. Round up to 330’.

    - (Phase 2) SB inside shoulder closure: 300’

      - PB starts at the south end of SB inside shoulder guardrail and ends at the other side of median/perpendicular to NB inside shoulder guardrail line at the south end point.

      - Enough space for multiple-line painting equipment setup.

    - (Phase 3) SB inside shoulder closure: 330’

    - Total: 330’ + 300’ + 330’ = 960’

(02/STR)

- Item 614E11110 LAW ENFORCEMENT OFFICER WITH PATROL CAR FOR ASSISTANCE

- AUG-197-0510: 100 Hours

- Item 614E12420 DETOUR SIGNING

- AUG-197-0510: LS

(03/IMS)

- Item 614E11110 LAW ENFORCEMENT OFFICER WITH PATROL CAR FOR ASSISTANCE

  - AUG-75-1047: 175 Hours

  - AUG-75-1234: 250 Hours

  - Total: 425 Hours

- Item 614E12380 WORK ZONE IMPACT ATTENUATOR, 24" WIDE HAZARDS, (UNIDIRECTIONAL)

  - AUG-75-1047: 3 phases x 1 attenuator/phase = 3 EA

  - AUG-75-1234: 3 phases x 1 attenuator/phase = 3 EA

  - Total: 6 EA

- Item 614E12420 DETOUR SIGNING

- AUG-75-1234: LS

- Item 614E13310 BARRIER REFLECTOR, TYPE 1 (ONE WAY)

  - AUG-75-1047: [MT-101.70] 50’ max intervals.

    - Phase 1: (1 reflector / 50’ portable barrier) x 330’ (total PB length) = 6.6 EA. Round to 7 EA

    - Phase 2: (1 reflector / 50’ portable barrier) x 400’ (total PB length) = 8 EA

    - Phase 3: (1 reflector / 50’ portable barrier) x 210’ (total PB length) = 4.2 EA. Round to 5 EA

  - AUG-75-1234:

    - Phase 1: (1 reflector / 50’ portable barrier) x 330’ (total PB length) = 6.6 EA. Round to 7 EA

    - Phase 2: (1 reflector / 50’ portable barrier) x 420’ (total PB length) = 8.4 EA. Round to 9 EA

    - Phase 3: (1 reflector / 50’ portable barrier) x 330’ (total PB length) = 6.6 EA. Round to 7 EA

  - Total = 7 + 8 + 5 + 7 + 9 +7 = 43 EA

- Item 614E13350 OBJECT MARKER, ONE WAY

  - AUG-75-1047:

    - Phase 1: (1 marker / 50’ portable barrier) x 330’ (total PB length) = 6.6 EA. Round to 7 EA

    - Phase 2: (1 marker / 50’ portable barrier) x 400’ (total PB length) = 8 EA

    - Phase 3: (1 marker / 50’ portable barrier) x 210’ (total PB length) = 4.2 EA. Round to 5 EA

  - AUG-75-1047:

    - Phase 1: (1 marker / 50’ portable barrier) x 330’ (total PB length) = 6.6 EA. Round to 7 EA

    - Phase 2: (1 marker / 50’ portable barrier) x 420’ (total PB length) = 8.4 EA. Round to 9 EA

    - Phase 3: (1 marker / 50’ portable barrier) x 330’ (total PB length) = 6.6 EA. Round to 7 EA

  - Total = 7 + 8 + 5 + 7 + 9 + 7 = 43 EA

- Item 622E41100 PORTABLE BARRIER, UNANCHORED

  - AUG-75-1047:

    - (Phase 1) SB outside shoulder closure: 330’

    - (Phase 2) SB inside shoulder closure: 400’

      - Entire run where median guardrail exists.

    - (Phase 3) NB outside shoulder closure: 210’

      - 330’ starting at the bridge - 120’ of guard rail run = 210’

    - Total: 330’ + 400’ + 210’ = 940’

  - AUG-75-1234:

    - (Phase 1) SB Outside shoulder closure: 330’

    - (Phase 2) SB inside shoulder closure: 420’

      - Entire run where median guardrail exists.

    - (Phase 3) NB outside shoulder and exit ramp lane closure: 330’

    - Total: 330’ + 420’ + 330’ = 1080’

  - Grand total: 940’ + 1080’ = 2020’

Incidentals

(01/NHS)

- Item 614E11000 MAINTAINING TRAFFIC

- LS

- Item 623E10000 CONSTRUCTION LAYOUT STAKES AND SURVEYING

- LS

- Item 624E10000 MOBILIZATION

- LS

(02/STR)

- Item 614E11000 MAINTAINING TRAFFIC

- LS

- Item 623E10000 CONSTRUCTION LAYOUT STAKES AND SURVEYING

- LS

- Item 624E10000 MOBILIZATION

- LS

(03/IMS)

- Item 614E11000 MAINTAINING TRAFFIC

- LS

- Item 623E10000 CONSTRUCTION LAYOUT STAKES AND SURVEYING

- LS

- Item 624E10000 MOBILIZATION

- LS

END OF CALCULATIONS