203		20000 	EMBANKMENT

Under App Slab(9’)[Length] x (21’)[Avg,Ht from Gound] x (32’)[Width]

6048 CF

Quantity:  224 CY

_____________________________________________________________________

203		35110 	GRANULAR MATERIAL, TYPE B

32x30 (App Slab) x 12.5’ (Avg ht of material under App Slab) = 12000 CF

Quantity:  444 CY

____________________________________________________________________

203		35120 	GRANULAR MATERIAL, TYPE C

Behind Wall 5750 [SF] (CAD measured) x 1.0 [FT] Infront of Wall

+ 327 (Wall length)1.0(thickness) x 2.5 (Length Infront)

= 5750+818

Quantity:  243 CY

_______________________________________________________________    __

512               10100	SEALING OF CONCRETE SURFACES (EPOXY-URETHANE)

5120 [SF] (Exposed wall area, CAD measured) +

(4.5) /12 (Copping bottom) x 352 [FT] + 10/12(Copping top) x 352 [FT]

= 5545 SF

Quantity: 616 SY

____________________________________________________________________

516                13200	1/2” PREFORMED EXPANSION JOINT FILLER

352/20 = 18 [JOINTS] x 1.31 [SF] (See coping details, CAD measured) = 13.1 [SF]

Quantity:  23.58 SF

____________________________________________________________________

516	13900		2” PREFORMED EXPANSION JOINT FILLER

32 [FT](width at app slab) x 0.5 [FT](thickness) x 2[joints in front of bridge]

+ (204 [FT] + 76 FT]) x 0.5 [FT] (at gutter)

= 172 [SF]

Quantity:  172 SF

____________________________________________________________________

530                13000	FORM LINER

6480 [SF] (Entire wall area CAD measured)

Quantity:  6480 SF

________________________________________________________________

601                21000	CONCRETE SLOPE PROTECTION

135 [SF] (CAD measured) + {(4.13 [SF] + 5.55 [SF]) / 2 x

(38.86+35.10) / 2} / COS(26.56) = 248.52 [SF] = 28 [SY]

Quantity:  28 SY

601                37500	PAVED GUTTER

Quantity:  216 SF

____________________________________________________________________

840                20000	MECHANICALLY STABILIZED EARTH WALL

6480 [SF] (Entire wall area CAD measured)

Quantity:  6480 SF

____________________________________________________________________

840                 21000         WALL EXCAVATION

5750 [SF] (From Cad Plan View) x 1.5[FT](Avg Ht of Ex. Ground)

+  327(wall length) x 2.5ft (in front of wall) x 1.5(from Ex round)

= 9851 CF

Quantity: 365 CY

____________________________________________________________________

840               22000            FOUNDATION PREPARATION

5750 [SF] (FROM CAD) + 327(wall length) x 2.5ft (in front of wall)

Quantity:  730 SY

___________________________________________________________________

840		23000		SELECT GRANULAR BACKFILL

| Station (along BL Ramp A5) |  |  | CAD measured Area (SF) | x | Average Width of Area (FT) | = | Volume of Select Granular Backfill (CY) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5014+84.75 | to | 5015+45.28 | 1506 | x | 20.0 | = | 1115.55 |
| 5015+45.28 | to | 5015+44.94 | 1018 | x | 30.0 | = | 1131.11 |
| 5015+44.94 | to | 5015+10.59 | 1324 | x | 25.0 | = | 1225.93 |
| 5015+10.59 | to | 5014+64.59 | 938 | x | 18.0 | = | 625.33 |
| 5014+64.59 | to | 5013+89.59 | 1204 | x | 15.0 | = | 668.88 |
| 5013+89.59 | to | 5013+68.70 | 353 | x | 8.0 | = | 104.59 |
|  |  |  |  |  |  |  | -119 |

Quantity: 4753 CY

840	25010		6” DRAINAGE PIPE, PERFORATED

2(112+48+53+32+82) +(3+6)(at the ends)  Measured in  cadd plan view

Quantity:  663 FT

840                 26000            CONCRETE COPING

352 [FT] Measured along wall

Quantity:  352 FT

840	26050	             AESTHETIC SURFACE TREATMENT

5120 [SF] (wall area above finished grade, CAD measured)

Quantity: 5120 SF

______________________________________________________________________

840		27000		ON-SITE ASSISTANCE

1 DAY

Quantity: 1 DAY

______________________________________________________________________

840		23050		NATURAL SOIL

((2+3)/ 2 x 7) + ((2+5)/2 x 14) /2 x 24 / 27 = 29.5 CY

((2+5)/ 2 x 14) + ((2+7)/2 x 21) /2 x 25 / 27 = 66.4 CY

((2+7)/ 2 x 21) + ((2+8)/2 x 23) /2 x 7 / 27 = 27.1 CY

((2+8)/ 2 x 23) + ((2+2)/2 x 5) /2 x 16 / 27 = 37.0 CY

((2+2)/ 2 x 4) + ((2+8)/2 x 24) /2 x 18.5 / 27 = 43.9 CY

((2+8)/ 2 x 24) + ((2+5)/2 x 17) /2 x 18.5 / 27 = 27.1 CY

((2+5)/ 2 x 17) /2 x 12 / 27 = 13.0 CY

(12 + 36) /2 x 32 / 27 = 28.4 CY

(36 + 77) /2 x 50 / 27 = 104.6 CY

(77 + 96) /2 x 50 / 27 = 160.2 CY

Quantity: 562 CY