203		35120 	GRANULAR MATERIAL, TYPE C

22271 [SF] (CAD SGB area + 2FT in front of Wall) x 1.0 [FT](Min thickness)

22271 CF

Quantity:  825 CY

_____________________________________________________________________

509		10000 	REINFORCING STEEL

See rebar list spreadsheet

Quantity:  89002 LBS

_____________________________________________________________________

511		46212 	CLASS QCI CONCRETE

(Section Area of Moment Slab from Cad Including parapet) = 15 SF

Length of sleeper slab 548ft

Volume of Concrete for Moment Slab = 15 x 548 = 8220 CF

Quantity:  305 CY

______________________________________________________________________

512               10100	SEALING OF CONCRETE SURFACES (EPOXY-URETHANE)

19216 [SF] (CAD measured) (measured along top of

Coping to the existing ground) +

10/12 [FT] x 937 ft (along top of coping) + 4.5/12[FT]x 937 (bot of coping)

8.06ft (Parapet surface length) x 548ft length of Moment Slab

19216+1132+4417

= 24765 [SF] = 2751 [SY]

Quantity: 2751 SY

____________________________________________________________________

516                13200	1/2” PREFORMED EXPANSION JOINT FILLER

937 [FT] (measured along top of coping) / 20 [FT/JOINT] = 46.85

= 48 [JOINTS] x 1.31 [SF/JOINT measured from CAD] = 62.88 [SF]

Quantity:  63 SF

516	13600		1” PREFORMED EXPANSION JOINT FILLER

548 [FT] (measured along top of coping) / 30 [FT/JOINT] = 18.26

Joint Area x number of joints =   (14.08 SF, from cad x 18 joints)

= 253.44SF

Quantity:  254 SF

______________________________________________________________________

516	13900		2” PREFORMED EXPANSION JOINT FILLER

548 ft (length of moment slab) x (1.75+1)’ (Pvmt + Mom Slb, thickness)  + 282 x 0.5

= 1648 [SF]

Quantity:  1648 SF

___________________________________________________________________

530                 13000         FORM LINER

CAD MEASURED

Quantity: 23305 CY

601                 37500         PAVED GUTTER, TYPE 1-2

CAD MEASURED

Quantity: 287 FT

840                20000	MECHANICALLY STABILIZED EARTH WALL

23410 [SF] (CAD measured)

Quantity:  23305 SF

____________________________________________________________________

840                 210000         WALL EXCAVATION

CAD MEASURED

Quantity: 8224 CY

______________________________________________________________________

840               22000            FOUNDATION PREPARATION

22257 [SF] (CAD measured SGB +2’ from wall)

Quantity:  2473 SY

840		23000		SELECT GRANULAR BACKFILL

| STATION | WALL AREA (SF) | THICKNESS(FT) | VOLUME (CF) |
| --- | --- | --- | --- |
| 0+00 TO 2+47 | 5185 | 20 | 103700 |
| 2+47 TO 5+36 | 9003 | 25 | 225075 |
| 5+36 TO 5+62 | 766 | 24 | 18384 |
| 5+62 TO 6+29 | 1835 | 23 | 42205 |
| 6+29 TO 7+15 | 2087 | 28 | 58436 |
| 7+15 TO 8+18 | 1798 | 22 | 39556 |
| 8+18 TO 8+40 | 315 | 18 | 5670 |
| 8+40 TO 8+65 | 276 | 16 | 4416 |
| 8+65 TO 8+90 | 237 | 10 | 2370 |
| 8+90 TO 9+12 | 175 | 8 | 1400 |
| 9+12 TO 9+45 | 184 | 8 | 1472 |
|  | TOTAL SGB VOLUME IN CY | DEDUCT 655 CY NATURAL SOIL | 17963 |

840                 26000            CONCRETE COPING

937 [FT] (measured along top of coping) = 937 [FT]

Quantity:  937

______________________________________________________________________

840	25010		6” PERFORATED CORRUGATED PLASTIC PIPE

950 [FT] + 975 [FT] CAD MEASURED

Quantity:  1925 FT

840	25020		6” NON-PERFORATED CORRUGATED PLASTIC PIPE,

INCLUDING SPECIALS

57.1 [FT] CAD MEASURED

Quantity:  58 FT

840		26050	             AESTHETIC SURFACE TREATMENT

23298 [SF] (CAD measured)

Quantity: 23298 SF

______________________________________________________________________

840		27000		ON-SITE ASSISTANCE

1 DAY

Quantity: 1 DAY

______________________________________________________________________

840		23050		NATURAL SOIL

(0 + 12) /2 x 50 / 27 = 11.1 CY

(12 + 73) / 2 x 50 / 27 = 78.7 CY

(73 + 96) / 2 x 50 / 27 = 156.5 CY

(96 + 77) /2 x 50 / 27 = 160.2 CY

(77 + 50) / 2 x 50 / 27 = 117.6 CY

(50 + 26) / 2 x 50 / 27 = 70.4 CY

(26 + 12) / 2 x 50 / 27 = 35.2 CY

(20 + 20) / 2 x 35 / 27 = 25.9 CY

Quantity: 655 CY