203		35000 	GRANULAR EMBANKMENT

Quantity from Ground Improvement at Wall 4W8 & 4W9

Report

Quantity:  15,870 CY

_____________________________________________________________________

203		35110 	GRANULAR MATERIAL, TYPE B

Quantity from Ground Improvement at Wall 4W8 & 4W9

Report

Quantity:  3285 CY

_____________________________________________________________________

203		35120 	GRANULAR MATERIAL, TYPE C

28628 [SF] (from Cad) x 3.0 [FT] / 27

Quantity:  3181 CY

______________________________________________________________________

203		98100 	ROADWAY MISC.: CONTROLLED MODULUS COLUMN

Quantity from Ground Improvement at Wall 4W8 & 4W9

Report

Quantity:  4816 SY

_____________________________________________________________________

509		10000 	REINFORCING STEEL

See rebar list spreadsheet

Quantity:  79170 LBS

______________________________________________________________________

511		46212 	CLASS QC1 CONCRETE

(Section Area of Moment Slab from Cad Including parapet) = 14.5 SF

Length of Moment Slab = 595ft

Volume of Concrete for Moment Slab = 14.5 x 595 = 8628 CF

Leveling Pad = 872 length   x 2 x 0.5 = 872 CF

Total concrete volume   = 9500 CF

Quantity:  352 CY

______________________________________________________________________

512               10100	SEALING OF CONCRETE SURFACES (EPOXY-URETHANE)

27356 [SF] (CAD measured) (measured along top of

Coping to the proposed ground) +

10[FT] x 877ft (along top of coping) +

4.5/12[FT]x 877 (along bottom of coping) +

7.90ft (Parapet surface length) x 596ft length of Moment Slab

= 33123 [SF] = 3951 [SY]

Quantity: 3680 SY

____________________________________________________________________

516                13200	1/2” PREFORMED EXPANSION JOINT FILLER

875 [FT] (measured along top of coping) / 20 [FT/JOINT] = 43.75

= 44 [JOINTS] x 1.31 [SF/JOINT measured from CAD] = 57.64 [SF]

Quantity:  58 SF

______________________________________________________________________

516	            13600	1” PREFORMED EXPANSION JOINT FILLER

569 [FT] (measured along top of coping) / 30 [FT/JOINT] = 19

Joint Area x number of joints = (14.08 SF, from cad x 18 joints)

= 268 SF

Quantity:  268 SF

________________________________________________________________

516	13900		2” PREFORMED EXPANSION JOINT FILLER

569 ft (along wall) x (1.75+1)’ (Pvmt + Mom Slb) + 2[1.67(@ App Slb) x 1.75 (Pvt. thickness)] [at the ends]

= 1753 [SF]

Quantity:  1753 SF

______________________________________________________________________

530                13000	FORM LINER

Quantity from Ground Improvement at Wall 4W8 & 4W9

Report

Quantity:  34,590 SF

____________________________________________________________________

840                20000	MECHANICALLY STABILIZED EARTH WALL

Quantity from Ground Improvement at Wall 4W8 & 4W9

Report

Quantity:  34,590 SF

____________________________________________________________________

840                 21000         WALL EXCAVATION

Quantity from Ground Improvement at Wall 4W8 & 4W9

Report

Quantity: 3368 CY

___________________________________________________________________

840               22000            FOUNDATION PREPARATION

28628 [SF]

Quantity:  3181 SY

___________________________________________________________

840		23000		SELECT GRANULAR BACKFILL

Quantity from Ground Improvement at Wall 4W8 & 4W9

Report

Quantity: 42,100 CY

840	25010 	6” DRAINAGE PIPE, PERFORATED

1586 [FT] (Measured from Cad) +34 [FT] = 1620 [FT]

Quantity:  1620 FT

840	25020		6” DRAINAGE PIPE, NON-PERFORATED

INCLUDING SPEACIALS

11 [FT] = 11 [FT]

Quantity:  11 FT

840                 26000            CONCRETE COPING

875 [FT] (measured along top of coping) = 875 [FT]

Quantity:  875 [FT]

840		26050	             AESTHETIC SURFACE TREATMENT

34590 [SF] (CAD measured) – 875 (Coping overlap)

Quantity: 33,715 SF

______________________________________________________________________

840		27000		ON-SITE ASSISTANCE

1 DAY

Quantity: 1 DAY

______________________________________________________________________