HIG-138-2460 OPUS SV1(C)

IRON PIN & CAP SET

PID: 109576

FILE: 87101480.DAT OP1622632406184

NGS OPUS SOLUTION REPORT

========================

All computed coordinate accuracies are listed as peak-to-peak values.

For additional information: https://gcc02.safelinks.protection.outlook.com/?url=https%3A%2F%2Fwww.ngs.noaa.gov%2FOPUS%2Fabout.jsp%23accuracy&amp;data=04%7C01%7Cjason.robinson%40dot.ohio.gov%7Cdc007fad102a41cd4ed708d925b79930%7C50f8fcc494d84f0784eb36ed57c7c8a2%7C0%7C0%7C637582292760732742%7CUnknown%7CTWFpbGZsb3d8eyJWIjoiMC4wLjAwMDAiLCJQIjoiV2luMzIiLCJBTiI6Ik1haWwiLCJXVCI6Mn0%3D%7C1000&amp;sdata=l%2BrvYhcK4YpaMiujof9CWV1yxcYHK4PpBwYBUncRZgE%3D&amp;reserved=0

USER: jason.robinson@dot.ohio.gov             DATE: June 02, 2021

RINEX FILE: 8710148n.21o                            TIME: 11:14:22 UTC

SOFTWARE: page5  2008.25 master73.pl 160321      START: 2021/05/28  13:12:00

EPHEMERIS: igr21595.eph [rapid]                    STOP: 2021/05/28  15:20:00

NAV FILE: brdc1480.21n                        OBS USED:  4830 /  5631   :  86%

ANT NAME: TRMR8           NONE             # FIXED AMB:    44 /    46   :  96%

ARP HEIGHT: 2.000                            OVERALL RMS: 0.015(m)

REF FRAME: NAD_83(2011)(EPOCH:2010.0000)              ITRF2014 (EPOCH:2021.4044)

X:       560530.330(m)   0.009(m)            560529.409(m)   0.009(m)

Y:     -4913087.261(m)   0.003(m)          -4913085.847(m)   0.003(m)

Z:      4015309.085(m)   0.006(m)           4015309.003(m)   0.006(m)

LAT:   39 15 54.22054      0.006(m)        39 15 54.24944      0.006(m)

E LON:  276 30 31.27961      0.009(m)       276 30 31.24814      0.009(m)

W LON:   83 29 28.72039      0.009(m)        83 29 28.75186      0.009(m)

EL HGT:          262.655(m)   0.004(m)               261.435(m)   0.004(m)

ORTHO HGT:	969.00’          295.349(m)   0.063(m) [NAVD88 (Computed using GEOID18)]

UTM COORDINATES    STATE PLANE COORDINATES

UTM (Zone 17)         SPC (3402 OH S)

Northing (Y) [meters]     4349149.890           140907.921	Grid: 462,295.4041	Ground: 462,343.0852

Easting (X)  [meters]      285066.576           514454.610             1,687,839.8330                     1,688,013.9170

Convergence  [degrees]    -1.57737500          -0.62900556

Point Scale                1.00016884           0.99993807

Combined Factor            1.00012763           0.99989687	P.A.F. USED FROM SV1(B) = 1.00010314

US NATIONAL GRID DESIGNATOR: 17SKD8506649149(NAD 83)

BASE STATIONS USED

PID       DESIGNATION                        LATITUDE    LONGITUDE DISTANCE(m)

AJ7192 LEBA LEBANON CORS ARP               N392549.788 W0841659.283   70685.8

DI1876 OHAD ADAMS COUNTY CORS ARP          N384709.347 W0833322.647   53490.0

DF4050 GRTN GEORGETOWN CORS ARP            N385134.330 W0835244.711   56155.5

NEAREST NGS PUBLISHED CONTROL POINT

DQ9432      HIG015                         N391426.535 W0832948.170    2744.0

This position and the above vector components were computed without any knowledge by the National Geodetic Survey regarding the equipment or field operating procedures used.