| Ver. Date | 11/18/2024 |  | PID |  |  | 115771 |
| --- | --- | --- | --- | --- | --- | --- |
| PARCEL |  |  |  | 11-SH |  |  |
| JAC-788-0.90 |  |  |  |  |  |  |

PERPETUAL EASEMENT FOR HIGHWAY PURPOSES WITHOUT LIMITATION OF EXISTING ACCESS RIGHTS

An exclusive perpetual easement for public highway and road purposes, including, but not limited to any utility construction, relocation and/or utility maintenance work deemed appropriate by the State of Ohio, Department of Transportation, its successors and assigns forever. Grantor/Owner, for himself and his heirs, executors, administrators, successors and assigns, reserves all existing rights of ingress and egress to and from any residual area (as used herein, the expression “Grantor/Owner” includes the plural, and words in the masculine include the feminine or neuter).

[Surveyor’s description of the premises follows]

Situated in the State of Ohio, Jackson County, Lick Township, Scioto Salt Reserve Lot No. 57, and conveyed to Rick Smith and Lora Smith by instrument of record in Official Record 149, Page 300, and being a parcel of land laying on the right side of the centerline of State Route 788 (hereafter referred to as S.R. 788), as shown on the JAC-788-0.90 plans made by the Ohio Department of Transportation, and is described as follows:

BEGINNING at ½” iron pin found at the Grantor’s northeasterly corner, 30.29 feet right of centerline Station 474+42.48;

Thence South 25° 09' 47" East a distance of 7.73 feet along the Grantor’s easterly line, to a point 38.00 feet right of centerline Station 474+43.00;

Thence North 73° 08' 57" West a distance of 93.76 feet across the Grantor’s tract, to a ½” iron pin found at the Grantor’s northwesterly corner, 30.77 feet right of centerline Station 473+49.52;

Thence South 68° 25' 54" West a distance of 92.96 feet along the Grantor’s northerly line, to the TRUE POINT OF BEGINNING.

The above described area contains 0.008 acres of land, more or less, of which the present road occupies 0.000 acres of land, which is part of the Jackson County Auditor's Permanent Parcel number H12004000501.

This description was prepared on November 18, 2024, by Michael James Ware, Professional Surveyor Number 8054.

Instrument reference as of the date this survey was prepared: Official Record 149, Page 300, Recorder's Office, Jackson County, Ohio.

Bearings for this description are based on Ohio State Plane Coordinate System, South Zone (SPC3402), and are for project use only.

The stations referred to in this description are from the centerline of construction and Right-of-Way of State Route 788, as found on the JAC-788-0.90 plans, prepared by the Ohio Department of Transportation.

All iron pins set are 5/8" diameter, 30" long rebar with aluminum cap stamped ‘ODOT S-8054’. All other monumentation is as described.

________________________________                                                  _________________

Michael James Ware								       Date

Professional Surveyor S-8054