STANDARD DRAWINGS AND SUPPLEMENTAL SPECIFICATIONS

REFER TO THE FOLLOWING SUPPLEMENTAL SPECIFICATION(S):

| 838 | DATED | 01/15/2021 |
| --- | --- | --- |
| 894 DATED 04/16/2021 |

DESIGN SPECIFICATIONS

THIS STRUCTURE CONFORMS TO THE 9th EDITION OF THE "LRFD BRIDGE DESIGN SPECIFICATIONS" ADOPTED BY THE AMERICAN ASSOCIATION OF STATE HIGHWAY AND TRANSPORTATION OFFICIALS, 2020 AND THE ODOT BRIDGE DESIGN MANUAL, 2020.

OPERATIONAL IMPORTANCE

A LOAD MODIFIER OF 1.0 HAS BEEN ASSUMED FOR THE DESIGN OF THIS STRUCTURE IN ACCORDANCE WITH THE AASHTO LRFD BRIDGE DESIGN SPECIFICATIONS, ARTICLE 1.3.5 AND THE ODOT BRIDGE DESIGN MANUAL.

DESIGN LOADING

DESIGN LOADING INCLUDES:

VEHICULAR LIVE LOAD:  HL-93

FUTURE WEARING SURFACE (FWS) OF 0.060 KIPS/SQ. FT

DESIGN DATA

CONCRETE CLASS QC1:

COMPRESSIVE STRENGTH 4.0 KSI (WINGWALLS, FOOTING AND HEADWALL

)

CONCRETE CLASS QC5, WITH 3/8 IN MAX AGGREGATE SIZE:

COMPRESSIVE STRENGTH 4.5 KSI (DRILLED SHAFT)

CONCRETE REINFORCEMENT:

EPOXY COATED STEEL REINFORCEMENT - MINIMUM YIELD STRENGTH 60-KSI

STRUCTURAL STEEL - ASTM A709 GRADE 50- YIELD STRENGTH 50 KSI

ITEM 202 – STRUCTURE REMOVED,

THE DEPTH AND MATERIAL TYPE OF THE BELOW GRADE BRIDGE ABUTMENTSIS UNKNOWN. REMOVE EXISTING FOUNDATIONS AS NEEDED TO INSTALL PROPOSED DRILLED SHAFTS AND ABUTMENTS.

PARTIAL REMOVAL OF THE EXISTING GABION WALLS ON THE SOUTHEAST CORNER OF THE BRIDGE IS INCLUDED WITH ITEM 202 – STRUCTURE REMOVED, OVER 20 FOOT SPAN, AS PER PLAN. SEE DETAIL THIS SHEET.

ITEM 503, COFFERDAMS AND EXCAVATION BRACING AS PER PLAN:

THE DESIGN SHOW ON THE PLANS FOR TEMPORARY SUPPORT OF EXCAVATION IS ONE REPRESENTATIVE DESIGN THAT MAY BE USED TO CONSTRUCT THE PROJECT. THE CONTRACTOR MAY CONSTRUCT THE DESIGN SHOWN ON THE PLANS OR PREPARE AN ALTERNATIVE DESIGN TO SUPPORT THE SIDE OF EXCAVATIONS. IF CONSTRUCTING AN ALTERNATIVE DESIGN FOR TEMPORARY SUPPORT OF EXCAVATION, PREPARE AND PROVIDE PLANS IN ACCORDANCE WITH C&MS 501.05. THE DEPARTMENT WILL PAY FOR THE TEMPORARY SUPPORT OF EXCAVATION AT THE CONTRACT LUMP SUM PRICE FOR COFFERDAMS AND EXCAVATION BRACING, AS PER PLAN. NO ADDITIONAL PAYMENT WILL BE MADE FOR PROVIDING AN ALTERNATIVE DESIGN.

INCLUDE ALL HP14x117 (50 KSI STEEL), CONCRETE, FLOWABLE FILL, 3”x10” TIMBER LAGGING, AND BEDROCK CORING WITH ITEM 503 COFFERDAMS AND EXCAVATION BRACING, AS PER PLAN.

THREE-SIDED CULVERT WALL AND TOP SLAB THICKNESS:

THE WALL AND TOP SLAB THICKNESS SHOWN ON THE PLANS WERE OBTAINED FROM CULVERT MANUFACTURERS AT THE TIME THE PLANS WERE PREPARED. IF THE WALL AND/OR TOP THICKNESS OF THE CULVERT PROPOSED ARE DIFFERENT FROM WHAT IS SHOWN IN THE PLANS, A MARKED COPY OF THE PROJECT PLANS, INCLUDING ALL PLAN NOTES AND DETAILS SHOWING ALL ITEMS AFFECTED BY THE DIFFERENT CULVERT DIMENSIONS, SHALL BE SUBMITTED FOR APPROVAL WITH THE SHOP DRAWINGS. ALL WORK REQUIRED TO ACCOMMODATE ANY REVISED DIMENSIONS SHALL BE AT NO EXTRA COST TO THE STATE.

ITEM 512 TYPE 2 WATERPROOFING

MEMBRANE WATERPROOFING (SHEET TYPE 2) SHALL EXTEND VERTICALLY DOWN ALL SIDES OF THE CULVERT WHICH SHALL BE IN CONTACT WITH THE BACKFILL.

ITEM 512 TYPE 3 WATERPROOFING

MEMBRANE WATERPROOFING (SHEET TYPE 3) SHALL BE APPLIED TO THE TOP SURFACE OF THE PRECAST CULVERT SECTIONS.

ROCK-SOCKETED DRILLED SHAFTS:

NORTH SIDE:

THE MAXIMUM FACTORED LOAD TO BE SUPPORTED BY EACH DRILLED SHAFT IS XXX KIPS AT THE NORTH FOUNDATION.  THIS LOAD IS RESISTED BY SIDE RESISTANCE WITHIN A PORTION OF THE BEDROCK SOCKET AND BY TIP RESISTANCE.  THE FACTORED SIDE RESISTANCE IS XXX KIPS, ASSUMED TO ACT ALONG THE BOTTOM 5 FEET OF THE BEDROCK SOCKET, AND THE FACTORED TIP RESISTANCE IS XXX KIPS.

SOUTH SIDE:

THE MAXIMUM FACTORED LOAD TO BE SUPPORTED BY EACH DRILLED SHAFT IS XXX KIPS AT THE SOUTH FOUNDATION.  THIS LOAD IS RESISTED BY SIDE RESISTANCE WITHIN A PORTION OF THE BEDROCK SOCKET AND BY TIP RESISTANCE.  THE FACTORED SIDE RESISTANCE IS XXX KIPS, ASSUMED TO ACT ALONG THE BOTTOM X FEET OF THE BEDROCK SOCKET, AND THE FACTORED TIP RESISTANCE IS XXX KIPS.

LATERALLY LOADED DRILLED SHAFTS:

THE MAXIMUM FACTORED LATERAL LOAD AND BENDING MOMENT TO BE SUPPORTED BY REAR ABUTMENT DRILLED SHAFTS ARE xxx KIPS, AND xxx KIP-FEET, RESPECTIVELY.  THESE LOADS PRODUCE A MAXIMUM FACTORED BENDING MOMENT OF xxx KIP-FEET, AND A MAXIMUM FACTORED SHEAR OF xxx KIPS, WITHIN THE DRILLED SHAFT.

SCOUR ELEVATIONS:

THE DESIGN FLOOD AND CHECK FLOOD SCOUR ELEVATIONS ARE PROVIDED BELOW.

|  | NORTH SIDE | SOUTH SIDE |
| --- | --- | --- |
| DESIGN FLOOD | 708.61 | 708.61 |
| CHECK FLOOD | 703 | 703 |

524 – DRILLED SHAFT 36” DIAMETER ABOVE BEDROCK, AS PER PLAN

524 – DRILLED SHAFT 30” DIAMETER INTO BEDROCK, AS PER PLAN

THE DRILLED SHAFT SHALL BE INSTALLED IN ACCORDANCE WITH THE REQUIREMENTS OF ODOT CMS 524. MAX CONCRETE COARSE AGGREGATE SHALL BE SIZE #8 FOR ALL DRILLED SHAFTS.

THE MINIMUM REQUIRED SHAFT LENGTH WAS DEFINED AS THE LENGTH AT WHICH THE MOMENT OF EQUILIBRIUM REQUIREMENTS WERE SATISFIED UTILIZING THE STRENGTH-I AND EXTREME EVENT LOAD CASES.

MINIMUM ROCK SOCKET SHALL BE 10’ MINIMUM UNLESS OTHERWISE INDICATED.

GROUNDWATER IN NOT ANTICIPATED DURING INSTALLATION OF THE DRILLED SHAFTS.

ITEM 894 – THERMAL INTEGRITY PROFILER (T.I.P) TEST:

PERFORM INTEGRITY TESTING ON 10% OF THE DRILLED SHAFTS (2 TOTAL TEST, 1 TEST AT EACH ABUTMENT DRILLED SHAFT) BY THERMAL INTEGRITY PROFILING (TIP).  PERFORM TIP TESTING PER ASTM D7949, “STANDARD TEST METHODS FOR THERMAL INTEGRITY PROFILING OF CONCRETE DEEP FOUNDATIONS," METHOD 8, AND PER THE PROJECT SPECIAL PROVISIONS.

ABBREVIATION LEGEND

APPROX - APPROXIMATE

BOT-BOTTOM

C/C -CENTER TO CENTER

CB - CATCH BASIN

CJ- CONSTRUCTION JOINT

CL- CENTERLINE

CLR - CLEARANCE

CONST - CONSTRUCTION

DIA - DIAMETER

DWG - DRAWING

EF - EACH FACE

EL - ELEVATION

E/P- EDGE OF PAVEMENT

E/S- EDGE OF SHOULDER

EX - EXISTING

EXP - EXPANSION

F/F-FACE TO FACE

HORIZ - HORIZONTAL

LT- LEFT

MAX - MAXIMUM

MIN - MINIMUM

N/A - NOT APPLICABLE

NPCCP NON-PERFORATED CORRUGATED PIPE

O/O – OUT TO OUT

PCPP – PERFORATED CORRUGATED PLASTIC PIPE

PEJF – PREFORMED EXPANSION JOINT FILLER

PG – PROFILE GRADE

PROP – PROPOSED

RT – RIGHT

PVMT – PAVEMENT

SER - SERIES

SHLDR – SHOULDER

SPA – SPACES

ST – STREET

STA – STATION

STD - STANDARD

TBRLO – TO BE RELOCATED BY OTHERS

TEMP – TEMPORARY

TOS – TOP OF SLOPE

T/T – TOE TO TOE

TYP – TYPICAL

VERT - VERTICAL